3GPP TSG-RAN2#129										                               R2-2500773

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	                  Consideration on IoT- NTN TDD mode

Agenda item:	      8.17

Document for:       Discussion and Decision

- Introduction

In RAN1#119 meeting, the following agreements have been made [1]:

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints. Agreement The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9. Agreement RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-U NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of U NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes. - Impact on segmented pre-compensation. Agreement RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-D NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of D NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. - The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications Agreement The downlink subframes within D=8 are to be down-selected among the following: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) - Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames) - Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames) - Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames)

In RAN#106, the revised WID on introduction of IoT-NTN TDD mode[2] has been agreed to support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.

In this contribution, we will discuss the IoT-NTN TDD mode related RAN2 impacts and give our proposals.

- Discussion

  - Identified RAN2 issues for IoT NTN TDD

In this section, RAN2 impacts are discussed considering RAN1 progress and agreements. In the proposal part, RAN2 issues to be discussed with high priority (e.g., issues that is less dependent on RAN1 progress) are identified.

Since the IoT NTN TDD pattern with a period of 9 radio frames is not aligned with the legacy NB-IoT periodicity(e.g. H-SFN period, MIB periodicity, SIB1 periodicity, Paging DRX cycle, NPRACH periodicity etc) and long “G” period, which may lead to the situation that channel and/or signal can be overlapped with invalid subframes. In order to support a TDD pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period, RAN1 has identified the following issues[3]:

Agreement The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.): - SIB1-NB - Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31) - Paging - NPRACH / RAR - NPSS/NSSS/NPBCH - NPDCCH, NPUSCH, NPDSCH - Note: Study for the above channels / signals may continue during the normative phase. - It is up to companies to perform link level evaluations for some of the aspects above.

Below gives some analysis of RAN1 identified issues and its RAN2 impact, as well as additional RAN2 issues.

Enhancements on NPSS/NSSS/NPBCH and/or SIB1-NB reception

Whether to enhance NPSS/NSSS/NPBCH and/or SIB1-NB reception depends on whether downlink coverage enhancement (e.g. reception with repetitions) is necessary for IoT-NTN TDD or not. For example, in the current 3GPP NB-IoT TDD system, the UE's MIB-TDD-NB uses a fixed schedule with a periodicity of 640 ms and repetitions are made within 640 ms. The first transmission of the MIB-TDD-NB is scheduled in subframe #9 of radio frames for which the SFN mod 64 = 0 and repetitions are scheduled in subframe #9 of all other radio frames. The transmissions are arranged in 8 independently decodable blocks of 80 ms duration. Since there is a MIB block transmission in each frame and each transmission is a independently decodable block, if robust reception and/or coverage enhancement is not considered for MIB reception (i.e., UE can receive and decode MIB based on only one transmission), the MIB transmission mechanism can work in IoT NTN TDD system.

In the current specification, the SystemInformationBlockType1-NB (SIB1-NB) uses a fixed schedule with a periodicity of 2560 ms, and the SIB1-NB transmission occurs every other frame in 16 continuous frames. Although the SIB1 periodicity of 2560 ms is not aligned with the TDD pattern period of 90ms, if robust reception and/or coverage enhancement is not considered for SIB1 reception, SIB1 transmission mechanism can still work in IoT NTN TDD system for dense repetition.

It is understood that RAN1 will evaluate the necessity of MIB/SIB1 repetition in IoT NTN TDD system considering the link budget as well as performance requirement. Therefore discussion on enhancements on MIB and SIB1 transmission mechanism can wait for more progress in RAN1.

Observation 1a: If coverage enhancement (e.g. reception with repetitions) is not needed for MIB and SIB1 reception, the existing MIB and SIB1 transmission mechanism can work in IoT NTN TDD system.

Observation 1b: Discussion on optimization of the MIB and SIB1 transmission mechanism in IoT NTN TDD system can wait for more RAN1 progress, e.g., on the necessity of MIB/SIB1 repetition in IoT NTN TDD system.

RACH adaption

In the current NB-IoT specification, NPRACH periodicity value range is [ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560], which cannot match the TDD pattern with a period of 90ms. To deal with these resource mismatch issue, RAN1 has the following agreements:

Agreement RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-U NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of U NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes. - Impact on segmented pre-compensation.

Considering the SFN_id is used for RA-RNTI calculation (e.g. RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id), postponing a preamble transmission when it overlaps with non-U NB-IoT subframes may lead to RA-RNTI confusion. Therefore it  may not be ideal to postpone a preamble transmission to deal with the mismatch issue between NPRACH periodicity and IoT-NTN TDD pattern with a period of 90ms.

Observation 2: Postponing a preamble transmission when it overlaps with non-U NB-IoT subframes may lead to RA-RNTI confusion.

Dropping a preamble transmission when they are overlapped with non-U NB-IoT subframes may lead to decrease of NPRACH capacity. Assuming the preamble format and RO mapping is unchanged, when the NPRACH periodicity is set to 40ms then there will be only one valid NPRACH occasion every 36 radio frames considering TDD pattern period of 90ms where D=U=8 with a fixed guard period, because 8/9 configured NPRACH resource will overlap with non-U NB-IoT subframes. As a consequence, the NPRACH capacity will be decreased to 11.11% from the configured NPRACH capacity, as shown in the following figure(the “RO” with red text is dropped for NPRACH resource overlaps with non-U NB-IoT subframes). An example is given as below assuming first RO for preamble transmission locates in the first subframe.

Observation 3: Dropping a preamble transmission when it overlaps with non-U NB-IoT subframes may lead to decrease of NPRACH capacity.

Considering that the preamble format is specified in RAN1 specification[4], RAN2 can wait for RAN1 conclusion on whether new preamble format will be introduced for IoT-NTN TDD system before discussion possible impacts on higher layer RACH procedure, e.g., RA-RNTI calculations.

Observation 4: RAN2 can wait for RAN1 conclusion on whether new preamble format will be introduced for IoT-NTN TDD system before discussion possible impacts on higher layer RACH procedure, e.g., RA-RNTI calculations.

dynamic scheduling

For dynamic scheduling, when UE receives a NPDCCH for UL scheduling, based on the current current offset between NPDCCH and NPUSCH, the NPUSCH transmission will overlap with non-U NB-IoT subframes; when UE receives a NPDCCH for DL scheduling, based on the current current offset between NPDCCH and NPDSCH, the NPDSCH transmission may overlap with non-D NB-IoT subframes. How to deal with these overlap issue depends on whether new offset between NPDCCH and NPUSCH/NPDSCH will be defined, and is more relevant to RAN1, therefore no need to be proactively discussed in RAN2.

Observation 5: For handling the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPDSCH with non-D NB-IoT subframes in dynamic scheduling, the discussion is more relevant to RAN1 therefore no need to be proactively discussed in RAN2.

Solutions to deal with offset between the 90ms TDD pattern and the 10240ms SFN period

Agreement in RAN1#119 The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Furthermore in RAN1#119, solutions have been identified for the offset between the 90ms TDD pattern and the 10240ms SFN period, which are shown as above. It can be observed that above options could lead to different understanding on H-SFN synchronization between eNB and MME, which will have impact on idle mode eDRX, therefore RAN2 should carefully discuss it from upper layer point of view.

Paging

For the paging monitoring, current paging DRX cycle(T) value range is [1280ms, 2560ms, 5120ms or 10240ms], which does not match with the IoT-NTN TDD pattern with a period of 90ms. Since the PF and PO determination is specified in RAN2, how to determine the PF and PO in IoT NTN TDD system should be discussed in RAN2.

Besides these, the following RAN2 related issue should also be considered by RAN2:

- UL SPS transmission:

- Since UL SPS in NB IoT is only introduced for BSR and the discussion was led by RAN2, therefore RAN2 can discuss possible adaptions without input from RAN1. Based on the current specification, the value range of semiPersistSchedIntervalUL is {sf128, sf256, sf512, sf1024, sf1280, sf2048, sf2560, sf5120}, which is not aligned with the IoT NTN TDD pattern with a period of 9 radio frames, may lead UL SPS overlap with non-U NB-IoT subframes. Possible enhancements can be discussed in RAN2.

- RAN2 timers in IoT NTN TDD system

- Considering the long “G” period in IoT NTN TDD pattern with a period of 9 radio frames, whether the existing value ranges of higher layer timers are sufficient in IoT NTN TDD system should be considered in RAN2.

Based on above analysis, below proposal is made:

Proposal 1a: Below issues can be discussed in RAN2:

- How to handle the offset between the 90ms TDD pattern and the 10240ms SFN period.

- Paging adaption, e.g., how to determine the PF and PO in IoT NTN TDD system.

- UL SPS transmission mechanism when it overlaps with non-U NB-IoT subframes.

- Higher layer timers related issue.

Proposal 1b: Below issues can wait for more progress in RAN1:

- Enhancements on NPSS/NSSS/NPBCH and/or SIB1-NB reception

- RACH adaptions caused due to ROs overlapped with non-U NB-IoT subframes (e.g., RA-RNTI calculation)

  - Initial analysis on identified RAN2 issues

In below section, initial analysis on identified RAN2 issues are given with corresponding proposals.

Issue 1: How to deal with the offset between the 90ms TDD pattern and the 10240ms SFN period

RAN1#119 Agreements The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Observation 6: Below options are identified by RAN1 to deal with the offset between he 90ms TDD structure and the 10240ms SFN, which could have higher layers impacts on idle mode eDRX:

- Option 1: Always the same across H-SFNs.

  - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9.

  - Option 1-2: H-SFN duration is unchanged

- Option 2: Different across some H-SFNs.

  - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB)

  - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Before down-selection of above RAN1 identified options, RAN2 shall first discuss whether to support extended idle mode DRX(eDRX) in IoT-NTN TDD network since the impacts are quite different. Considering that UE power saving is vital for NB-IoT UEs, and idle mode eDRX is a basic mechanism for UE power saving in IoT system, idle mode eDRX should be supported in IoT-NTN TDD network.

Observation 7: Idle mode eDRX is a basic mechanism for UE power saving in IoT system, which is essential for NB-IoT UEs power saving, therefore shall be supported.

Proposal 2: RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

If idle mode eDRX is confirmed to be supported in IoT-NTN TDD network, based on SA specifications, H-SFN level synchronization between eNodeB and MME will be necessary, as shown in Appendix[5][6]. But option 1-1 changes the H-SFN duration(e.g. SFN value range), and option 2-2 changes the total number of H-SFN (e.g. H-SFN value range), which will impact the H-SFN level synchronization between eNodeB and MME, therefore SA specification need be updated and the feasibility should be evaluated in SA2 and/or CT1.

Observation 8: If idle mode eDRX is supported in IoT-NTN TDD network, change of H-SFN duration(Option 1-1) and/or H-SFN total number(option 2-2) will impact SA specification and the feasibility should be evaluated in SA2 and/or CT1 first.

Considering RAN1 might not be aware of this situation it is suggested to send RAN1 an LS to inform them about the issues so that they can evaluate all options with whole picture.

Proposal 3: RAN2 send an LS to RAN1 to indicate that idle mode eDRX will be supported in IoT-NTN TDD network. And option 1-1 and 2-2 will impact H-SFN level synchronization of idle mode eDRX mechanism specified in SA2 and CT1’ specification, where the feasibility shall be evaluated by SA2 and CT1 first.

Before receiving confirmation from RAN1/SA2/CT1 on the feasibility of option 1-1, and option 2-2, RAN2 can study possible RAN2 impacts based on option 1-2 and option 2-1.

For option 2-1, additional indication in SIB is needed for UE to dynamically determine the TDD pattern start occasion in a H-SFN, how to save UE power saving to reduce frequent SIB (re)acquiring (e.g. for the indication) to determine the TDD pattern start occasion in each H-SFN should be considered.

Observation 9: If option 2-1 is selected, how to reduce frequent SIB (re)acquiring should be considered for UE UE power saving consideration.

While using option 1-2, the UE’s behaviour is as legacy and simple, but it may lead to partial IoT-NTN TDD pattern in each H-SFN (e.g. incomplete TDD pattern in the end of each H-SFN, which will include only 7 radio frames), which is less resource efficient.

Observation 10: Using option 1-2(e.g. H-SFN duration is unchanged) may lead to partial IoT-NTN TDD pattern in each H-SFN, which is less resource efficient.

Based on above analysis, below proposal is made:

Proposal 4: For handling of offset between the 90ms TDD pattern and the 10240ms SFN period, RAN2 study potential RAN2 impact of option 1-2 or option 2-1 of observation 6, taking UE power saving and resource efficiency into account.

Issue 2: Paging occasion determination

In the current NB-IoT specification, the paging DRX cycle(T) value range is [1280ms, 2560ms, 5120ms or 10240ms], which does not match with the IoT-NTN TDD pattern with a period of 90ms. And the nB value range is [fourT, twoT, oneT, halfT, quarterT, one8thT, one16thT, one32ndT, one64thT, one128thT, one256thT, one512thT, one1024thT], UE determines it's Paging frame based on UE-ID, DRX cycle(T) and nB value. Thus, when the nB value range is fourT, twoT or oneT, each radio frame may be used as a UE's paging frame. But in IoT-NTN TDD pattern, some frame may have no DL subframe, which cannot be used as paging frame, as shown in the following figure.

Observation 11: Based on the current NB-IoT paging mechanism, the DRX cycle does not match with the IoT-NTN TDD pattern with a period of 90ms, and some nB values may lead to an invalid DL radio frame being selected by the UE as paging frame.

To deal with these resource mismatch issue, RAN1 has the following agreement:

Agreement RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-D NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of D NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. - The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications

Per our understanding, “The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications” is not a solution, but a explanation for the “non-D NB-IoT subframes”, thus, possible options are summarized in observation 12.

Observation 12:Below options are identified by RAN1 to deal with paging frame being mapped to non-DL NB IoT frames:

- New periodicities to align with the TDD structure.

- Postponement of a channel when it overlaps with non-D NB-IoT subframes.

- Restriction of a channel to be fully confined within a single set of D NB-IoT subframes.

- Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes.

In our consideration:

- Using “Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes” mechanism may lead to lose of paging, which is not acceptable.

- Using “Postponement of a channel when it overlaps with non-D NB-IoT subframes” mechanism may lead paging collision(e.g. lots of UEs are postponed to one single frame for paging reception), which may lead to reduce of paging capacity, therefore is not desired.

Observation 13: “Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes” and “Postponement of a channel when it overlaps with non-D NB-IoT subframes” mechanism is not suitable for Paging occasion determination.

If defining new DRX cycle(T) (e.g. 36 radio frames, 72 radio frames, 144 radio frames, 288 radio frames, or 1024 radio frames) to align with the TDD structure and restrict that the PO occasions(e.g. restrict the nB value range and update the PO determination mechanism) only locates in NB-IoT DL subframes, the Paging occasion determination issue can be solved perfectly. This solution is also more aligned with current UE behavior in determining the PFs and POs.

Proposal 5: Define new DRX cycle(T) (e.g. 36 radio frames, 72 radio frames, 144 radio frames, 288 radio frames, or 1024 radio frames) and restrict that the PO occasions only locates in NB-IoT DL subframes to solve the Paging occasion determination issue in IoT-NTN TDD system.

Issue 3: Impact on UL SPS transmission

In NB-IoT, uplink SPS is only used for BSR transmission. In case PS transmission is dropped, SR procedure and/or dynamic scheduling can still be used for BSR transmission, and the service performance will not be impacted too much. Therefore dropping the SPS transmission when the UL SPS overlaps with non-U NB-IoT subframes is feasible.

Proposal 6: UL SPS transmission is dropped when the UL SPS overlaps with non-U NB-IoT subframes.

Issue 4: Impact on higher layer Timers

In NB-IoT system, there are two type of timers as follows:

- Timer with unit of PP(PDCCH period): consider the PDCCH period is defined as “the interval between the start of two consecutive PDCCH occasions and depends on the currently used PDCCH search space, as specified in TS 36.213. A PDCCH occasion is the start of a search space and is defined by subframe k0 as specified in clause 16.6 of TS 36.213. The calculation of number of PDCCH-subframes for the timer configured in units of a PDCCH period is done by multiplying the number of PDCCH periods with npdcch-NumRepetitions-RA when the UE uses the common search space or by npdcch-NumRepetitions when the UE uses the UE specific search space”[7]. Since only downlink subframes will be calculated as PDCCH-subframes (For NB-IoT UE, all subframes that are part of the NPDCCH search space represent PDCCH-subframes among all NB-IoT downlink subframes, including those which the UE is not required to monitor as specified in clause 16.6 of TS 36.213)[5], e.g. the “G” subframes and “U” subframes will not be countered for the timer. Thus, the value range of timer with unit of PP is enough, e.g. not impacted by the IoT-NTN TDD pattern.

- Observation 14: Only DL subframes will be counted towards timers with length unit of PP (PDCCH period), therefore the existing value range of timers with unit of pp are enough for IoT-NTN TDD, and no extension is needed.

- Proposal 7: RAN 2 confirms that the existing value range of timers with unit of pp are enough for IoT NTN TDD.

- Timer with unit of ms or s(milliseconds or seconds): When determine the value range of rel-15 NB-IoT TDD timer with unit of ms or s, the maximal PUSCH repetition number(e.g. 128 repetitions) , the maximal PDCCH repetition number(e.g. 2048 repetitions) and the maximal PDSCH repetition number(e.g. 2048 repetitions) are considered, if coverage enhancement(e.g. transmission with repetitions) is not considered in IoT-NTN TDD pattern with a period of 9 radio frames(e.g. the repetition number of all channels will be 1), the existing value range of Timer with unit of ms or s is enough for IoT-NTN TDD; but if same coverage enhancement(e.g. UL with maximal 128 repetitions and DL with maximal 2048repetitions ) are needed, the existing value range of Timer with unit of ms or seconds will be insufficient and need to be extended.

Observation 15: Whether the value range of timer with unit of ms or seconds need to be extended depends on whether coverage enhancement(e.g. transmission with repetitions) is needed, and can wait for more RAN1 progress.

Because there are several RAN2 issues require RAN1’ inputs on whether coverage enhancements (e.g., supported repetition numbers) is supported or not , it is suggested to ask RAN1 about more input on this aspect.

- Proposal 8a: RAN2 send LS to consult RAN1 on whether coverage enhancement(e.g. transmission with repetitions) is needed in IoT-NTN TDD system. If needed, what’s the value range of supported repetitions.

- Proposal 8b: RAN2 postpone the discussion on extending the value range of timers with unit of ms or seconds until receiving RAN1’s response on whether to support coverage enhancement(e.g. transmission with repetitions) or not.

- Conclusion

The following observations and proposals are made:

Observation 1a: If coverage enhancement (e.g. reception with repetitions) is not needed for MIB and SIB1 reception, the existing MIB and SIB1 transmission mechanism can work in IoT NTN TDD system.

Observation 1b: Discussion on optimization of the MIB and SIB1 transmission mechanism in IoT NTN TDD system can wait for more RAN1 progress, e.g., on the necessity of MIB/SIB1 repetition in IoT NTN TDD system.

Observation 2: Postponing a preamble transmission when it overlaps with non-U NB-IoT subframes may lead to RA-RNTI confusion.

Observation 3: Dropping a preamble transmission when it overlaps with non-U NB-IoT subframes may lead to decrease of NPRACH capacity.

Observation 4: RAN2 can wait for RAN1 conclusion on whether new preamble format will be introduced for IoT-NTN TDD system before discussion possible impacts on higher layer RACH procedure, e.g., RA-RNTI calculations.

Observation 5: For handling the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPDSCH with non-D NB-IoT subframes in dynamic scheduling, the discussion is more relevant to RAN1 therefore no need to be proactively discussed in RAN2.

Observation 6: Below options are identified by RAN1 to deal with the offset between he 90ms TDD structure and the 10240ms SFN, which could have higher layers impacts on idle mode eDRX:

- Option 1: Always the same across H-SFNs.

  - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9.

  - Option 1-2: H-SFN duration is unchanged

- Option 2: Different across some H-SFNs.

  - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB)

  - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Observation 7: Idle mode eDRX is a basic mechanism for UE power saving in IoT system, which is essential for NB-IoT UEs power saving, therefore shall be supported.

Observation 8: If idle mode eDRX is supported in IoT-NTN TDD network, change of H-SFN duration(Option 1-1) and/or H-SFN total number(option 2-2) will impact SA specification and the feasibility should be evaluated in SA2 and/or CT1 first.

Observation 9: If option 2-1 is selected, how to reduce frequent SIB (re)acquiring should be considered for UE UE power saving consideration.

Observation 10: Using option 1-2(e.g. H-SFN duration is unchanged) may lead to partial IoT-NTN TDD pattern in each H-SFN, which is less resource efficient.

Observation 11: Based on the current NB-IoT paging mechanism, the DRX cycle does not match with the IoT-NTN TDD pattern with a period of 90ms, and some nB values may lead to an invalid DL radio frame being selected by the UE as paging frame.

Observation 12:Below options are identified by RAN1 to deal with paging frame being mapped to non-DL NB IoT frames:

- New periodicities to align with the TDD structure.

- Postponement of a channel when it overlaps with non-D NB-IoT subframes.

- Restriction of a channel to be fully confined within a single set of D NB-IoT subframes.

- Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes.

Observation 13: “Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes” and “Postponement of a channel when it overlaps with non-D NB-IoT subframes” mechanism is not suitable for Paging occasion determination.

- Observation 14: Only DL subframes will be counted towards timers with length unit of PP (PDCCH period), therefore the existing value range of timers with unit of pp are enough for IoT-NTN TDD, and no extension is needed.

Observation 15: Whether the value range of timer with unit of ms or seconds need to be extended depends on whether coverage enhancement(e.g. transmission with repetitions) is needed, and can wait for more RAN1 progress.

Proposal 1a: Below issues can be discussed in RAN2:

- How to handle the offset between the 90ms TDD pattern and the 10240ms SFN period.

- Paging adaption, e.g., how to determine the PF and PO in IoT NTN TDD system.

- UL SPS transmission mechanism when it overlaps with non-U NB-IoT subframes.

- Higher layer timers related issue.

Proposal 1b: Below issues can wait for more progress in RAN1:

- Enhancements on NPSS/NSSS/NPBCH and/or SIB1-NB reception

- RACH adaptions caused due to ROs overlapped with non-U NB-IoT subframes (e.g., RA-RNTI calculation)

Proposal 2: RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

Proposal 3: RAN2 send an LS to RAN1 to indicate that idle mode eDRX will be supported in IoT-NTN TDD network. And option 1-1 and 2-2 will impact H-SFN level synchronization of idle mode eDRX mechanism specified in SA2 and CT1’ specification, where the feasibility shall be evaluated by SA2 and CT1 first.

Proposal 4: For handling of offset between the 90ms TDD pattern and the 10240ms SFN period, RAN2 study potential RAN2 impact of option 1-2 or option 2-1 of observation 6, taking UE power saving and resource efficiency into account.

Proposal 5: Define new DRX cycle(T) (e.g. 36 radio frames, 72 radio frames, 144 radio frames, 288 radio frames, or 1024 radio frames) and restrict that the PO occasions only locates in NB-IoT DL subframes to solve the Paging occasion determination issue in IoT-NTN TDD system.

Proposal 6: UL SPS transmission is dropped when the UL SPS overlaps with non-U NB-IoT subframes.

- Proposal 7: RAN 2 confirms that the existing value range of timers with unit of pp are enough for IoT NTN TDD.

- Proposal 8a: RAN2 send LS to consult RAN1 on whether coverage enhancement(e.g. transmission with repetitions) is needed in IoT-NTN TDD system. If needed, what’s the value range of supported repetitions.

- Proposal 8b: RAN2 postpone the discussion on extending the value range of timers with unit of ms or seconds until receiving RAN1’s response on whether to support coverage enhancement(e.g. transmission with repetitions) or not.

- References

- Draft_Minutes_report_RAN1#119_v030

- RP-243293 Revised WID on introduction of IoT-NTN TDD mode, RAN#106

- Final_Minutes_report_RAN1#118b_v100

- 3GPP T3GPP TS 36.211 V18.0.1 (2024-04), Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation(Release 18)

- 3GPP TS 23.682 V19.1.0 (2024-12), Architecture enhancements to facilitate communications with packet data networks and applications

- 3GPP TS 24.301 V19.1.0 (2024-12), Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS);Stage 3(Release 19)

- 3GPP T3GPP TS 36.321 V18.3.0 (2024-09), Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification(Release 18)

- Appendix

##### <TS 23.682> ##### 4.5.13.3.2 Loose Hyper SFN synchronization NOTE: This clause applies for extended DRX cycle lengths of 10.24s or longer. In order for the UE to be paged at roughly similar time, the H-SFN of all eNodeBs and MMEs should be loosely synchronized. Each eNodeB and MME synchronizes internally the H-SFN counter so that the start of H-SFN=0 coincides with the same a preconfigured time epoch. If eNodeBs and MMEs use different epochs, e.g. due to the use of different time references, the GPS time should be set as the baseline, and the eNodeBs and MMEs synchronize the H-SFN counter based on the GPS epoch considering the time offset between GPS epoch and other time-reference epoch a preconfigured time. It is assumed that eNodeBs and MMEs are able to use the same H-SFN value with accuracy in the order of legacy DRX cycle lengths, e.g. 1 to 2 seconds. There is no need for synchronization at SFN level. There is no signalling between network nodes required to achieve this level of loose H-SFN synchronization.

##### <TS 24.301> #### 5.6.2.2 Paging for EPS services ##### 5.6.2.2.1 Paging for EPS services through E-UTRAN using S-TMSI 5.6.2.2.1.1 General ... For the UE using eDRX, the network initiates the paging procedure when NAS signalling messages, cdma2000® signalling messages or user data is pending to be sent to the UE within the paging time window. If NAS signalling messages, cdma2000® signalling messages or user data is pending to be sent to the UE outside the paging time window and the eDRX value that the network provides to the UE in the Extended DRX parameters IE during the last attach procedure or the last tracking area updating procedure is not all zeros (i.e. the E-UTRAN eDRX cycle length duration is higher than 5.12 seconds), the network initiates the paging procedure at T time ahead of the beginning of the next paging time window. NOTE 2: T time is a short time period based on implementation. The operator can take possible imperfections in the synchronization between the CN and the UE into account when choosing T time. Figure 5.6.2.2.1.1: Paging procedure using S-TMSI