3GPP TSG-RAN WG2 Meeting #129							        R2-2500778

Athens, Greece, 17 – 21 Feb, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	Random-access procedure for A-IoT

Agenda item:	8.2.3

Document for: Discussion and Decision

# Introduction

A-IoT WI has the following objective for to specify random access procedure in MAC:

A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

In this paper, we discuss the details of random-access procedure for A-IoT.

# Unified RACH solution

RAN2 needs to decide between solution 1 (3-step RA only) and solution 3 (unified RA). The overall procedure for the two mechanisms is depicted in Figure 1 below.

Figure 1: Unified RACH and 3-step RA (with collisions)

It should be noted that in Rel-19 there is at most one paging ID included in a given paging message. This could either be a group ID or a single UE ID. The unified RACH has a clear advantage for the case when a single UE is paged (no collision). However, the question is regarding the operation of the system in presence of contention. In a system that operates with reasonable degree of contention, the overall performance is still very heavily biased towards unified RACH. This is depicted in Figure 2 below where the system is still operating normally (in the linear region of the slotted aloha contention procedure). In this region, the gains from unified RACH are significant and these gains scale linearly with the number of devices.

Figure 2: Comparison between various RACH options under normal operating conditions

If the offered load to the system is increased significantly, then there will be more collisions at MSG1 phase and this will impact the performance. However, it is important to understand where this point of crossover between unified RACH and 3-step RACH happens. Figure 3 below shows the overall region of operation.

Figure 3: Comparison between various RACH options in a system with a varying degree of contention

As can be seen from the above, even until a relatively high degree of contention rates, unified RACH is still much better. Furthermore, in the normal operating region, the gains of unified RACH scale linearly with the number of devices, so, under these conditions, the more the number of devices the larger the gains from unified RACH hence. Only if the system is operating in the extreme non-linear part of the slotted aloha protocol, 3-step RA is slightly better. However, operating the system in such region is unrealistic as can be seen from the extremely long inventory times and very large group size of paged devices (inventory times in days and paged devices in 100s of thousands). In other words, in this region, there is so much offered traffic that there is too much contention and not enough good-put and only in this region, 3-step RACH will have marginal benefits over unified RACH.

Thus, with a contention control procedure (such as the Q algorithm in RFID) in place during the RACH procedure, under any operating condition that can be considered to be reasonable, unified RACH has significant advantage over 3-step RACH. The overall times to complete inventory procedure will be significantly lower under such conditions with unified RACH (see Figure 2).

Observation 1: Under normal operating conditions, unified RACH offers significantly better performance over 3-step RACH and the gains of unified RACH scale linearly with the load under these conditions

Observation 2: The reader can control the contention levels by using a contention control protocol (such as Q algorithm in RFID) to ensure that the system operates under normal contention levels.

Observation 3: When there is extremely heavy contention for MSG1, 3-step RACH will be slightly better than unified RACH, but in this region, the system is unstable and the overall times to complete the procedure will increase exponentially. Operating the system in this region is unrealistic and will never happen in practice

Then, we have to consider the coverage and complexity issues with supporting unified RACH. If indeed, it is complex for the device to support more than one RACH option, it would be logical that one would select the option that is more optimal (i.e. the unified RACH). It is unclear why the option that would not result in reasonable performance is considered as a baseline. However, as a compromise, it was agreed to consider both options in the standard, but the reader should have a choice to choose whichever option to use. It should be noted that under normal conditions the reader will likely never use the 3-step RACH option in the field.

With regards to coverage, supporting a payload size of RN16+UEID will result in a slightly smaller coverage region compared to including just the UEID. However, it is expected that the initial UEID is anyway received by the reader. So, the only additional overhead here is RN16. It is not reasonable to design the system such that the overall coverage is limited by MSG1/MSGA coverage. As can be seen from RAN1 evaluations, the DL (i.e. R2D link is the bottleneck for coverage). So, coverage penalty of the small addition of RN16 in MSGA is not the bottleneck for system when it results in overall gains anyway. If indeed accommodating this small additional overhead is a showstopper, then one should consider going for a 2-step RACH like approach and go with just UEID in MSG1. This will provide the best performance and coverage is not compromised as can be seen from the results.

Based on the above, we propose the following:

Proposal 1: Unified RACH is supported in Rel-19

# Overall RACH procedure

# Procedure for MSG1 transmission

The random-access procedure for A-IoT uses slotted Aloha. Once the device randomly selects the access occasion for A-IoT MSG1, the device then needs to count the slots so that the device can determine when it is allowed to transmit. Counting the slots can be either based on a counter maintained at the device side or by a trigger that is received from the reader. Using a device-based counter only works if the device can maintain a stable clock so that the slot boundaries are synchronized between the reader and the device and among other devices. However, the devices for A-IoT may not have a stable clock and hence a safer approach is to use a trigger from the reader for counting down.

Furthermore, the reader is aware of the level of contention based on the number of devices that are paged and number of devices that respond. So, the reader may send the trigger for counting down the slots at non-uniform intervals to ensure that the contention is minimized. This is also necessary since the time taken to transmit a given message from different devices may not be the same. Thus, a counter that is decremented autonomously at the device side is not suitable to determine the access occasion. In case of RFID a similar mechanism is used and the count-down procedure is based on explicit triggers from the reader.

Based on the above, it is suggested to introduce a subsequent DL trigger message after the paging message to trigger the countdown of the slot counter at the device.

Proposal 2: After the device randomly selects an access occasion for A-IoT MSG1 transmission, the device shall use a trigger from the reader to count down the slot counter to determine the actual transmit occasion

# Unified procedure for contention resolution

After transmitting the first UL message, the device should simply listen to the DL. For the purpose of contention resolution, the reader echoes the received information in the first UL message in the response message in DL. Thus, the contents of the response message in DL should match the contents of the UL message transmitted by the UE in the first UL message. In order to simplify the device procedure and to enable a unified procedure from the device perspective, the contents of the response message should be limited to the minimum payload size of the first UL message which is common to all types of RACH procedures as described above. This means that the contents of the response message should contain at least the random number and is copied from the first N bits (e.g. first 16 bits) of the first UL message from the device regardless of the payload size of the first UL message. If the contents of the response message match the first N bits (e.g. 16 bits) that the device transmitted, then the device considers the contention resolution as successful regardless of the 2-step/3-step RACH procedure (i.e. regardless of the payload size of the first UL message).

The MSG1 may be lost and in this case the reader cannot echo the RN16. Although the assumption is that the device shall monitor PRDCH for receiving MSG2 with the RN16, we should discuss how long the device shall monitor PRDCH for its own RN16. There should be an upper bound on the maximum time during which the device shall monitor PRDCH for its RN16. If the device doesn’t receive RN16 in R2D direction within this time it should, start monitoring PRDCH for retrigger messages to retransmit MSG1.

Finally, when two devices choose the same random number, then there will be contention and resolving this may need further discussion. In NR, the contention resolution is based on the network echoing part of the upper layer device ID and the UE will then initiate a new RACH procedure if this happens. In case of A-IoT, since the device cannot initiate a new RACH procedure anyway, designing a complex contention resolution mechanism for the rare case of the devices choosing same random number is unnecessary. Instead, when this happens, the reader may simply not schedule the colliding devices in the current paging round and use the next paging round to page these devices. Thus, no further optimization is needed for this case.

Proposal 3: After transmitting MSG1, the device listens to DL (for receiving MSG2) in all RACH options

Proposal 4: If MSG2 with the transmitted RN16 is not received within a specified duration, the device shall start monitoring PRDCH for retrigger message to retransmit MSG1

Proposal 5: Regardless of the RACH option (unified/3-step/CBRA/CFRA), contention resolution is deemed successful when MSG2 containing RN16 from MSG1 is received

Proposal 7: If two devices choose the same random number in MSG1, it is up to the reader to handle the collision (i.e. no further mechanism is specified to resolve this within the current paging round)

# Procedure for CFRA

In Rel-19 only a single UE ID (either a device ID or a group ID) is included in MSG0. Hence, CFRA can be assumed to be applicable for the case when the paging ID matches the device ID (i.e. single device being paged). Of course, in case of a group ID, if the number of RACH resources is equal to the group size, then there is possibility to provide a one-to-one mapping between the devices and the RACH resources. However, providing such mapping for a large number of devices increases the MSG0 size. On the other hand, if a fixed rule is used, then this is not scalable and may be suboptimal for CBRA. Thus, it is proposed to use CFRA only for the case when a single device ID is included in MSG0 (i.e. not for group paging).

Proposal 8: For Rel-19, CFRA is applicable only when a single ID corresponding to a device ID (i.e. not a group ID) is included in paging message

From the device perspective, if there is a unique UL resource for the transmission of the first UL message, then this resource shall be chosen for transmission of the UL message. On the other hand, if there is more than one UL resource available (CFRA), the device shall choose one of the UL resources randomly and transmit the first UL message (CBRA). After choosing the resource for transmitting the first UL message, there should be no further difference between CFRA and CBRA from the device perspective.

Proposal 9: Subsequent steps in CFRA (after MSG0 transmission) are same as CBRA

# Conclusion

The following proposals are made:

Observation 1: Under normal operating conditions, unified RACH offers significantly better performance over 3-step RACH and the gains of unified RACH scale linearly with the load under these conditions

Observation 2: The reader can control the contention levels by using a contention control protocol (such as Q algorithm in RFID) to ensure that the system operates under normal contention levels.

Observation 3: When there is extremely heavy contention for MSG1, 3-step RACH will be slightly better than unified RACH, but in this region, the system is unstable and the overall times to complete the procedure will increase exponentially. Operating the system in this region is unrealistic and will never happen in practice

Proposal 1: Unified RACH is supported in Rel-19

Proposal 2: After the device randomly selects an access occasion for A-IoT MSG1 transmission, the device shall use a trigger from the reader to count down the slot counter to determine the actual transmit occasion

Proposal 3: After transmitting MSG1, the device listens to DL (for receiving MSG2) in all RACH options

Proposal 4: If MSG2 with the transmitted RN16 is not received within a specified duration, the device shall start monitoring PRDCH for retrigger message to retransmit MSG1

Proposal 5: Regardless of the RACH option (unified/3-step/CBRA/CFRA), contention resolution is deemed successful when MSG2 containing RN16 from MSG1 is received

Proposal 7: If two devices choose the same random number in MSG1, it is up to the reader to handle the collision (i.e. no further mechanism is specified to resolve this within the current paging round)

Proposal 8: For Rel-19, CFRA is applicable only when a single ID corresponding to a device ID (i.e. not a group ID) is included in paging message

Proposal 9: Subsequent steps in CFRA (after MSG0 transmission) are same as CBRA

# References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#102

- 3GPP TR 38848 V18.0.0 (2023-09), Study on Ambient IoT (Internet of Things) in RAN

- 3GPP TR 23.700-13 V0.2.0 (2024-03), Study on Architecture support of Ambient power-enabled Internet of Things

# Annex (simulation assumptions)

| RN16 length | 16 bits |
| --- | --- |
| UEID length | 100 bits |
| ReTrigger msg length | 100 bits |
| Ta | (switching time from DL to UL) 125 µsec |
| Tb | (switching time from DL to UL) 62.5 µsec |
| Data Rate | 100 kbps for successful transmissions (no collisions) 0 kbps when there is a collision (i.e. in case of collision all packets are lost) |
| Q | Qinit (Initial value of Q): 1 (number of slots in a frame = 2Qinit= 2) Qmax (Max value of the Q): 15 (number of slots in a frame = 2Qmax = 32768) Q is incremented by 1 if there is a collision in a frame and is decremented by 1 if more than half of the frame is empty |