3GPP TSG-RAN2#129									         R2-2500783

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	DSR enhancements for XR

Agenda item:	8.7.4.2

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for RAN2 to specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG, to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

In RAN2#126 meeting, the following related agreements has been made [3]:

DSR enhancements – multiple thresholds and grouping of data - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 meeting, the following related agreements has been made [4]:

Enhanced DSR contents - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In RAN2#127bis meeting, the following related agreements has been made [5]:

- We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In RAN2#128 meeting, the following related agreements has been made [6]:

- Let the network configure the triggering and reporting thresholds without constraints. - RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR. - The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. - The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this contribution we will further investigate the possible DSR enhancements per this objective.

# Discussion

It has been agreed in RAN2#127 meeting that network should be able to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer sizes per LCG, but there will be a single triggering threshold. The value range of remainingTimeThreshold-r18 is INTEGER (1..64) ms. Very small remaining time (e.g. 1ms, 2ms etc ) are not useful as there is not enough time to report these such that network can receive the DSR, process this and respond with scheduling. Considering the useful range of the remaining times, at most three remaining time thresholds seem a reasonable choice of split and this. Only these configured remaining time thresholds along with the buffered data associated with these remaining time will be reported. For each of the portions reported, the eDSR should report the corresponding buffer size and the shortest remaining time among the PDCP SDUs in this portion, which is shown in the following figure. And for flexibility, variable number of remaining time thresholds (e.g.1, 2 or 3 thresholds) should be allowed. The total  number of remaining time thresholds to be used can be configurable.

Proposal 1: For DSR enhancement, at most three remaining time thresholds can be configured for one LCG, and variable number of remaining time thresholds are allowed.

Considering that the data volume with remaining time larger than the largest reporting threshold can be reported by BSR, it is not necessary to report data volume with remaining time larger than the largest reporting threshold in the DSR.

Proposal 2: UE shall not report data volume with remaining time larger than the largest reporting threshold in the DSR.

In RAN2 #127bis meeting, it has been agreed that a one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. If the bit is used to indicate whether further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, then some unnecessary payload will be increased in some cases. E.g. when there are 3 reporting threshold, and there is data related to the smallest reporting threshold and the largest reporting threshold, but there is no data related to the in-between reporting threshold, if the bit should be set to indicate further pair of remaining time information and buffer size information is present, three pairs of remaining time and buffer size need be included in the eDSR, e.g. the pair of remaining time(what value to be set is not known) and buffer size 0 related to the in-between reporting threshold should be included, which will increase the eDSR payload. If the bit is used to indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, then the only two pairs of remaining time and buffer size need be included in the eDSR, e.g. the pair related to the in-between reporting threshold need not be included, which can save eDSR payload.

Observation 1: If the bit is used to indicate whether a certain pair of remaining time information and buffer size information is present, it can minimize the overhead.

Proposal 3: RAN2 confirm that the one-bit indication is used indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

To report with multiple pairs of remaining time and buffer size for one LCG, DSR MAC CE should be enhanced to report multiple pairs of remaining time and buffer size for one LCG. Considering that there is not enough “R” bits to extend the DSR MAC CE to indicate whether each pair of remaining time and buffer size is present for one LCG, and it is a rare case that DSR are triggered for multiple LCGs simultaneously, a new DSR MAC CE can be defined as follows:

| R | LCG ID |  | R | F3 | F2 | F1 | Oct 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BT1 | R | Remaining Time 1 |  |  |  |  | Oct 2 |
| Buffer size1 |  |  |  |  |  |  | Oct 3 |
| BT2 | R | Remaining Time 2 |  |  |  |  | Oct 4 |
| Buffer size2 |  |  |  |  |  |  | Oct 5 |
| BT3 | R | Remaining Time 3 |  |  |  |  | Oct 6 |
| Buffer size3 |  |  |  |  |  |  | Oct 7 |

Enhanced DSR MAC CE for reporting multiple pairs of remaining time and buffer size

In which:

-	LCG ID: This field identifies the group of logical channel(s) whose DSR is being reported.

-	Fi: This field indicates whether the pair of remaining time i and buffer size i are present. Fi is set to 0 to indicate that the Oct including BTi, remaining time i and buffer size i are absent; Fi is set to 1 to indicate that the Oct including BTi, remaining time i and buffer size i are present;

-	Remaining Time i: This field indicates the shortest remaining value of running PDCP discardTimer among all PDCP SDUs that are buffered for an LCG, with remaining time less than the remaining time threshold i but equal to or larger than the remaining time threshold i-1, but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

-	BT i: This field is present only if the corresponding LCG is configured with additionalBS-TableAllowed and the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the BT field set to 1 indicates that the buffer sizes with finer granularity are used to set the value of the Buffer Size field, while the BT field set to 0 indicates that the buffer sizes specified in legacy Table are used instead;

-	Buffer Size i: The Buffer Size i field indicates the total amount of UL data with remaining time less than the remaining time threshold i but equal to or larger than the remaining time threshold i-1 for an LCG after the MAC PDU has been built.

-	R: Reserved bit, set to 0.

With new DSR format, UE should report it’s capability on supporting enhanced DSR. Only if the UE reports it’s capability, multiple remaining time threshold for one LCG can be configured for the UE.

Proposal 4: To enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG, the following aspects are specified:

- Define new DSR MAC CE to report multiple pairs of remaining time and buffer.

- Introduce UE capability on support enhanced DSR.

In RAN2#127bis meeting, there is a FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds. This FFS depends on how to configure the multiple reporting thresholds. If additional reporting thresholds are configured, and the legacy threshold is always used as the smallest reporting threshold, then it can be based on UE implementation which DSR format is used. And if the multiple reporting thresholds are configured independent of the legacy threshold, then the new DSR should be used for the LCG when triggered by the new triggering threshold.

Proposal 5: RAN2 discuss how to configure the multiple reporting thresholds, e.g. additional reporting thresholds are configured or the multiple reporting thresholds are configured independent of the legacy threshold.

# Conclusion and proposals

The following observations/proposals are made:

Proposal 1: For DSR enhancement, at most three remaining time thresholds can be configured for one LCG, and variable number of remaining time thresholds are allowed.

Proposal 2: UE shall not report data volume with remaining time larger than the largest reporting threshold in the DSR.

Proposal 3: RAN2 confirm that the one-bit indication is used indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

Proposal 4: To enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG, the following aspects are specified:

- Define new DSR MAC CE to report multiple pairs of remaining time and buffer.

- Introduce UE capability on support enhanced DSR.

Proposal 5: RAN2 discuss how to configure the multiple reporting thresholds, e.g. additional reporting thresholds are configured or the multiple reporting thresholds are configured independent of the legacy threshold.

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1