3GPP TSG-RAN WG2 Meeting #129		R2-2500785

Athens, Greece, Feb. 17th – 21st, 2025

Title: 	Data rate control for XR applications

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.7.6

Document for:	Discussion and decision

# Introduction

In RAN#105 meeting[1], Revised WID on XR (eXtended Reality) for NR Phase 3 has been agreed and the following objective has been included:

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

At RAN2#127-bis[2], it was agreed that further analysis is needed to decide between per-QoS flow vs per DRB indication of XR rate control. It was also agreed to define a new table for bit rate recommendation with better granularity for XR applications.

At RAN2#128, the following agreements have been made[3]:

RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

In this paper, we will discuss the remaining issues for this and propose a way forward for the XR rate control in Rel-19.

# Discussion

Based on the agreements at RAN2#128, we need to discuss further how the gNB can configure the XR rate control mechanism. The most flexible way to control the XR rate is to define a per-QoS flow level XR rate control using a MAC CE. However, in order to specify such an approach, work may also be needed in RAN3 to support the necessary signalling between the CU and DU nodes in the network side. If it is preferred in RAN2 to avoid such impact, then one option is to configure the QoS flow level applicability using RRC and then using the MAC CE to perform the dynamic rate control. It should be noted that this makes it slightly less flexible for the gNB, in the sense that only one QoS flow per DRB can be rate controlled with this approach. However, the gNB can then also choose to map only one QoS flow that is subject to rate control to such a DRB.

Observation 1: The MAC based solution to signal the XR rate control on a per QoS flow basis is the most flexible option, but will require changes to RAN3 specs

Observation 2: If the RRC based approach (to configure which QoS flow in a DRB is subject to rate control) is adopted, then the gNB can map at most one XR flow that is subject to rate control per DRB

If there is no consensus to go with the per-QoS flow level rate control using a new MAC CE, then, RAN2 should go with the approach of RRC based solution hence.

This means that for a given DRB, RRC will configure which QoS flow is subject to rate control. Then the MAC CE will indicate the actual recommended bit rate. So, we suggest the following:

Proposal 1: For the RRC based solution, when more than one QoS flow is mapped to a given DRB, RRC can configure which one of these QoS flows is subject to rate control

Proposal 2: MAC level signalling is used then to enable XR rate control on a per DRB basis

# DL rate control

At RAN plenary, the rate control for DL was discussed further and it was agreed that RAN2 can continue working on the uplink solution and to liaise with SA2/SA4 if there are issues if the same approach would be extended for the DL as well, assuming the solution extension for DL. This also requires that the solution doesn’t require too much additional work in RAN2 to specify this.

The existing recommended bit rate MAC CE works for both UL and DL rate control. Whilst UL rate control is negotiated between the lower layers and application layer within the UE, the DL rate control requires application-level signalling between the UE and the application server. This is done by the UE by initiating end-to-end bit rate adaptation with its peer (UE or application server) using application-level signalling. Such signalling is feasible already with the existing recommended bit rate MAC control mechanism. Thus, by reusing this existing mechanism it is also possible to control the DL bit rates for XR applications. Thus, it is proposed not to artificially restrict the new rate control mechanism for RAN level recommended bit rate to just UL.

Based on the agreement at RAN plenary, it is proposed to send and LS to SA2/SA4 informing them RAN2 intends to reuse the new XR rate control mechanism for both UL and DL (similar to what is specified today).

Proposal 3: Send an LS to SA2/SA4 (CC RAN3) informing them that RAN2 intends to reuse the new XR rate control mechanism for both UL and DL (similar to what is specified today for the recommended bit rate MAC CE).

# Conclusion

Observation 1: The MAC based solution to signal the XR rate control on a per QoS flow basis is the most flexible option, but will require changes to RAN3 specs

Observation 2: If the RRC based approach (to configure which QoS flow in a DRB is subject to rate control) is adopted, then the gNB can map at most one XR flow that is subject to rate control per DRB

Proposal 1: For the RRC based solution, when more than one QoS flow is mapped to a given DRB, RRC can configure which one of these QoS flows is subject to rate control

Proposal 2: MAC level signalling is used then to enable XR rate control on a per DRB basis

Proposal 3: Send an LS to SA2/SA4 (CC RAN3) informing them that RAN2 intends to reuse the new XR rate control mechanism for both UL and DL (similar to what is specified today for the recommended bit rate MAC CE).

# References

- RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3	RAN#105

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1

- 3GPP TS 38.321, V18.2.0 (2024-06), NR; Medium Access Control (MAC) protocol specification