3GPP TSG-RAN WG2 Meeting #129	R2-2500806

Athens, Greece, 17th – 21st Feb, 2025

Agenda item:	8.7.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM enhancements for XR

Document for:	Discussion and Decision

1. Introduction

During the last meeting, RAN2 has discussed the interactions between MG skipping and RAN2 features such as DRX and DSR. It was agreed not to introduce any enhancement for the interactions. In this contribution, we will continue to consider the specific impacts in RAN2 specifications to support DCI-based MG skipping considering the conclusions from RAN1 and RAN4.

Besides, based on the RAN4 LS, semi-static solutions are considered as another alternative. We will also have some analysis on the semi-static solutions and give our views.

2. Discussion

2.1 Specification impacts for the DCI-based solution

From the RAN1 agreement, an explicit skip indication is carried by the physical layer DCI to indicate the MG skipping. While the indication is carried by the DCI, the exact UE procedure for UL/DL channels during the MG is performed by the MAC layer, e.g., as specified in TS 38.321 [1]. Moreover, the MG configuration is generated and maintained in the RRC layer.

Considering that the skip indication in DCI will not affect the MG configuration, and the UE handling of measurement gap occasions is specified in MAC spec, it is better to let the PHY layer indicate such skipping indication to MAC layer and the RRC layer should not be aware of such dynamic MG occasion skip. Hence, from spec modelling point of view, the lower layer should indicate the need of gap skipping to the MAC layer and the MAC layer shall consider the first MG occasion after the DCI reception with an offset to be deactivated. The following modification shows a potential way to implement the MAC spec.

## 5.14 Handling of measurement gaps During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5], for each measurement gap occasion which is not indicated to be skipped by the indication from the lower layer as specified in TS 38.212 [9]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

Proposal 1: PHY layer indicates the first MG after the DCI reception with an offset as skipped to MAC layer, and MAC layer considers the measurement gap occasion to be skipped.

Besides, there are also some RRC impacts to support the DCI-based solution. Since the skipping indication is carried via the existing scheduling DCI, the UE will need to interpret the DCI differently when the MG skipping feature is enabled or disabled. To align the interpretation, the gNB should configure the UE to enable or disable the feature. Thus, a new parameter can be introduced to the measurement gap (re-)configuration messages, e.g., to add a new IE in the MeasGapConfig IE. Also, a new UE capability shall be introduced to indicate whether the UE supports to skip MG based on the indication in DCI.

Proposal 2: RAN2 to introduce the following changes for the DCI-based MG skipping solution for the RRC spec:

- Introduce a new RRC parameter to configure MG skipping based on DCI;

- Introduce a new UE capability.

2.2 Semi-static solutions

Although the DCI-based solution can be used to avoid the collision between MG and dynamic data transmission, it is not efficient when CG or SPS is used. If the DCI-based solution is used to perform MG skipping for CG/SPS transmissions, the gNB would have to repeatedly send unnecessary scheduling DCIs, which is redundant to the CG/SPS transmission and decreases the CG/SPS benefit. Since CG and SPS are configured in a semi-static way, the collision with MG can be easily derived, so that the semi-static solutions is much more appropriate.

Besides, the semi-static solutions are also considered by RAN4 according to the LS in R4-2420198 [2].

[Excerpted from R4-2420198:] Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

According to the discussion in RAN4 as shown below, it is noted that semi-static solution can simplify the UE implementation complexity and such solution is preferred from the UE’s point of view. From RAN4 perspective, performing MG skipping leads to changes to the RRM measurement pattern. If the skipping is indicated dynamically, the UE has to determine the new measurement pattern frequently based on the indications, which causes more complexity to the UE processing. If semi-static solution is supported, the UE can determine the RRM measurement pattern according to the configuration and there is no need to change it dynamically.

[Excerpted from RAN4#113 report:] Option 1: Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions as it eases the UE implementation complexity associated with this feature. (QC, vivo, MTK, Apple, HW) Option 2: Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions. (E/// - compromosie, Nokia) Session Chair: all the UE vendors support option 1 in RAN4 RRM discussion.

With the above analysis, we can see that semi-static solution has the advantage not only to reduce the DCI overhead, but also allows to simplify the UE implementation. Hence there is a clear benefit to introduce such solution from both system efficiency and UE complexity point of view. We believe RAN2 should investigate semi-static solutions to achieve additional benefits in situation where this is possible, i.e. in case CG/SPS is used to carry XR traffic.

Observation 1: When CG/SPS is used to carry XR traffic, semi-static gap skipping approaches have a clear benefit over DCI-based solution in terms of system efficiency and UE complexity.

Proposal 3: RAN2 to confirm that semi-static solutions should be considered for MG skipping.

The following RRC-based semi-static solutions can be taken into account.

- Alternative 1: Configure a periodic pattern for MG skipping. When the MG overlaps with the configured pattern with a ratio larger than a threshold, the MG is skipped.

Considering the delay budget for data transmission, when the data transmission collides with the MG, it is still possible to complete the transmission in time as long as the MG does not consume too much time. For example, assuming there is an MG from time point T0 to T0+5ms, if the PDU set comes at time point T0 with PSDB equal to 10ms, it cannot be transmitted before the MG finishes unless the MG is skipped. However, even if the MG is not skipped, the transmission of the PDU set may also be completed within the PSDB since there are still 5ms left, during which DG or CG may be used to schedule the PDU set.

To reduce the impact on the RRM measurement, the MG can be kept when it is still possible to complete the transmission of the colliding data before or after the MG. To decide whether the MG should be skipped, a periodic pattern can be configured with the parameters such as periodicity, offset and duration via RRC, and this pattern can be related to XR traffic’s periodicity and delay budget. By comparing the ratio of the overlapping between MG duration and configured data duration, the UE can determine whether to skip the MG. Specifically, when the overlapping ratio is larger than a threshold, the MG is skipped to guarantee the data transmission. Otherwise, it is regarded that there is sufficient time for data transmission and the MG is performed.

A specific example is given in Fig.1, the pattern periodicity is 16.67ms, which is the same as the XR traffic periodicity and the duration of pattern (yellow box in Fig.3) is 10ms which corresponds to PSDB. Whether the MG overlapping with the configured pattern is skipped or not can be derived by the overlapping ratio. The overlap between the duration of pattern and the duration of MG is marked as blue box. The higher the overlapping ratio, the greater impact on XR traffic. Then, if the overlapping ratio is larger than a RRC configured threshold, the MG can be skipped. The gNB can configure a proper threshold based on historical XR frames delivery time.

Fig.1 Example of cancelling the measurement gaps based on overlapping ratio.

- Alternative 2: Configure the priority between the CG/SPS configuration and MG.

Different CG/SPS configurations may be used to transmit the data of different traffics. When the traffic requires stringent latency, the corresponding CG/SPS occasions are not expected to be disabled due to RRM measurements. While for the traffic without such requirements, it could be tolerable when the CG/SPS occasions colliding with MG are disabled. With the above considerations, it would be helpful to allow some of the CG/SPS configurations not to be impacted by RRM measurements

It should be noted that in the legacy positioning discussion in R17, priority between the PDSCH and PRS positioning window (PPW) was already supported. During the positioning procedure, the UE can determine whether to perform PRS reception within the PPW or PDSCH reception by the priority defined between these two DL signals. Similar to PPW, one feasible method is to set the priorities between the CG/SPS configuration and RRM measurement. For example, when configuring CG/SPS, it can be configured with a priority higher or lower than the RRM measurement. In this way, the CG/SPS transmission for low-latency XR traffic can be guaranteed without having frequent dynamic indications for each CG/SPS occasion.

Proposal 4: RAN2 consider the following semi-static solutions for MG skipping:

- Alternative 1: Configure a periodic pattern for MG skipping. When the MG overlaps with the configured pattern with a ratio larger than a threshold, the MG is skipped.

- Alternative 2: Configure the priority between the CG/SPS configuration and MG.

3. Conclusion

In this contribution, we analyzed the potential RAN2 impacts of the DCI-based solution for RRM measurement enhancements, and further propose to consider RRC-based solutions from RAN2 perspective. The following proposals were made:

Proposal 1: PHY layer indicates the first MG after the DCI reception with an offset as skipped to MAC layer, and MAC layer considers the measurement gap occasion to be skipped.

Proposal 2: RAN2 to introduce the following changes for the DCI-based MG skipping solution for the RRC spec:

- Introduce a new RRC parameter to configure MG skipping based on DCI;

- Introduce a new UE capability.

Proposal 3: RAN2 to confirm that semi-static solutions should be considered for MG skipping.

Proposal 4: RAN2 consider the following semi-static solutions for MG skipping:

- Alternative 1: Configure a periodic pattern for MG skipping. When the MG overlaps with the configured pattern with a ratio larger than a threshold, the MG is skipped.

- Alternative 2: Configure the priority between the CG/SPS configuration and MG.

4. Reference

[1]. 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[2]. R4-2420198, TSG RAN WG4, Response LS on gaps/restrictions that are caused by RRM measurements.