3GPP TSG-RAN WG2 Meeting #129	R2-2500815

# Athens, Greece, 17th – 21st Feburary, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | 0182 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PRS priority subset for DL-AoD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, VIVO, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PRS priority list is only applicable for UE-assisted positioning methods for DL-AoD for measurement reporting, while for the current spec, UE-based is listed as applicable. The previous RAN 1 agreement is attached below ***************UPDATE after RAN2 #128************* During RAN2#128, companies expressed the concern that the PRS priority list still can be useful for measurement for UE-based positioning although the field is not inlcluded in the PosCalculationAssistanceInfo. It is further confirmed that the field is not needed for measurement in UE-based case, while it is only used for measurement report in the UE-assisted case. This can be seen from the following part of spec from TS 38.214 |  |  |  |  |  |  |  |  |
| Summary of change: |  | PRS priority list is modified to clarify that it is not applicable to UE-based DL-AoD method. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current spec is not aligned with the intended usage on PRS priority list. Inter-operability analysis: Impacted functionality: DL-AoD enhancements Inter-op Analysis: If the UE is implemented according to the CR while the network is not, or If the network is implemented according to the CR while the UE is not, there is no inter-operability issue because in the current LPP spec, it is not possible to deliver the PRS priority subset assistance information in the part of config for UE-based and it is not supported by the RAN1 procedure to use it for UE-based. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2.1.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

================CHANGE BEGIN=====================

8.11.2.1	Information that may be transferred from the LMF to UE

The information that may be transferred from the LMF to the UE are listed in table 8.11.2.1-1.

Table 8.11.2.1-1: Assistance data that may be transferred from LMF to the UE

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | Yes | Yes |
| Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes | Yes |
| DL-PRS configuration of candidate NR TRPs | Yes | Yes |
| SSB information of the TRPs (the time/frequency occupancy of SSBs) | Yes | Yes |
| Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | Yes | Yes |
| Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | No | Yes |
| PRS-only TP indication | Yes | Yes |
| TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | No | Yes |
| LOS/NLOS indicators | No | Yes |
| On-Demand DL-PRS-Configurations | Yes | Yes |
| Expected Angle Assistance information | Yes | Yes |
| PRS priority list | Yes | No |
| Validity Area of the Assistance Data | Yes | Yes |

================CHANGE END=====================