3GPP TSG-RAN2#129 	R2-2500827

Athens, Greece, 17th Feb – 21th Feb, 2025

Agenda item:		8.4.4 (NR_LPWUS-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 made some agreements for LP-WUS in RRC_CONNECTED as follows.

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

In this contribution, we further discuss issues on LP-WUS in RRC_CONNECTED based on previous agreements.

2.	Discussion

In the last meeting, both RAN2 and RAN1 agreed not to support Option 1-1 and Option 1-2 simultaneously configured for the same UE. Thus, RAN2 does not need to discuss PDCCH monitoring for the case where both Option 1-1 and Option 1-2 work simultaneously.

RAN1 also agreed that UE monitors LP-WUS when UE is not in C-DRX active time for both Option 1-1 and Option 1-2.

[RAN1#119] Agreement For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask the feasibility of the above potential solution. Conclusion For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask whether RAN2 has same understanding or not.

Based on agreements, the UE behavior can be modelled as follows.

In Option 1-1,

- If the UE is not in Active time,

  - the UE monitors LP-WUS monitoring occasion prior to drx-onDurationTimer period.

  - If LP-WUS indicates to monitor PDCCH,

    - the UE starts drx-onDurationTimer and performs PDCCH monitoring and, i.e., Active time.

    - During Active time, the UE starts other DRX timers depending on Tx/Rx and Active Time can be extended.

  - Otherwise,

    - the UE does not start drx-onDurationTimer and not perform PDCCH monitoring

In Option 1-2,

- If the UE is not in Active time,

  - the UE monitors LP-WUS but the LP-WUS monitoring occasion is related to the new timer, not drx-onDurationTimer period.

  - If LP-WUS indicates to monitor PDCCH,

    - the UE starts a new Timer and performs PDCCH monitoring and, i.e., Active time.

    - During Active time, the UE starts other DRX timers depending on Tx/Rx and Active Time can be extended.

  - Otherwise, the UE does not start the new Timer and not perform PDCCH monitoring

We propose to consider above UE behaviour as a baseline for PDCCH monitoring in Option 1-1 and Option 1-2.

Proposal 1. For PDCCH monitoring in Option 1-1 and Option 1-2, RAN2 considers followings as a baseline.

In Option 1-1,

- If the UE is not in Active time,

  - the UE monitors LP-WUS monitoring occasion prior to drx-onDurationTimer period.

  - If LP-WUS indicates to monitor PDCCH,

    - the UE starts drx-onDurationTimer and performs PDCCH monitoring and, i.e., Active time.

    - During Active time, the UE starts other DRX timers depending on Tx/Rx and Active Time can be extended.

  - Otherwise,

    - the UE does not start drx-onDurationTimer and not perform PDCCH monitoring

In Option 1-2,

- If the UE is not in Active time,

  - the UE monitors LP-WUS but the LP-WUS monitoring occasion is related to the new timer, not drx-onDurationTimer period.

  - If LP-WUS indicates to monitor PDCCH,

    - the UE starts a new Timer and performs PDCCH monitoring and, i.e., Active time.

    - During Active time, the UE starts other DRX timers depending on Tx/Rx and Active Time can be extended.

  - Otherwise,

    - the UE does not start the new Timer and not perform PDCCH monitoring

We believe that the UE behaviour related to PDCCH monitoring is almost clear, but it is still unclear whether the UE monitors LP-WUS occasion only outside Active time or not, i.e., whether the UE monitors LP-WUS occasion in Active time or not.

In Rel-16 Power saving, RAN2 introduced DCP mechanism, which is very similar to LP-WUS, and DCP is monitored only outside Active time. This is because once the UE is in Active time, the UE is already awake and DCP is no more needed. During Active time, if the network determines the UE to stay awake, the network can extend Active time simply by scheduling UL/DL scheduling for the UE. Thus, DCP monitoring is not needed in Active time.

We think that it is simple to inherit the principle of DCP and there is no problem with LP-WUS. In Option 1-1, if the UE is in Active time, the UE does not monitor LP-WUS monitoring occasion prior to drx-onDurationTimer period. In Option 1-2, if the UE is in Active time, the UE does not monitor LP-WUS monitoring occasion related to the new timer period.

Proposal 2. The UE does not monitor LP-WUS monitoring occasion if the UE is in Active time.

According to contributions in the last meeting, some companies proposed LP-WUS per DRX group. However, we think LP-WUS per DRX group may have RAN1 impact.

Currently, DCP per group is not supported. In Rel-16 TEI, RAN2 asked RAN1 about the impact of the secondary DRX group in [1], and RAN1 replied that RAN1 impact of secondary DRX related to the UE’s behavior of detecting DCP and the respective procedures in [2]. Thus, RAN2 agreed that both the secondary DRX group and DCP are not configured simultaneously.

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

Thus, it would be better to ask RAN1 whether LP-WUS and the secondary DRX group can be configured simultaneously, different from DCP. If RAN1 replies yes, RAN2 continues further discussion, e.g. whether the new timer can have different value for FR1 and FR2.

Proposal 3. RAN2 asks RAN1 whether LP-WUS and the secondary DRX group can be configured simultaneously, different from DCP.

3.	Conclusion

In this document, we discuss issues on LP-WUS in RRC_CONNECTED, and make proposals as follows.

Proposal 1. For PDCCH monitoring in Option 1-1 and Option 1-2, RAN2 considers followings as a baseline.

In Option 1-1,

- If the UE is not in Active time,

  - the UE monitors LP-WUS monitoring occasion prior to drx-onDurationTimer period.

  - If LP-WUS indicates to monitor PDCCH,

    - the UE starts drx-onDurationTimer and performs PDCCH monitoring and, i.e., Active time.

    - During Active time, the UE starts other DRX timers depending on Tx/Rx and Active Time can be extended.

  - Otherwise,

    - the UE does not start drx-onDurationTimer and not perform PDCCH monitoring

In Option 1-2,

- If the UE is not in Active time,

  - the UE monitors LP-WUS but the LP-WUS monitoring occasion is related to the new timer, not drx-onDurationTimer period.

  - If LP-WUS indicates to monitor PDCCH,

    - the UE starts a new Timer and performs PDCCH monitoring and, i.e., Active time.

    - During Active time, the UE starts other DRX timers depending on Tx/Rx and Active Time can be extended.

  - Otherwise,

    - the UE does not start the new Timer and not perform PDCCH monitoring

Proposal 2. The UE does not monitor LP-WUS monitoring occasion if the UE is in Active time.

Proposal 3. RAN2 asks RAN1 whether LP-WUS and the secondary DRX group can be configured simultaneously, different from DCP.

4.	Reference

- R2-1916597	LS on secondary DRX group	Ericsson 	LS Out	To: RAN1, RAN4

- R2-2004325	LS response on secondary DRX group (R1-2002961; contact: Ericsson)	RAN1	LS in