3GPP TSG RAN WG2 #129	R2-2500848

Athens, Greece, February 17th – 21st, 2025

Agenda item:		8.9.3

Source:	Nordic Semiconductor ASA

Title:					IoT-NTN uplink capacity enhancement

Document for:		Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#128, RAN2 has reached the following agreements on CB-Msg3 EDT.

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In this contribution, we continue discussion on details of the CB-Msg3 configuration, DSA mechanism, and details of Msg4.

# Configuration for CB-Msg3 transmission

A preamble-less EDT is beneficial for reducing uplink and downlink signaling in NTN. Preamble transmission and RAR reception unnecessarily increases delay and, even more importantly, consumes significant uplink and downlink resources. Work on contention based Msg3 (CB-Msg3) in RAN2 has progressed and it has already been agreed on configuring a pool of PUSCH resources for a plurality of CE levels via SIB signaling. UEs are choosing randomly a resource for their Msg3 transmission using a CE level which is selected based on RSRP measurement. There will be different RSRP threshold values for each specified/configured CE level. In addition, RAN2 has agreed to introduce DSA (Diversity Shotted Aloha) mechanism for CB-Msg3-EDT transmission where more than one replica of the Msg3 packet can be transmitted.

Given the PUSCH resource with a fixed length in time for each CE level, it is up to RAN1 to discuss on details of mapping CB-Msg3 on the given physical resource. For example, whether the number of resource units and TBS size are fixed per CE level or UE can select TBS and the number of resource units from a configured set of values similarly what was done in Rel-15 EDT. Also, RAN1 should discuss aspects related to potential scheduled transmission of CB-Msg3, e.g., whether new DCI format is needed.

Proposal 1: Send LS to RAN1 informing on the potential RAN1 work pertaining to detailed physical resource mapping of PUSCH payload data to the configured PUSCH resource and potential other RAN1 topics.

There should be an upper limit on the total data volume that UE is allowed to transmit via preamble-less EDT procedure. However, the UE should be allowed to transmit the given total data volume irrespective of the specific coverage situation it finds itself in. Essentially this means that if the UE is not able to transmit its payload in a single shot transmission due to bad coverage situation the UE is allowed to request a scheduled transmission for the rest of the payload bits. This helps to improve the network capacity as the preconfigured PUSCH resources do not need to be dimensioned according to worst case coverage levels but, instead, the UEs in worst coverage situation are allowed to segment their payload data and transmit part of it in scheduled transmission occasions, similarly as is done in Rel-17 SDT in NR.

Proposal 2: The total data volume per preamble-less EDT session is upper limited, but UE is allowed to transmit part of it via scheduled transmission if UE is not able to transmit it in a single-shot manner.

# DSA Mechanism and Msg4 enhancements

UE is allowed to transmit multiple replicas of the payload using DSA mechanism. The exact way how the replicas are to be transmitted is yet to be determined. We propose that different replicas are transmitted in different periodical PUSCH resources by selecting randomly time and frequency resources from a configured pool of resources. Maximum time difference of subsequent replicas could be fixed in specification. Each replica is associated with its own timer for Msg4 monitoring.

Proposal 3: Different replicas of DSA transmission are transmitted in different periodical PUSCH resources, and each replica is associated with its own timer for Msg4 monitoring.

Since Msg3 is contention based, the contention should be resolved in Msg4. As in normal RACH procedure, a contention resolution MAC CE can be included in Msg4. To improve DL capacity of NB-IoT over NTN, several contention resolution MAC CEs (of different UEs) could be included in the same Msg4 transmission or, alternatively, multi-TB scheduling of Rel-16 could be used for scheduling of multiple MAC CEs.

Proposal 4: Several contention resolution MAC CEs (of different UEs) could be included in the same Msg4 transmission or, alternatively, multi-TB scheduling of Rel-16 could be used for scheduling of multiple MAC CEs.

# Conclusions

In this contribution we discussed issues related to uplink capacity enhancement for NB-IoT-based NTN system, and we have the following proposals:

Proposal 1: Send LS to RAN1 informing on the potential RAN1 work pertaining to detailed physical resource mapping of PUSCH payload data to the configured PUSCH resource and potential other RAN1 topics.

Proposal 2: The total data volume per preamble-less EDT session is upper limited, but UE is allowed to transmit part of it via scheduled transmission if UE is not able to transmit it in a single-shot manner.

Proposal 3: Different replicas of DSA transmission are transmitted in different periodical PUSCH resources, and each replica is associated with its own timer for Msg4 monitoring.

Proposal 4: Several contention resolution MAC CEs (of different UEs) could be included in the same Msg4 transmission or, alternatively, multi-TB scheduling of Rel-16 could be used for scheduling of multiple MAC CEs.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)