3GPP TSG-RAN WG2 #129	R2-2500850

Athens, Greece, 17 – 21 February 2025

Agenda Item:	7.0.2.15 Enhancement on NR QoE management and optimizations for diverse services

Source:	Ericsson

Title:	Discussion on RRC issues for QoE

Document for:	Discussion, Decision

# 1	Introduction

This document includes discussion and a proposed resolution on the incoming LS from RAN3 related to inter-gNB coordination of RRC segmentation for QoE reporting on SRB4 and/or SRB5 in NR-DC scenarios [1]. It also includes another proposed correction for rel-18 QoE.

# 2	Discussion

## 2.1	Discussion on incoming LS

RAN3 has discussed inter-gNB coordination of RRC segmentation for QoE reporting on SRB4 and/or SRB5 in NR-DC scenarios and sent and LS to RAN2 with the following questions:

- Question 1: Can the UE receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter?

- Question 2: Can the UE receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter?

- Question 3: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled?

- Question 4: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

### 2.1.1	RRC parameters

To be able to reply to the questions raised by RAN3, an analysis of the RRC parameters is needed. In RRC, the MCG and the SCG configurations are separate and the MCG configuration for QoE is prepared by the MN and the SCG configuration for QoE is prepared by the SN. The signalling is structured so that the MN does not have to read the content of the SCG configuration. This is important for the cases when the MN may not be able to read the SCG configuration, e.g. if the MN and the SN are of different RATs or if they support different releases of the RRC specification. The MN cannot make changes to the SCG configuration as that would create a mismatch in the configuration between the UE and the SCG.

An MN cannot change an SCG configuration (or vice versa) as that would create a configuration mismatch between the UE and the SCG.

This can e.g. be seen by the way the message RRCReconfiguration is structured, where for QoE the QoE configuration for the MCG is included in the MN part of the message, whereas the QoE configuration for the SCG is included in the SN part of the message.

The MCG QoE configuration is included in the outer RRCReconfiguration message:

RRCReconfiguration-v1700-IEs ::=        SEQUENCE {

otherConfig-v1700                       OtherConfig-v1700                                              OPTIONAL, -- Need M

sl-L2RelayUE-Config-r17                 SetupRelease { SL-L2RelayUE-Config-r17 }                       OPTIONAL, -- Need M

sl-L2RemoteUE-Config-r17                SetupRelease { SL-L2RemoteUE-Config-r17 }                      OPTIONAL, -- Need M

dedicatedPagingDelivery-r17             OCTET STRING (CONTAINING Paging)                               OPTIONAL, -- Cond PagingRelay

needForGapNCSG-ConfigNR-r17             SetupRelease {NeedForGapNCSG-ConfigNR-r17}                     OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17          SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                  OPTIONAL, -- Need M

musim-GapConfig-r17                     SetupRelease {MUSIM-GapConfig-r17}                             OPTIONAL, -- Need M

ul-GapFR2-Config-r17                    SetupRelease { UL-GapFR2-Config-r17 }                          OPTIONAL, -- Need M

scg-State-r17                           ENUMERATED { deactivated }                                     OPTIONAL, -- Need S

appLayerMeasConfig-r17                  AppLayerMeasConfig-r17                                         OPTIONAL, -- Need M

ue-TxTEG-RequestUL-TDOA-Config-r17      SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17}              OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1800-IEs                                   OPTIONAL

}

The SCG QoE configuration is included in the embedded RRCReconfiguration message for the SCG:

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                      ENUMERATED {true}                                                     OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup                 CHOICE {

nr-SCG                                  OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                               OCTET STRING

}

}

The embedded RRCReconfiguration message is a corresponding RRCReconfiguration prepared by the SN, where appLayerMeasConfig is included in the SN configuration of QoE.

The appLayerMeasConfig contains the following:

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowedSRB4-r17               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

...,

[[

rrc-SegAllowedSRB5-r18               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

idleInactiveReportAllowed-r18        ENUMERATED {enabled}                                                       OPTIONAL  -- Need R

]]

}

It is the parameters rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 that have caused a discussion in RAN3. The underlying question is: If one node configures one of the parameters, e.g. rrc-SegAllowedSRB4, does the other node also have to include the parameter rrc-SegAllowedSRB4 to indicate that segmentation is allowed for SRB4 as the field is Need R, or does the UE understand that segmentation for SRB4 is still allowed even if a configuration of QoE for the SCG is sent without the parameter rrc-SegAllowedSRB4 included?

The RAN2 understanding has been that the MCG and the SCG configurations are completely separate, and that one configuration cannot impact the other, unless the network explicitly instructs the UE to update the configuration. In order to have it clear, there is a clarification in the field descriptions of the two parameters, to clarify that rrc-SegAllowedSRB4 can only be configured by the MN and that rrc-SegAllowedSRB5 can only be configured by the SN.

rrc-SegAllowedSRB4 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB4. The field is only configured for an MCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

rrc-SegAllowedSRB5 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB5. The field is only configured for an SCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

However, the fact that RAN3 is discussing the issue, may imply that it is not captured clearly enough. RAN2 needs to decide whether the existing field description is sufficient or whether any update is needed.

Normally, the same fields are used in both the MCG and the SCG RRC messages and the UE only determines whether the field is configured for the MCG or the SCG based on where in the message the field is configured, if it is in the MN part of the message or in the embedded SN part of the message. It is not common practice to have different parameters for the MN and the SN respectively. Therefore, it may have been clearer to have only one parameter rrc-SegAllowed, which would be used for SRB4 if it was part of an MCG configuration and for SRB5 if it was part of an SCG configuration. However, it seems a bit late to merge the parameters at this stage.

Another option could be to clarify the field description even further and change the Need code to S. The field descriptions may look like the following:

rrc-SegAllowedSRB4 (change need code R -> S)

This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB4. The field is applicable only when the application layer measurement configuration is for an MCG. If the field is absent when the application layer measurement configuration is for an MCG, RRC segmentation of MeasurementReportAppLayer is disabled on SRB4. The UE takes no action if the application layer measurement configuration is for an SCG.

It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

rrc-SegAllowedSRB5

This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB5. The field is applicable only when the application layer measurement configuration is for an SCG. If the field is absent when the application layer measurement configuration is for an SCG, RRC segmentation of MeasurementReportAppLayer is disabled on SRB5. The UE takes no action if the application layer measurement configuration is for an MCG.

It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

The proposed changes would make the handling clearer and prevent erroneous UE implementation.

Change the need code to Need S and update the field descriptions of the fields rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5.

### 2.1.2	RAN3 questions

The questions that RAN3 raised need to be replied. These are the proposed replies based on the analysis above.

- Question 1: Can the UE receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter?

No, rrc-SegAllowedSRB5-r18 cannot be included in AppLayerMeasConfig for an MCG configuration.

- Question 2: Can the UE receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter?

No, rrc-SegAllowedSRB4-r17 cannot be included in AppLayerMeasConfig for an SCG configuration.

- Question 3: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled?

The reception of an MN AppLayerMeasConfig-r17 does not impact the SCG configuration, i.e. the UE keeps its previously configured state related to RRC segmentation for SRB5.

- Question 4: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

The reception of an SN AppLayerMeasConfig-17 does not impact the MCG configuration, i.e. the UE keeps its previously configured state related to RRC segmentation for SRB4.

Furthermore, RAN2 will update the need code to Need S and the field descriptions of the fields rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 to further clarify the intended UE behaviour.

Send a reply LS to RAN3 with the above proposed text.

## 2.2	RRC issue on QoE reports

Another RRC issue is related to the following Note in 5.7.16.2:

NOTE 0:	If the application layer measurement configurations and reports exceed the size of an RRC message, the UE includes the appLayerIdleInactiveConfig and corresponding appLayerSessionStatus in the first MeasurementReportAppLayer message after the UE received the RRCSetup message, and transmits application layer measurement report containers in subsequent MeasurementReportAppLayer messages.

In the Note, it is not clear that the UE can include QoE reports also in the first MeasurementReportAppLayer message that it transmits after entering RRC_CONNECTED. As that is likely to be the most common case, it is proposed to add such clarification in the Note. The session status indications are also missing in the first part of the sentence and could be added as well. The proposed change is highlighted:

NOTE 0: If the size of the application layer measurement configurations, session status indications and reports exceeds the size of an RRC message, the UE includes the appLayerIdleInactiveConfig and corresponding appLayerSessionStatus in the first MeasurementReportAppLayer message after the UE received the RRCSetup message, and transmits application layer measurement report containers in the same and subsequent MeasurementReportAppLayer messages.

Update Note 0 in RRC chapter 5.7.16.2 according to the proposed TP.

# Conclusion

RAN2 is kindly asked to discuss the following proposal:

Proposal 1	Change the need code to Need S and update the field descriptions of the fields rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5.

Proposal 2	Send a reply LS to RAN3 with the above proposed text.

Proposal 3	Update Note 0 in RRC chapter 5.7.16.2 according to the proposed TP.

# References

R3-247888, LS on QMC Coordination for RRC Segmentation in NR-DC, ZTE