3GPP TSG-RAN WG2 #129	R2-2500851

Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.3.2.2 Model Generalization

Source:	Ericsson

Title:	Generalization of the AI/ML models for RRM prediction

Document for:	Discussion

# Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. In this contribution, generalization aspects of the RRM measurement prediction models are discussed and the relevant simulation results are included.

The generalization of an AI/ML model refers to how well such model can adapt what it has learned from the training data to make accurate predictions using new data.

In RAN2#128 meeting, the following agreements have been taken about model generalization aspects [2]:

Agreements on generalization - Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. 2 Companies can choose which case they compare with and should report it with simulation results. 3 Generalization issues on RRM measurement prediction are prioritized. 4 Start the study with generalization issue with RRM measurement prediction in temporal domain. Companies can chose to study frequency domain prediction cases and report what they have simulated. 5 Study generalization over UE speeds 6 The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h. FFS on combinations 7 The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h. FFS on combinations

Besides the generalization aspects related to UE speed for in temporal domain prediction, as agreed in RAN2#128, the generalization over the frequency domain predictions and network deployment are also discussed in this contribution.

# Discussion

## Generalization over UE speeds (temporal predictions)

Additional agreements were taken in the RAN2#128 meeting regarding possible combinations of UE speeds when training the AI/ML model and during the inference:

Agreements on generalization 1 Between GC#1 and GC#2, RAN2 focus on GC#2 2 Simulation combination for FR1 generalization study: Training @Dataset: 30km/h; Training @Dataset: 60km/h; Training @Dataset: 90km/h; Inference @30km/h; Inference @60km/h; Inference @90km/h. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. 2a Simulation combination for FR2 generalization study: Training @Dataset: 60km/h; Training @Dataset: 90km/h; Training @Dataset: 120km/h; Inference @60km/h; Inference @90km/h; Inference @120km/h. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes.

There seems to be a mismatch on GC#1 definition between the evaluation methodology taken from TR38.843 and the simulation combinations agreed in RAN2#128:

| Evaluation methodology in TR38.843 | RAN2#128 agreement |
| --- | --- |
| Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. GC#1: The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. ... That is, comparing GC#1 to the baseline, model in GC#1 is trained on a different dataset, and tested on the same dataset. | ... That is, comparing GC#1 to the baseline, model in GC#1 is trained on the same dataset, and tested on a different dataset |  |

Since model generalization is investigated in this study item, we would like to stick to the GC#1 proposed in simulation combination table agreed in RAN2#128 to make sure it’s the same model trained on a same dataset but inference on different dataset for baseline and GC#1 respectively.

GC#1 as defined in simulation combinations table agreed in RAN2#128 makes more sense from perspective of model generalization study.

### Temporal prediction case B, RRM sub case 2.

Goal 1: Measurement overhead reduction

RRM sub case 2: Cell-level->Cell-level

Generalize over UE moving speed 30km/h, 60km/h 90km/h with MRRT 50% and MRRT 80% respectively.

Figure 1. Prediction patterns with MRRT 50% and 80% used in simulation for generalization over UE moving speed in FR1.

Table 1. 50% MRRT, averaged RSRP difference for simulation combination in FR1 generalization study over speed

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | 0.143 |  |  |
| GC#1 | Yes |  |  |  | 0.195 | 0.228 |
| GC#2 | Yes | Yes | Yes | 0.103 |  |  |
| Baseline |  | Yes |  |  | 0.148 |  |
| GC#1 |  | Yes |  | 0.11 |  | 0.173 |
| GC#2 | Yes | Yes | Yes |  | 0.131 |  |
| Baseline |  |  | Yes |  |  | 0.155 |
| GC#1 |  |  | Yes | 0.106 | 0.136 |  |
| GC#2 | Yes | Yes | Yes |  |  | 0.15 |

Table 2. 80% MRRT, averaged RSRP difference for simulation combination in FR1 generalization study over speed

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | 0.227 |  |  |
| GC#1 | Yes |  |  |  | 0.325 | 0.396 |
| GC#2 | Yes | Yes | Yes | 0.217 |  |  |
| Baseline |  | Yes |  |  | 0.316 |  |
| GC#1 |  | Yes |  | 0.22 |  | 0.385 |
| GC#2 | Yes | Yes | Yes |  | 0.312 |  |
| Baseline |  |  | Yes |  |  | 0.38 |
| GC#1 |  |  | Yes | 0.224 | 0.313 |  |
| GC#2 | Yes | Yes | Yes |  |  | 0.381 |

The results in the tables above show that the model generalizes well over UE speed. The performance is a bit better for a model trained with high UE speed and that generalizes over low UE speed, but also the other direction generalizes well. The generalization also shows slightly better results when the MRRT is lower, but the results are still good with 80% MRRT.

For study goal 1, a model trained with high UE moving speed data generalizes well on data with low UE speed.

The model also generalizes well for a model trained with low UE speed generalized with high UE speed

For study goal 1, a model trained with mixed dataset generalizes well on each separate dataset.

### Temporal prediction case A, RRM sub case 2.

Goal 2: HO performance improvement

RRM sub case 2: Cell-level->Cell-level

Generalize over UE moving speed 60km/h, 90km/h, 120km/h in ratio 1 case.

Figure 2. Prediction patterns with ratio 1 used in simulation for generalization over UE moving speed in FR2.

Table 3. Ratio 1, averaged RSRP difference for simulation combination in FR2 generalization study over speed

|  | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Training @Dataset: 120km/h | Inference @60km/h | Inference @90km/h | Inference @120km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | 0.87 |  |  |
| GC#1 | Yes |  |  |  | 0.993 | 1.078 |
| GC#2 | Yes | Yes | Yes | 0.806 |  |  |
| Baseline |  | Yes |  |  | 0.990 |  |
| GC#1 |  | Yes |  | 0.885 |  | 1.065 |
| GC#2 | Yes | Yes | Yes |  | 0.930 |  |
| Baseline |  |  | Yes |  |  | 1.072 |
| GC#1 |  |  | Yes | 0.901 | 1.003 |  |
| GC#2 | Yes | Yes | Yes |  |  | 1.018 |

For study goal 2, a model trained with high UE moving speed data generalizes well on data with low UE speed.

The model also generalizes well for a model trained with low UE speed generalized with high UE speed

For study goal 2, model trained with mixed dataset generalizes well on each separate dataset.

In summary, both case A and case B generalize well over UE speed and a model trained with mixed dataset as input has better performance.

Capture in the TR that a model generalizes well over UE speed and training with mixed dataset gives better performance.

## Generalization over frequency prediction

In this subsection, we investigate generalization cases for frequency prediction.

### Generalization over flipped frequency

In this subsection, we investigate frequency prediction cases when the model is trained on 2GHz-to-4 GHz while infer 4 GHz -to-2 GHz for both GC1 and GC2, and vice-versa. Here are some definitions for the performance results in Table 4:

- Baseline: train on 2 GHz to predict 4 GHz  inference on 2 GHz to predict 4 GHz

- GC#1: train on 2 GHz to predict 4 GHz  Generalize over 4 GHz to predict 2 GHz

- GC#2: train on 2 GHz & 4 GHz to predict 4 GHz & 2 GHz  Generalize over 4 GHz to predict 2 GHz

  - GC#2a: No Knowledge about input & output frequency during training and inference.

  - GC#2b: Knowledge about input & output frequency during training and inference.

Table 4. Performance evaluation of frequency flipping (with training of 2 GHz to 4 GHz) for baselines, GC#1 and GC#2

|  | Training: 2 GHz>4 GHz | Training: 4 GHz>2 GHz | Inference 2 GHz>4 GHz, RSRP Difference (dB) | Inference 4 GHz>2 GHz, RSRP Difference (dB) |
| --- | --- | --- | --- | --- |
| #Cells@Input=8 & #Cells@Output=8, UMa Configurations |  |  |  |  |
| Baseline | Yes |  | 0.12 |  |
| GC#1 | Yes |  |  | 19 |
| GC#2a | Yes | Yes |  | 4.15 |
| GC#2b | Yes | Yes |  | 2.23 |
| #Cells@Input=8 & #Cells@Output=8, UMi Configurations |  |  |  |  |
| Baseline | Yes |  | 0.17 |  |
| GC#1 | Yes |  |  | 15.6 |
| GC#2a | Yes | Yes |  | 7.44 |
| GC#2b | Yes | Yes |  | 1.98 |

In Table 5, we performed the same evaluation of Table 4 but flipping the training frequencies, such input=4 GHz to output=2 GHz, note that the definition of Baseline, GC#1, and GC#2 still hold, but in the opposite direction of frequencies.

Table 5. Performance evaluation of frequency flipping (with training of 4 GHz to 2 GHz) for baselines, GC#1 and GC#2

|  | Training: 2 GHz>4 GHz | Training: 4 GHz>2G | Inference 2 GHz>4 GHz, RSRP Difference (dB) | Inference 4 GHz>2 GHz, RSRP Difference (dB) |
| --- | --- | --- | --- | --- |
| #Cells@Input=8 & #Cells@Output=8, UMa Configurations |  |  |  |  |
| Baseline |  | Yes |  | 0.12 |
| GC#1 |  | Yes | 33 |  |
| GC#2a | Yes | Yes | 5.22 |  |
| GC#2b | Yes | Yes | 3 |  |
| #Cells@Input=8 & #Cells@Output=8, UMi Configurations |  |  |  |  |
| Baseline |  | Yes |  | 0.14 |
| GC#1 |  | Yes | 17.2 |  |
| GC#2a | Yes | Yes | 11 |  |
| GC#2b | Yes | Yes | 2 |  |

From the above Table 4 and Table 5, we observe the following:

Generalization using GC#2 always outperform that of GC#1

Training and generalization using the knowledge about the input & output frequency (or even an indication) outperforms the case where the model cannot recognize which frequency at the input or output.

Even though GC#2 outperforms GC#1, it still performs poorly (2 to 3dB which is still > 1dB).

It is not easy for an AI/ML model to generalize over flipped frequencies, because the relation between two different frequencies is not the same (can be even opposite relations in some cases). Though, there are many techniques to overcomes this, but those techniques need some effort to be devised and verified. Some techniques require information exchange, some do not. Hence, we need to spend even more effort on handling this case. The questions are, do we have a clear motivation for the flipped frequency use-case? Isn’t having a third and (maybe) fourth frequencies and test generalization over them is more logical use-case?

Generalization over flipped frequencies is difficult and may require additional effort.

There is no clear and useful use-case for flipped frequency generalization.

Based on the above argument, we suggest investigating another generalization scenario for frequency prediction. One case is to have a third (or even a fourth) frequency, and training on, e.g., two frequencies and evaluate the generalization over the third or fourth or both frequencies. This case enables efficient and faster across multi-carrier mobility process especially when the UE and the network are capable of supporting three and above frequency carriers, which is a realistic assumption.

Investigate generalization over a third (and maybe fourth) frequency carrier, as a generalization use-case of frequency prediction instead of flipped frequency.

## Generalization over deployment (UMa - UMi)

It was proposed in the last e-mail discussion ([Post128][018][AI Mob] generalization) that two cell configurations can be considered for evaluating the generalization over cell configurations.

| Parameter | Configuration #A | Configuration #B |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 2: generalization parameters for FR1

| Parameter | Configuration #A | Configuration #B |
| --- | --- | --- |
| Deployment scenario | UMa | UMi |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 3: generalization parameters for FR2

The parameters for ISD, antenna height and Tx Power do not seem to be aligned with the deployment scenario and the last three rows need to be swapped.

Swap the last three rows for ISD, BS antenna height and BS Tx power between Configuration #A and Configuration #B in Table 3 in Proposal 3 in e-mail discussion Post128][018][AI Mob] generalization.

In the results provided below, we used the configurations from a table with swapped rows.

### Temporal prediction case B, RRM sub case 2

The following case was evaluated:

Goal 1: Measurement overhead reduction

RRM sub case 2: Cell-level->Cell-level

Generalize from FR1-Uma -> FR1-UMi in combinations of

- MRRT 50%

- MRRT 80%,

- low UE moving speed, e.g., 30km/h

- high UE moving speed, e.g., 90km/h

Figure 3. Prediction patterns with MRRT 50% and 80% used in simulation for generalization over Cell configuration in FR1

Table 6. MRRT 50%, averaged RSRP difference for simulation combination in FR1 generalization study over cell configuration

| UE moving speed |  | Training @Dataset: UMa | Training @Dataset: UMi | Inference @Uma | Inference @Umi |
| --- | --- | --- | --- | --- | --- |
| 30km/h | Baseline | Yes |  | 0.143 |  |
|  | GC#1 | Yes |  |  | 0.254 |
|  | GC#2 | Yes | Yes | 0.101 |  |
| 90km/h | Baseline | Yes |  | 0.148 |  |
|  | GC#1 | Yes |  |  | 0.312 |
|  | GC#2 | Yes | Yes | 0.154 |  |

Table 7. MRRT 80%, averaged RSRP difference for simulation combination in FR1 generalization study over cell configuration

| UE moving speed |  | Training @Dataset: UMa | Training @Dataset: UMi | Inference @Uma | Inference @Umi |
| --- | --- | --- | --- | --- | --- |
| 30km/h | Baseline | Yes |  | 0.227 |  |
|  | GC#1 | Yes |  |  | 0.418 |
|  | GC#2 | Yes | Yes | 0.227 |  |
| 90km/h | Baseline | Yes |  | 0.38 |  |
|  | GC#1 | Yes |  |  | 0.644 |
|  | GC#2 | Yes | Yes | 0.397 |  |

For study goal 1, model trained with FR1-UMa data generalize well in FR1-UMi scenario with RSRP difference lower than 1db.

For study goal 1, comparable performance is obtained for a model trained with dataset mixed from FR1-UMa and FR1-UMi and the baseline where model is trained with dataset of FR1_UMa when inference on dataset of FR1-UMa.

### Temporal prediction case A, RRM sub case 2

The following case was evaluated:

Goal 2: HO performance improvement

RRM sub case 2: Cell-level->Cell-level

Generalize from FR2-UMi -> FR2-UMa in combinations of

- Ratio 1

- low UE moving speed, e.g., 30km/h

- high UE moving speed, e.g., 90km/h

Figure 4. Prediction patterns with ratio 1 used in simulation for generalization over Cell configuration in FR2

Table 8. Ratio 1, averaged RSRP difference for simulation combination in FR2 generalization study over cell configuration

| UE moving speed |  | Training @Dataset: UMi | Training @Dataset: Uma | Inference @Umi | Inference @Uma |
| --- | --- | --- | --- | --- | --- |
| 60km/h | Baseline | Yes |  | 0.87 |  |
|  | GC#1 | Yes |  |  | 1.593 |
|  | GC#2 | Yes | Yes | 0.845 |  |
| 120km/h | Baseline | Yes |  | 1.072 |  |
|  | GC#1 | Yes |  |  | 2.022 |
|  | GC#2 | Yes | Yes | 1.118 |  |

For study goal 2, model trained with FR2-UMi data generalizes in FR2-UMa scenario with a cost of additional RSRP difference more than 0.5 db.

Comparable performance is obtained model trained with dataset mixed from FR2-UMi and FR2-UMa and baseline where model is trained with dataset of FR2_Umi when inference on dataset of FR2_UMi.

### Frequency prediction generalization over UMa/UMi configurations

In this subsection, we investigate frequency prediction (2 GHz>4 GHz) generalization when training on UMi and infer on UMa, versus training on UMa and infer on UMi. Here are some definitions for the results in Table 5:

- Baseline: train on 2 GHz to predict 4 GHz @UMa and/or UMi  2 GHz to predict 4 GHz@UMa or Umi.

- GC#1: train on 2 GHz to predict 4 GHz @UMi  Generalize over 2 GHz to predict 4 GHz@UMa

- GC#2: train on 2 GHz to predict 4 GHz @UMi & UMa Generalize over 2 GHz to predict 4 GHz@UMi

Table 5. Frequency prediction evaluation of cell’s configuration generalization (over UMi and UMa) for baseline, GC#1 and GC#2

|  | Training@UMi: 2 GHz>4 GHz | Training@UMa: 2 GHz>4 GHz | Inference@UMi 2 GHz>4 GHz | Inference@UMa 2 GHz>4 GHz |
| --- | --- | --- | --- | --- |
|  | UMa to UMi frequency prediction generalization |  |  |  |
| Baseline |  | YES |  | 0.16 |
| GC#1 |  | YES | 18.8 |  |
| GC#2 | YES | YES | 2.5 |  |
|  | UMi to UMa frequency prediction generalization |  |  |  |
| Baseline | YES |  | 0.2 |  |
| GC#1 | YES |  |  | 19 |
| GC#2 | YES | YES |  | 3.9 |

When generalizing for frequency prediction from UMa to UMi, GC#2 achieves reasonable performance, however, its performance is highly unsatisfactory for GC#1

Focus on GC#2 when generalizing over UMi and UMa in frequency predictions.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	GC#1 as defined in simulation combinations table agreed in RAN2#128 makes more sense from perspective of model generalization study.

Observation 2	For study goal 1, a model trained with high UE moving speed data generalizes well on data with low UE speed.

Observation 3	The model also generalizes well for a model trained with low UE speed generalized with high UE speed

Observation 4	For study goal 1, a model trained with mixed dataset generalizes well on each separate dataset.

Observation 5	For study goal 2, a model trained with high UE moving speed data generalizes well on data with low UE speed.

Observation 6	The model also generalizes well for a model trained with low UE speed generalized with high UE speed

Observation 7	For study goal 2, model trained with mixed dataset generalizes well on each separate dataset.

Observation 8	Generalization using GC#2 always outperform that of GC#1

Observation 9	Training and generalization using the knowledge about the input & output frequency (or even an indication) outperforms the case where the model cannot recognize which frequency at the input or output.

Observation 10	Even though GC#2 outperforms GC#1, it still performs poorly (2 to 3dB which is still > 1dB).

Observation 11	Generalization over flipped frequencies is difficult and may require additional effort.

Observation 12	There is no clear and useful use-case for flipped frequency generalization.

Observation 13	For study goal 1, model trained with FR1-UMa data generalize well in FR1-UMi scenario with RSRP difference lower than 1db.

Observation 14	For study goal 1, comparable performance is obtained for a model trained with dataset mixed from FR1-UMa and FR1-UMi and the baseline where model is trained with dataset of FR1_UMa when inference on dataset of FR1-UMa.

Observation 15	For study goal 2, model trained with FR2-UMi data generalizes in FR2-UMa scenario with a cost of additional RSRP difference more than 0.5 db.

Observation 16	Comparable performance is obtained model trained with dataset mixed from FR2-UMi and FR2-UMa and baseline where model is trained with dataset of FR2_Umi when inference on dataset of FR2_UMi.

Observation 17	When generalizing for frequency prediction from UMa to UMi, GC#2 achieves reasonable performance, however, its performance is highly unsatisfactory for GC#1

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture in the TR that a model generalizes well over UE speed and training with mixed dataset gives better performance.

Proposal 2	Investigate generalization over a third (and maybe fourth) frequency carrier, as a generalization use-case of frequency prediction instead of flipped frequency.

Proposal 3	Swap the last three rows for ISD, BS antenna height and BS Tx power between Configuration #A and Configuration #B in Table 3 in Proposal 3 in e-mail discussion Post128][018][AI Mob] generalization.

Proposal 4	Focus on GC#2 when generalizing over UMi and UMa in frequency predictions.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RAN2 Chairman’s Notes, RAN2#128