3GPP TSG-RAN WG2 Meeting #129			           			R2-2500856

Athens, Greece, Feb. 17th – 21st,  2025

Agenda Item:	8.6.4

Source: 	Lenovo

Title: 	CLTM Scenarios and remaining points

Document for:	Discussion and decision

# Introduction

In RAN plenary in Melbourne instructed RAN2 to work on the Conditional LTM topic. The following agreements were arrived at in RAN2#128:

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

This document addresses the remaining points for the C-LTM work.

# Discussion

The Intra-CU LTM feature has been specified in Rel. 18 and some further enhancements (Inter-CU) is in progress in Rel. 19. Here UE receives an RRC configuration message allowing the UE to receive RRC Configuration of one or more candidate cells. In the due course of the procedure, UE initiates an Early TA for one or more candidate cell based on receiving PDCCH order. An Early TA may be determined and sent to the UE in a new MAC CE, if the UE has already measured and reported at least one target cell. The conditional LTM (C-LTM) builds on LTM and uses conditional events in the UE to decide on the mobility execution on its own. The C-LTM work however needs some further discussion:

Early TA handling

It was agreed that the early TA is signalled to the UE from the source cell via a new MAC CE, and each early TA for a candidate cell is maintained by a new timer. That means UE may maintain multiple timers if UE receives multiple early TA values for the multiple candidate cells. Once the condition is met, UE switches to one of the candidate cells, called the Target cell.

How does UE handle early TA timers for the remaining candidate cells after UE is triggered to switch to the Target cell? RAN2 can discuss the following options.

- Option 1: UE releases remaining early TA values after cell switching is triggered;

- Option 2: UE releases remaining early TA values only after successful cell switching;

- Option 3: UE maintains them after Cell switching until timer expires.

Proposal 1: RAN2 to discuss how the UE handle early TA timers for other candidate cells after UE is triggered to switch to one of candidate cells.

- Option 1: UE releases remaining early TA values after cell switching is triggered;

- Option 2: UE releases remaining early TA values after successful cell switching;

- Option 3: UE maintains them after Cell switching until timer expires.

Coexistence of L1 Event and L3 event:

It was agreed that the triggering condition of conditional LTM can be based on L1 event or L3 event. RAN2 needs to discuss whether both L1 event and L3 event for a same candidate cell can be configured to UE. If allowed, the CLTM can be triggered once either of L1 event or L3 event(s) is met for the candidate cell.

Proposal 2: RAN2 to discuss whether both L1 event and L3 event can be configured for a same candidate cell. If allowed, the CLTM can be triggered once either of the L1 event or L3 event(s) is met for the candidate cell.

If the source gNB-CU decides to prepare a L3 event based CLTM candidate cell, the source CU is responsible for L3 condition generation. We understand that early TA is also applied to L3 event based CLTM. If L3 event based CLTM is configured, source DU should be aware of that this candidate cell is related to L3 condition. Otherwise, source DU may trigger cell switching towards this candidate cell via LTM cell switching command MAC CE if LTM cell switching command MAC CE is not allowed to trigger CLTM candidate cell.

Proposal 3: If L3 event based CLTM is configured, source DU should be aware of that this candidate cell is related to L3 condition. We should send an LS to RAN3 accordingly.

To support initial and subsequent conditional LTM, the configuration of execution condition is provided by source cell and candidate cell, respectively. Specifically, the CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. Both L1 event and L3 event are allowed for CLTM. RAN2 needs to discuss whether the same event type should be configured to a candidate cell for initial CLTM and subsequent CLTM.

Proposal 4: RAN2 to discuss whether the execution condition for subsequent CLTM should be same type (L1, L3 or both) or independent from the execution condition for initial CLTM. Should be included in the LS to RAN3.

LTM command MAC CE for CLTM

In conditional LTM, UE will receive LTM candidate configuration and execution condition. The framework of LTM candidate configuration should be same as legacy LTM candidate configuration. Then, the question is whether LTM cell switch command MAC CE can be used to trigger LTM cell switch toward conditional LTM candidate cell. If L1 measurement report is configured, source DU can decide whether to trigger LTM cell switch via LTM cell switch command MAC CE before the condition is met.

Proposal 5: RAN2 to discuss whether LTM cell switch command MAC CE can be used to trigger LTM cell switch toward conditional LTM candidate cell.

Scenario Discussion based on timeline of Early TA reception and C-LTM Condition fulfilment

The LTM Condition fulfilment and Early TA availability are two independent events that may occur at different point of time. In the following a few scenarios are depicted and also some basic solutions are mentioned, RAN2 should discuss these.

Scenario 1

In case of conditional handover procedure, a handover may only be executed by the UE when included condition(s) for at least one candidate cell are fulfilled. The fulfilment of radio condition(s) is not dependent on acquisition of an Early TA – UE receives it from the source.

One question therefore arises: What if at the point of fulfilment of included condition(s) for a first candidate cell, the early TA acquisition of that candidate cell has not been received by the UE? In other words, if an Early TA value is not yet available with the UE but radio condition(s) for a potential candidate is fulfilled, should it initiate afresh RACH procedure towards the candidate (target) cell?

Figure 1: Condition(s) for a candidate cell C1 is fulfilled at point A, i.e., before receiving a PDCCH Order for C1

In our opinion, the UE shall start handover procedure execution immediately after Point A. To this end, the UE shall use the PRACH resource indicated in the Conditional Configuration message – which may include a Contention Free PRACH resources. The RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 6: If the LTM condition is fulfilled before a PDCCH order for the same target cell is received or the Msg1 transmission has not started, RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Scenario 2

Figure 2: Point A is after transmitting Msg1 (a PDCCH Order for C1 was received)

Scenario 2 depicted in Figure 2 similar to Scenario1 but here the UE can wait to receive a valid TA from the source cell if Msg1 has already been transmitted. Or, to have unified behaviour same solution as in Scenario 1 applied. If however, CFRA resources distinguishing if the TA is sent back to the UE (and not to the source cell) is not available, UE may initiate CBRA, if Msg1 has not yet been transmitted.

Proposal 7: RAN2 to discuss correct UE behaviour when the LTM condition is fulfilled, a PDCCH order for the same target cell is received and the Msg1 transmission has started, UE may wait to receive an “Early” TA from the source or may initiate CBRA transmission.

Scenario 3

UE starts condition evaluation immediately, as soon as possible based on the measurement gaps received in the configuration, of one or more candidate cells. In one example, as shown in Figure 3, the condition(s) for a candidate cell C1 is fulfilled at point A, i.e., after receiving a TA for C1 from the source cell but at which point it is unclear if the TA for C1 is still valid.

Figure 3: Condition(s) for a candidate cell C1 is fulfilled at point A, i.e., after receiving an Early TA for C1

UE starts a TA-timer, which may have been configured earlier in the Conditional Configuration message.

If Point A occurs before the expiry of this timer (Case1 in Figure 3), UE transmits handover completion. If Point A occurs after the expiry of this timer (Case2 in Figure 3), UE shall start handover procedure execution immediately after Point A. To this end, the UE shall (re)use the PRACH resource indicated in the Conditional Configuration message irrespective of if a Msg1 transmission to cell C1 has been made already. Previously received configuration may include a Contention Free PRACH resources. The RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 8: A TA-timer is used to check the validity of received TA value. If LTM execution is fulfilled before the expiry of the TA-timer, UE sends the HO Complete to the target cell using RACH-less LTM procedure; else, UE may start a CFRA indicating that Msg2 be sent to the UE directly (not to the source cell).

Co-existence of condition SCG LTM and MCG failure recovery

In Rel-18 Intra-CU SCG LTM, Intra-CU SCG LTM switch is allowed while MCG failure recovery procedure is ongoing. More specifically, UE will trigger fast MCG link recovery upon RLF on MCG. After UE transmits MCGFailureInformation message to MCG via SCG, UE starts T316 and waits for the response e.g., handover command. Intra-CU SCG LTM could be triggered while T316 is running. That means the response will be pending in SCG until UE completes SCG change triggered LTM. Regarding conditional Intra-CU SCG LTM, we can follow the principle of SCG LTM.  Therefore, we propose:

Proposal 9: Conditional Intra-CU SCG LTM switch is allowed to be triggered if the execution condition is met while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

# Conclusion

This document attempted to develop a basic understanding of “conditional LTM” and proposed some first scenarios for RAN2 consideration. In summary, we have following observations and proposals:

Early TA delivery

Proposal 1: RAN2 to discuss how the UE handle early TA timers for other candidate cells after UE is triggered to switch to one of candidate cells.

- Option 1: UE releases remaining early TA values after cell switching is triggered;

- Option 2: UE releases remaining early TA values after successful cell switching;

- Option 3: UE maintains them after Cell switching until timer expires.

Coexistence of L1 Event and L3 event:

Proposal 2: RAN2 to discuss whether both L1 event and L3 event can be configured for a same candidate cell. If allowed, the CLTM can be triggered once either of the L1 event or L3 event(s) is met for the candidate cell.

Proposal 3: If L3 event based CLTM is configured, source DU should be aware of that this candidate cell is related to L3 condition. We should send an LS to RAN3 accordingly.

Proposal 4: RAN2 to discuss whether the execution condition for subsequent CLTM should be same type (L1, L3 or both) or independent from the execution condition for initial CLTM. Should be included in the LS to RAN3.

LTM command MAC CE for CLTM

Proposal 5: RAN2 to discuss whether LTM cell switch command MAC CE can be used to trigger LTM cell switch toward conditional LTM candidate cell.

Scenario Discussion based on timeline of Early TA reception and C-LTM Condition fulfilment

Proposal 6: If the LTM condition is fulfilled before a PDCCH order for the same target cell is received or the Msg1 transmission has not started, RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 7: RAN2 to discuss correct UE behaviour when the LTM condition is fulfilled, a PDCCH order for the same target cell is received and the Msg1 transmission has started, UE may wait to receive an “Early” TA from the source or may initiate CBRA transmission.

Proposal 8: A TA-timer is used to check the validity of received TA value. If LTM execution is fulfilled before the expiry of the TA-timer, UE sends the HO Complete to the target cell using RACH-less LTM procedure; else, UE may start a CFRA indicating that Msg2 be sent to the UE directly (not to the source cell).

Co-existence of condition SCG LTM and MCG failure recovery

Proposal 9: Conditional Intra-CU SCG LTM switch is allowed to be triggered if the execution condition is met while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

# References

- Chair notes for RAN2#128