3GPP TSG-RAN WG2 Meeting #129	R2-2500859

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	8.10.2.1

Source:	Samsung

Title:	MRO enhancements for LTM

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs and subsequent CPAC. From RAN2, this involves identification and specification of necessary UE reporting to enhance the mobility parameter tuning as given below. We discuss the remaining issues for MRO for LTM in this contribution.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

Reporting L1 measurements in RLF report

In RAN2#126 [2], the following agreement is made for RLF.

- =>If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Further in RAN2#127-bis [3], the following agreement is made for SHR.

=>UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

The UE reports the differential RSRP for different beams and different candidates in CSI reports for aiding the network to perform LTM cell switch. This makes sense as the CSI reports may be reported periodically and there could be a large number of reports. In the SON/MDT reports, it would be better to prioritise simplicity and include the absolute RSRP range values.

Proposal 1: L1 measurement results of the LTM candidate cells are reported using absolute RSRP-Range.

In NR-DC, there may be two set of LTM candidate cells, one associated with MCG and one associated with SCG. The L1 measurements associated with SCG are not much helpful in RLF and SHR. We also note that there is no information available with MN on whether an LTM cell is configured by SN, as there is no inter-node message defined which shares the LTM configuration from SN to MN. Considering that the measurement reporting is the main overhead for the UE (and the network) in the SON/MDT reports, which are stored, it is important to log only the necessary information.

Proposal 2: Only the L1 measurement results of the LTM candidate cells associated to MCG are logged in RLF report or SHR.

Random Access Information for LTM

We think that it is useful for logging the RA purpose for early synchronisation and LTM cell switch separately.

Proposal 3: A new RA purpose is used for LTM cell switch and early synchronisation for TA.

We understand that the random access optimisation for the early TA needs to be done by both the source cell and candidate cell. For e.g. the thresholds at which the early TA is triggered is decided by source cell, while the RA parameters are decided by the candidate cell. The configuration of thresholds will make an impact on whether early TA is successful or not. So UE may include both source cell and candidate cell identifier in the RA report.

Proposal 4: UE includes the source and candidate cell identifiers in the RA information for early synchronisation for TA.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: L1 measurement results of the LTM candidate cells are reported using absolute RSRP-Range.

Proposal 2: Only the L1 measurement results of the LTM candidate cells associated to MCG are logged in RLF report or SHR.

Proposal 3: A new RA purpose is used for LTM cell switch and early synchronisation for TA.

Proposal 4: UE includes the source and candidate cell identifiers in the RA information for early synchronisation for TA.

# References

[1] RP-234038: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] R2-2406202: RAN2#126 Meeting Report

[3] RAN2#127-bis Chair notes