3GPP TSG-RAN WG2 Meeting #129	R2-2500861

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	8.10.4

Source:	Samsung

Title:	SON/MDT for NTN

Document for:	Discussion and Decision

# Introduction

One of the objectives for Rel-19 SON/MDT WI [1] is to support Support of SON/MDT enhancements for Intra-NTN mobility. RAN3 had send a LS [2] to RAN2 including the agreements for SON/MDT enhancements for NTN. In this contribution, we discuss the objective based on the LS.

# Discussion

## MDT for NTN

RAN3 LS considers MDT enhancements for NTN. RAN3 considers two options, one based on mapped cell ID and other based on the geographical area. Both of them can be used according to RAN3, so RAN2 can consider one based on the UE overheads.

On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

Most important issue with the geographical area based Area Scope is the impact on the UE performance, especially UE’s battery performance. A UE is not required to have updated location in the RRC_IDLE and RRC_INACTIVE states. Any new requirement contrary to this will have serious implications on the UE battery performance. It will also affect the computational load of the UEs.

Observation 1: Geographical area based Area Scope for logged MDT has a huge impact on UE battery performance.

For logged MDT, the base station can configure an Area scope for the UE in RRC-LoggedMeasurementConfiguration message. Network can restrict the area in which the UE performs measurement logging to cells broadcasting any of the included cell identities, the included tracking area codes/ frequencies configure the measurement. If required, the network can use logging based on event L1 to ensure that the logging is performed based on the measurement values. Moreover, it is beneficial only in limited scenarios such as high speed UEs and even if a geographic area is configured, the geographic area itself cannot be an extension in the current Area scope IE. It is still necessary to use the current Area scope to limit the cells or routing areas measured by the UE. Otherwise, there may be other operators in the same geographical area, or other cells of the same operator that do not require MDT information collection. Operational way to further limit the measurement range of the UE is to use another separated information outside the current Area scope, and this information is only used in the NTN cell. We also note that RAN3 has agreed that mapped cell ID can be used as area scope and this will not have any major UE impacts.

Proposal 1: Geographical area based Area Scope is not supported for logged MDT.

## MRO for NTN

From RAN3 LS, RAN3 has agreed to report the fulfilled CHO trigger conditions for MRO for NTN and has informed the below in the LS:

On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

We think that it is helpful to report the fulfilled CHO trigger conditions and go with RAN3 agreement. In the RLF report, UE currently logs CHO conditions using choConfig. It makes sense to use the same for NTN also. However only two CHO conditions can be reported at a time using choConfig.

Since it is possible to configure A3/A5 events along with any of time or measurement based trigger condition, choConfig-r19 can be introduced to report one more CHO trigger condition. Since CondTriggerConfig is a choice which contains different conditions, a specific field for NTN may not be needed, and the choConfig-r19 can be introduced on similar lines as choConfig.

Proposal 2: UE reports fulfilled CHO trigger conditions for MRO for NTN using choConfig. choConfig-r19 can be introduced on similar lines as choConfig to report one more CHO trigger condition.

There could be some issues specific to NTN which may lead to radio link failures. A handover failure may occur if the Timing Advance value that the UE estimated to the target cell is incorrect. Thus including the Timing Advance field of the Timing Advance Report MAC CE, if it was triggered during the random access procedure will be helpful for the networks to identify the exact cause of failure.

Similarly, in order to get the satellite position for the UE to self-precompensate the TA, the network signals the satellite position in SIB19, also called the ephemeris. This ephemeris value has a validity duration, which forces the UE to re-acquire the SIB19 every now and then. Some of the typical cause of failures in NTN are related to the uplink validity. It will be useful to report information related to the uplink validity and ephemeris.

Proposal 3: UE reports following additional information in RLF report for NTN related failures

- Timing Advance reported in the Timing Advance Report MAC CE, if it was triggered during the random access procedure during a handover.

- A flag indicating whether T430 had expired when RLF or HOF occurred.

- Information related to satellite position.

## Supporting multiple TACs

UE logs the TAC of the relevant cells in various SON/MDT reports. CGI-Info-Logging including tracking area code of the cell. Since a NTN cell can have multiple TAC(s), UE may log the tackingAreaList when it is logging CGI-Info-Logging in various SON/MDT reports.

Proposal 4: UE may log tracking area list in CGI-Info-Logging in various SON/MDT reports.

# Conclusion

Based on the discussion, we have the following observation and proposals.

Observation 1: Geographical area based Area Scope for logged MDT has a huge impact on UE battery performance.

Proposal 1: Geographical area based Area Scope is not supported for logged MDT.

Proposal 2: UE reports fulfilled CHO trigger conditions for MRO for NTN using choConfig. choConfig-r19 can be introduced on similar lines as choConfig to report one more CHO trigger condition.

Proposal 3: UE reports following additional information in RLF report for NTN related failures

- Timing Advance reported in the  Timing Advance Report MAC CE, if it was triggered during the random access procedure during a handover.

- A flag indicating whether T430 had expired when RLF or HOF occurred.

- Information related to satellite position.

Proposal 4: UE may log tracking area list in CGI-Info-Logging in various SON/MDT reports.

# References

[1] RP-234038: New WID on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] R3-247908: LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT