3GPP TSG-RAN WG2 Meeting #129					           R2-2500890

Athens, Greece, 17 – 21 February, 2025

Agenda Item:		8.3.2

Source:			Huawei, HiSilicon

Title:			Additional simulation results for RRM measurement prediction

Document for:		Discussion and Decision

# 1   Introduction

In the previous meetings RAN2 analysed RRM measurement prediction accuracy when using AIML models in different scenarios. The following scenarios were prioritized for evaluation:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

We have provided our simulation results for these scenarios in [1] and in this Tdoc we provide some additional results, in particular to:

- Compare the prediction accuracy of AIML models with that of simple non-AIML methods

- Compare the prediction accuracy of cell-based approach with that of cluster-based approach

- Verify the impact of UE location information being used as an additional input to the model on the prediction accuracy

# 2   Simulation results

## 2.1 AIML vs. non-AIML methods

We have compared the prediction accuracy of the AIML model we use in our previous simulations with the results achieved when a simple non-AIML method, i.e. same and hold, is used. The model details for both cases are provided in the table below.

Table 1. Model details for the AIML and non-AIML methods used in the simulations

| Model input | Model output/label | Model type | Detail of model | Model complexity in a number of model parameters (K) | Model complexity in a number of model size (KByte) | Computational complexity [FLOPs] (M) | Training data size in number of samples (K) | Testing data size in number of samples (K) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 cell, L3 cell-level RSRP | 1 cell, L3 cell-level RSRP | AI: LSTM | 3 LSTM layers and 1 full-connectivity layer | 80 | 350 | 1.03 | 4300 | 30 |
| 1 cell, L3 cell-level RSRP | 1 cell, L3 cell-level RSRP | Non-AI: Sample and hold |  |  |  |  | 4300 | 30 |

The prediction accuracy was compared for Scenario 2, i.e. intra-frequency Case B in FR1 for the following cases:

- MRRT=50%, OW=600ms, PW =200ms, UE speeds: 30, 60 and 90km/h

- MRRT=66%, OW=800ms, PW =400ms, UE speeds: 30, 60 and 90km/h

L1 filtering with non-sliding window was used for all simulations. We simulated MRRT of 50% and 66%, according to the following patterns:

| Measurement | Prediction |
| --- | --- |

|  |  |  |  | MRRT=50% |
| --- | --- | --- | --- | --- |

|  |  |  |  |  |  | MRRT=66% |
| --- | --- | --- | --- | --- | --- | --- |

The remaining simulation assumptions were aligned with those agreed by RAN2 earlier and details can be found in the excel sheet attached to this Tdoc. The table below provides the

The results are presented in tables below.

Table 2. AIML vs. non-AIML comparison for scenario 2 with MRRT=50%, OW=600ms, PW=200ms

|  | Average L3 cell RSRP difference (dB) |  |
| --- | --- | --- |
| UE speed | AI Method | Non-AI Method (S&H) |
| 30km/h | 0.96 | 0.84 |
| 60km/h | 1.59 | 1.89 |
| 90km/h | 1.93 | 2.04 |

Table 3. AIML vs. non-AIML comparison for scenario 2 with MRRT=66%, OW=800ms, PW=400ms

|  | Average L3 cell RSRP difference (dB) |  |
| --- | --- | --- |
| UE speed | AI Method | Non-AI Method (S&H) |
| 30km/h | 1.93 | 1.86 |
| 60km/h | 2.92 | 2.97 |
| 90km/h | 3.68 | 3.98 |

Based on the above results we can observe that sample and hold method achieves comparable results to those achieved by the used AIML method for lower UE speeds, but performs worse for higher UE speeds, in particular:

- For UE speed of 30 km/h, the results are slightly better for sample and hold (0.07 – 0.12 dB in favour of S&H)

- For UE speed of 60km/h, the results are slightly for the AIML method (0.05 – 0.3 dB in favour of AIML method)

- For UE speed of 90km/h, the results are also better for the AIML method (0.11 – 0.3 dB in favour of AIML method)

The small difference between AIML and non-AIML for the lower UE speeds is a consequence of both methods being impacted by the noise. Since the channel conditions for lower speeds do not change abruptly, sample and hold can achieve good prediction accuracy. For higher speeds on the other hand the channel conditions change more dynamically and sample and hold cannot perform as well as utilized AIML method.

Observation 1: For scenario 2 (intra-frequency case B), sample and hold method achieves similar prediction accuracy as AI method for lower UE speeds (up to 60 km/h).

Observation 2: An advantage of AIML method over sample and hold becomes more visible for higher UE speeds (especially 90 km/h but also 60 km/h), where channel conditions change more dynamically.

## 2.2 Cluster-based vs. cell-based prediction

Our previous simulation results focused on a single-cell approach where measurement results from only a single cell are used as input to the model, and the output is also an RSRP prediction of a single cell. RAN2 agreed to study also a so-called cluster approach where measurement results from more than one cell are used as input to the model. In this section we present the results of the comparison we did for these two approaches in scenario 2, i.e. intra-frequency case B in FR1, with simulation assumptions as agreed by RAN2 previously and for the following settings:

- MRRT: 50%

- OW=600ms, PW=200ms

- RRM sub-use case 2, non-sliding L1 filtering

Table 4. Cell vs. cluster approach for scenario 2 with MRRT=50%, OW=600ms, PW=200ms

|  | Average L3 cell RSRP difference (dB) |  |
| --- | --- | --- |
| UE speed | Cell approach | Cluster approach (21 cells) |
| 30 km/h | 0.96 | 5.8 |
| 60 km/h | 1.59 | 6.3 |
| 90 km/h | 1.93 | 5.8 |

As can be seen, in the simulated scenario cluster-based approach performs much worse than single-cell approach. Using measurements from neighbouring cells seems to only introduce more noise into the model and is not helpful at all.

Observation 3: For scenario 2 (intra-frequency case B), single cell approach outperforms significantly cluster based approach.

## 2.3 Impact of using UE location information

Another dimension that we studied in addition to the previous results was an impact of feeding the AIML model with UE location information, in addition to RSRP measurements. The simulated scenario was again scenario 2, i.e. intra-frequency case B in FR1, with simulation assumptions as agreed by RAN2 previously and with the following settings:

- MRRT: 50%

- OW=600ms, PW=200ms

- RRM sub-use case 2, non-sliding L1 filtering

The results of this exercise are gathered in the table below.

Table 5. Impact of using UE location information for scenario 2 with MRRT=50%, OW=600ms, PW=200ms

|  | Average L3 cell RSRP difference (dB) |  |
| --- | --- | --- |
| UE speed | Without UE location info | With UE location info |
| 30 km/h | 0.96 | 1.07 |
| 60 km/h | 1.59 | 1.68 |
| 90 km/h | 1.93 | 1.96 |

As can be seen, in the simulated scenario, using UE location information impacted the RSRP prediction accuracy negatively, with results being 1.5–11% worse compared to the case where location information was not used. It can be then seen that UE location information introduced noise to the model.

Observation 4: For scenario 2, using UE location information as an input to the AIML model decreases the RSRP prediction accuracy comparing to the case where location information was not used.

# 3   Summary

Based on the simulations presented in this paper, the following observations are made:

AIML vs. non-AIML

Observation 1: For scenario 2 (intra-frequency case B), sample and hold method achieves similar prediction accuracy as AI method for lower UE speeds (up to 60 km/h).

Observation 2: An advantage of AIML method over sample and hold becomes more visible for higher UE speeds (especially 90 km/h but also 60 km/h), where channel conditions change more dynamically.

Single cell vs cluster approach

Observation 3: For scenario 2 (intra-frequency case B), single cell approach outperforms significantly cluster based approach.

Impact of UE location information

Observation 4: For scenario 2, using UE location information as an input to the AIML model decreases the RSRP prediction accuracy comparing to the case where location information was not used.

# References

- R2-2410539	Simulation results for RRM measurement prediction	Huawei, HiSilicon

# Annex A – simulation assumptions

Below are the company selectable assumptions chosen by us.

| Configurable/selectable simulation assumption |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spatial consistency | Whether LOSsoft is modeled or not | UE trajectory model | UE trajectory boundary processing | L1/L3 filtering (sliding/non-sliding) | Number of Tx beams | Number of Rx beams |
| Procedure B in TR38.901 | Not modelled | Option 1 in 38.843 section 6.3.1 | Wrap-around model | Both non-filtered and non-sliding L1 filtering cases were simulated | FR1: 4 FR2: 64 | FR1: 1 FR2: 8 |

All the simulation assumptions are listed in the table below with the selectable ones highlighted in yellow.

Table 1: Summarized simulation assumptions

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft and inter-frequency correlation; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction | 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure B in TR38.901 | Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | wrap round model | wrap round model |