3GPP TSG-RAN WG2 Meeting #129	R2-2500894

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.0.2.16

Source: 	Huawei, HiSilicon

Title: 	Discussion for DSR cancellation in DC

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, the DSR procedure in dual-connectivity scenario is discussed.

# 2.	Discussion

For R18 DSR, the DSR triggering is specified as follows. MAC entity triggers a DSR if the highlighted DSR triggering conditions are met.

3GPP TS 38.321 V18.4.0 (2024-12) ### 5.4.9 Delay status reporting …… If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Furthermore, the cancellation of a pending DSR is specified as follows:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR. NOTE 2: It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

In case of DC scenario, a split bearer which is a radio bearer with RLC bearers both in MCG and SCG can be configured to UE. An example of a split bearer from a UE perspective is shown below in Figure (1), where the split bearer includes the primary RLC entity (MN RLC) and primary MAC entity (MN MAC) for MCG, and includes the split secondary RLC entity (SN RLC) and the split secondary MAC entity (SN MAC) for SCG. The MN MAC entity and the SN MAC entity in the split bearer performs the DSR procedure respectively according to the specified DSR procedure.

Figure (1) Example of a split bearer from a UE perspective

At reception of a PDCP SDU from upper layers, the PDCP entity submits the buffered PDCP SDU for a split bearer according to total data volume. Specifically, the PDCP entity submit the buffered PDCP SDU to either primary RLC entity or split secondary RLC entity if the total data volume is equal to or larger than ul-DataSplitThreshold, otherwise, the PDCP entity submit the buffered PDCP SDU only to the primary RLC entity, wherein the total data volume includes the total amount of PDCP data volume and RLC data volume pending for initial transmission both in the primary RLC entity and the split secondary RLC entity.

Taking the case where the MN MAC and SN MAC are configured with the same remainingTimeThreshold as an example, assuming that the PDCP entity receives PDCP SDU#1 from upper layers at T0 as shown in Figure (2), at T1, both MN MAC and SN MAC trigger a DSR if the remaining time of PDCP SDU#1 which is buffered in PDCP entity becomes below the configured remainingTimeThreshold and the total data volume (including both delay-critical and non-delay critical data) is above the configured ul-DataSplitThreshold. However, subsequently PDCP will submit PDCP SDU#1 to only of MN RLC or to SN RLC, which means that the PDCP SDU#1 may be transmitted either via MCG or SCG and it is not predictable which RLC entity will receive PDCP SDU#1 after triggering the DSR.

Observation 1:	In Dual Connectivity scenario, DSR will be triggered both in primary and secondary MAC entity, when the remaining time of a PDCP SDU buffered in PDCP entity becomes below the remainingTimeThreshold of the primary MAC entity and the remainingTimeThreshold of the secondary MAC entity.

At T2, a MN MAC PDU including PDCP SDU#1 is sent. For example, the MN MAC obtains an UL grant and informs the RLC entity and PDCP entity to submit UL data available for transmission to generate the MAC PDU, so a MN MAC PDU including PDCP SDU#1 is sent at T2. Then MN MAC will cancel the pending DSR according to the cancelling condition “when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR”, as copied above. However, SN MAC cannot cancel SN’s pending DSR based on this condition, because PDCP SDU#1 is sent by the MN path, not SN path. Later on, if the SN MAC has an UL grant, the SN MAC may send a SN MAC PDU at T3. Since a DSR has been triggered previously and not cancelled, this MAC PDU would include a DSR MAC CE, even though the data which triggered and was supposed to be included in the DSR had been already sent via MN path.

Observation 2:	In Dual Connectivity case, a MAC entity is required to unnecessarily send a DSR even though all the associated data had been already sent by the other MAC entity.

Furthermore, based on current specifications, SN MAC cannot cancel the pending DSR even after sending the DSR, because the DSR cancelation condition (highlighted in blue above) cannot be satisfied, because the PDCP SDU#1 which is the PDCP SDU associated with SN’s pending DSR has been sent by the other MAC entity and PDCP SDU#1 is not available for transmission in SN MAC. In other words, the DSR cannot contain information about the “PDCP SDUs associated with the DSR” as they no longer exist, which makes it impossible to meet the current DSR cancellation conditions.

Observation 3:	According to current specifications, in Dual Connectivity case, the MAC entity cannot cancel a pending DSR in case all the data associated with this DSR has been sent by the other MAC entity.

Figure (2) Example of DSR procedure for split bearer when MN and SN MAC have same remainingTimeThreshold

As per the above, the UE would have to either report wrong information to the gNB which would impact the scheduling and can lead to waste of resources, or otherwise the UE may not be able to cancel the pending DSR. Meanwhile, the issue can be easily resolved with small modifications to the current cancelling conditions in TS 38.321 section 5.4.9 and we provide several options below.

Option 1 After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR, or when upper layers indicate that the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

Option 2 After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted by any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Option 3 After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR, or if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

Option 4 After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR, or all the PDCP SDUs associated with the DSR become unavailable for transmission even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

Proposal 1:	RAN2 should address an issue of the UE having to send and being unable to cancel the pending DSR even though no delay-critical data is available in the MAC entity, e.g. suing one of the four proposed options.

In our view, option 1 seems the simplest and cleanest approach and we provide a draft CR for this option in the Appendix.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for DSR enhancements. The following observations and proposals were made:

Observation 1:	In Dual Connectivity scenario, DSR will be triggered both in primary and secondary MAC entity, when the remaining time of a PDCP SDU buffered in PDCP entity becomes below the remainingTimeThreshold of the primary MAC entity and the remainingTimeThreshold of the secondary MAC entity.

Observation 2:	In Dual Connectivity case, a MAC entity is required to unnecessarily send a DSR even though all the associated data had been already sent by the other MAC entity.

Observation 3:	According to current specifications, in Dual Connectivity case, the MAC entity cannot cancel a pending DSR in case all the data associated with this DSR has been sent by the other MAC entity.

Proposal 1:	RAN2 should address an issue of the UE having to send and being unable to cancel the pending DSR even though no delay-critical data is available in the MAC entity, e.g. suing one of the four proposed options.

# 4. Appendix

3GPP TSG-RAN WG2 Meeting #129

Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for DSR cancellation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Dual Connectivity, a DSR can be triggered both in primary and secondary MAC entity, when the remaining time of a PDCP SDU buffered in PDCP entity becomes below the remainingTimeThreshold of the primary MAC entity and the remainingTimeThreshold of the secondary MAC entity. One of the MAC entity cannot cancel a pending DSR according to the current DSR cancellation conditions if the delay-critical data associated with the pending was sent by the other MAC entity and is not available in the MAC entity in question. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.4.9, clarify that when upper layers indicate that the volume of delay-critical data associated with the DSR is zero, the DSR is cancelled by the MAC entity. Impact analysis: Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: XR, DSR cancelation Inter-operability: There are no interoperability issues related to the CR implementation as the CR only impacts the UE behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In Dual Connectivity, a UE may end up with a constantly pending DSR as a MAC entity cannot cancel a pending DSR according to the current DSR cancellation conditions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the logical channel:

2>	trigger a DSR for the logical channel.

If there is at least one DSR pending, the MAC entity shall:

1>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72.

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE 1:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR, or when upper layers indicate that the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.