3GPP TSG RAN WG2 #129		                                                       R2-2500921

Athens, Greece, 17th – 21st February 2025

Agenda item:	8.5.3

Source: 	Vodafone

Title: 	Views on On-demand SIB1 operation for idle/inactive UEs

Document for:	Discussion

- Introduction

In the RAN#106 meeting [1], the following way forward related to the on-demand SIB1 operation for idle/inactive UEs was agreed:

Way Forward on NES RAN confirms that Case 2 also includes the case where a particular Cell A can switch to become a NES Cell. This means that Cell A can provide UL-WUS configuration for itself before becoming a NES cell. - No further optimizations will be considered for NES UEs for this scenario. - No further optimizations to handle non-NES capable UEs will be considered for this scenario. It is assumed that these UEs can be handled by careful deployment. - No WID update is required On RRC Connected mode - The scope of WID will not be updated to include RRC Connected. - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission. No need to change the scope of WI to indicate this.

In this contribution we present our views on a remaining aspect regarding the way forward and related discussion in RAN1.

- Discussion

## 2.1 Handling of a Cell A becoming a NES Cell

Now that it was agreed that a Cell A can switch to become a NES Cell, in multicarrier scenarios UEs no longer have to unnecessarily re-select to another cell to obtain the UL WUS configuration, as implicitly there would be an indication that the Cell A is about to become NES Cell by sending its own UL WUS configuration.

Observation 1: A Rel-19 NES-capable UE expects Cell A to become a NES cell if the Cell A transmits its own UL WUS configuration.

One remaining issue may appear depending on the scenario when the switch happens. From standard point of view, with this agreement there is no restriction for any Cell A to become a NES Cell, even for the case when it is the only cell providing coverage to the UEs that are camped in the cell (for example nighttime when capacity layer is switched OFF and coverage layer is kept ON). In this single cell scenario, it would be important to prevent a Cell A to become NES Cell to avoid the creation of coverage holes, for UEs trying to access the cell. So, in these scenarios, when a Cell A becomes NES Cell, upon the transmission of UL WUS by Rel-19 UEs that were camping already in the cell, it would be important that this cell falls back to being a Cell A and transmitting its own SIB1 as before, rather than transmit OD-SIB1 as NES Cell. Alternatively, it should be captured that the gNB should not switch to NES mode without ensuring that it will not lead to service degradation for the UEs under its coverage.  We would like to bring this into discussion and also how to capture in specification (as usually gNB behaviour is not specified), for example an alternative would be by adding Stage 2 (TS 38.300) text describing the behaviour above

Proposal 1: For single-carrier scenario, when a Cell A switches to become NES Cell, upon receiving UL WUS it should fall back to become Cell A rather than sending OD-SIB1 as NES Cell. Alternatively, the gNB should not switch to NES mode without ensuring that it will not lead to service degradation for the UEs under its coverage.

- FFS: Discuss how to capture this in specification, e.g. Stage 2 specification (TS 38.300).

- Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: A Rel-19 UE expects Cell A to become a NES cell if the Cell A transmits its own UL WUS configuration.

Proposal 1: For single-carrier scenario, when a Cell A switches to become NES Cell, upon receiving UL WUS it should fall back to become Cell A rather than sending OD-SIB1 as NES Cell. Alternatively, the gNB should not switch to NES mode without ensuring that it will not lead to service degradation for the UEs under its coverage.

- FFS: Discuss how to capture this in specification, e.g. Stage 2 specification (TS 38.300).

# References

- RP-243297, Way forward on NES, RAN2 Chair (Interdigital), 3GPP RAN#106 meeting, 2024.