3GPP TSG-RAN WG2 Meeting #129	R2-2500948

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.3.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Generalization of ML mobility use-cases

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#128 the following agreements on the generalization performance were made:

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. 2 Companies can choose which case they compare with and should report it with simulation results. 3 Generalization issues on RRM measurement prediction are prioritized.

In this contribution, we present our views on the agreed generalization cases and suggest introducing a new generalization case which can be used to evaluate the model’s ability to interpolate between different distributions. We also provide some preliminary simulation results and discussion on the simulation assumptions.

# 2	Discussion

In addition to the agreed generalization cases listed in the introduction, generalization could be studied when training with a more diverse dataset and testing with an unforeseen configuration. More specifically, training the model with UE speeds S1 and S3, and testing with S2. This evaluates, how well the model is able to generalize and interpolate the two types of configurations it has seen in the training dataset. The difference compared to GC#1 is in the more diverse training dataset. The updated generalization case Table 2-1 is shown below, with a new case (GC#3) added. To limit the evaluation effort, GC#3 may be studied with only one set of speeds, i.e., train with S1 and S3 and test with S2.

Table 2-1 Simulation combinations for FR1 and FR2 generalization study on UE speed

|  | Training @Dataset: S1 | Training @Dataset: S2 | Training @Dataset: S3 | Inference @S1 | Inference @S2 | Inference @S3 |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 | Yes | Yes |  | Yes | Yes | Yes |
| GC#1 |  |  | Yes | Yes |  |  |
| GC#2 | Yes | Yes | Yes | Yes |  |  |
| GC#3 |  | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1 | Yes | Yes |  | Yes | Yes | Yes |
| GC#1 |  |  | Yes |  | Yes |  |
| GC#2 | Yes | Yes | Yes |  | Yes |  |
| GC#3 | Yes |  | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1 | Yes |  | Yes | Yes | Yes | Yes |
| GC#1 |  | Yes |  |  |  | Yes |
| GC#2 | Yes | Yes | Yes |  |  | Yes |
| GC#3 | Yes | Yes |  |  |  | Yes |

Observation 1: Assessing the ability of a model to generalize and interpolate between two types of configurations in the training dataset (e.g., training with S1 and S3 km/h and testing with S2 km/h) would be of practical interest.

Proposal 1: RAN2 to discuss the addition of Generalization Case #3 (GC#3), in which training dataset contains speeds S1 and S3, and the test dataset speed S2, where S1 < S2 < S3.

# 3	Evaluation

## 3.1	RRM Measurement Prediction Case A

To assess the generalization capabilities of RRM measurement prediction in temporal case A, we conducted a study using different generalization configurations, as outlined in Table 2-1. To ensure a fair comparison, the inference data remained consistent across the baseline and generalization configurations (GC#1, GC#2, and GC#3). The simulation assumptions are highlighted in Table 3.1-1.

Table 3.1-1: Simulation parameters for the 7-site scenario based on TR38.843 (FR2)

| Reported simulation assumptions | UE trajectory option | Option 3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |  |  |  |
|  | UE speed | 30/60/120 km/h |  |  |  |
|  | SoftLOS | Yes |  |  |  |
|  | Spatial consistency option | Option B |  |  |  |
|  | Number of TX beams | 8 |  |  |  |
|  | Number of RX beams | 4 |  |  |  |
|  | Measurement reduction rate | N/A |  |  |  |
|  | Sub-use-case | Case 2 (cell to cell) |  |  |  |
|  | Observation window (OW) | 200 ms |  |  |  |
|  | Prediction window (PW) | 200 ms |  |  |  |
| Data Size (Number of Samples) | Training | Baseline | GC#1 | GC#2 | GC#3 |
|  |  | 350MB | 500MB | 1350MB | 1000MB |
|  | Testing | 150MB |  |  |  |
| AI/ML model input/output | Model input | L3 filtered serving cell RSRP measurements |  |  |  |
|  | Model output | L3 filtered serving cell RSRP measurements |  |  |  |
| AI/ML model description | Model type | Linear regression, Random forest, Gradient Boosting, |  |  |  |
|  | Model complexity in number of parameters | Linear regression, Random forest, Gradient Boosting: N/A |  |  |  |

The evaluation results are then presented in the sequel for different baseline speed categories.

Table 3.1-2: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different generalization configurations and AI/ML models, baseline UE speed 30km/h

| Method GC | Linear regression | Random forest | Gradient boosting |
| --- | --- | --- | --- |
| Baseline (30km/h) | 1.5128 | 1.3988 | 1.3962 |
| GC#1 (60km/h) | 1.5199 | 1.4504 | 1.4332 |
| GC#1 (120km/h) | 1.5634 | 1.5414 | 1.5037 |
| GC#2 | 1.5224 | 1.4415 | 1.4329 |
| GC#3 | 1.5373 | 1.4757 | 1.4544 |

Figure 3.1-1: Average MAE comparisons for different generalization configurations, 30km/h baseline

Table 3.1-3: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different generalization configurations and AI/ML models, baseline UE speed 60km/h

| Method GC | Linear regression | Random forest | Gradient boosting |
| --- | --- | --- | --- |
| Baseline (60km/h) | 2.0357 | 1.8550 | 1.8992 |
| GC#1 (30km/h) | 2.1088 | 1.9172 | 1.9504 |
| GC#1 (120km/h) | 2.0526 | 1.9386 | 1.9525 |
| GC#2 | 2.0369 | 1.8540 | 1.9021 |
| GC#3 | 2.0382 | 1.8762 | 1.9166 |

Figure 3.1-2: Average MAE comparisons for different generalization configurations, 60km/h baseline

Table 3.1-4: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different generalization configurations and AI/ML models, baseline UE speed 120km/h

| Method GC | Linear regression | Random forest | Gradient boosting |
| --- | --- | --- | --- |
| Baseline (120km/h) | 2.6778 | 2.5076 | 2.5714 |
| GC#1 (30km/h) | 2.7971 | 2.7417 | 2.7868 |
| GC#1 (60km/h) | 2.6872 | 2.5439 | 2.6195 |
| GC#2 | 2.6843 | 2.4776 | 2.5908 |
| GC#3 | 2.7145 | 2.5366 | 2.6391 |

Figure 3.1-3: Average MAE comparisons for different generalization configurations, 120km/h baseline

The above simulation results analysis on generalization study for RRM measurement prediction temporal domain case A led to the following observations and proposals.

Observation 2: GC#1 generally exhibits the worst performance in terms of MAE. The larger the speed difference between the training data and the baseline, the higher the MAE tends to be.

Observation 3: GC#2 achieves performance comparable to the baseline and demonstrates the best generalization as compared to GC#1 and GC#3.

Observation 4: GC#3 performs below the baseline, but better than GC#1 and worse than GC#2.

Observation 5: Generalization across speed categories shows little variation among different configurations. This may be due to the limited simulation scenario, which may not induce significant measurement differences across speed settings and UE trajectories.

### 3.2 Additional Simulation Assumptions for Generalization

Considering the agreed-on trajectory options, the UEs move in a random walk fashion, making it difficult for an ML model to learn any recurring patterns in the UE's mobility. In the sequel, we present a simulation scenario wherein the UEs move in straight parallel lines along a street of width 12 meters. The UEs begin their trajectory at a random point on the beginning of the street and move along the street till they reach its end. The simulation time is adjusted such that all UEs of different speeds move through the full length of the street once. This is illustrated in Figure 3.2-1. We call this the “street trajectory”.

Observation 6: Using a random-walk UE trajectory inhibits the ML model’s ability to learn recurring patterns in the data.

Proposal 2: RAN2 to consider a UE trajectory option where a pattern can be more easily learned by an ML model, e.g. UEs moving along a street.

Figure 3.2-1: Illustration of the UEs trajectories used in our simulations

Further, the random seed for the large-scale propagation (slow fading, LOS/NLOS) are held constant across the simulation drops, while the random seed for the small-scale propagation change across the drops. The rationale behind holding large-scale propagation random seed fixed is that it’s otherwise very challenging for a machine learning model to learn the patterns in UE RSRP measurements, which are used as input data. This is because if both large-scale and small-scale random seeds change across simulation drops, we found that there is a very large variation in the RSRP time series of UEs that move along the exact same trajectory in each drop.

Consider, for example, if there is a coverage hole at the end of a trajectory leading to a mobility event (e.g. handover failure). It would be difficult for an ML model to learn that the RSRP time series associated with that trajectory frequently leads to a handover failure, because the variation between those trajectories across simulation drops is significant. At the very least, we should expect some correlation between the RSRP time series of UEs moving along the exact same trajectory. Otherwise, there would be no pattern to learn for an ML model.

On the other hand, if we were to hold all propagation random seeds constant, UE moving along identical trajectories would have identical RSRP time series, which is unrealistic. Therefore, we decided to add some variation to the propagation conditions across the drops (small-scale propagation) while also adding some variation in the UE trajectories across the drops (each UEs starts from a random position at the beginning of the street).

Observation 7: Changing all the propagation random seeds across the drops makes it challenging to learn patterns in the RSRP time series, because in that case, UEs following identical trajectories would have vastly different RSRP time series.

Proposal 3: RAN2 to investigate additional generalization scenarios, such as the usage of different random seeds (e.g., LOS/NLOS scenarios), beyond speed categories.

# 3	Conclusion

This document has made the following observations:

Observation 1: Assessing the ability of a model to generalize and interpolate between two types of configurations in the training dataset (e.g., training with S1 and S3 km/h and testing with S2 km/h) would be of practical interest.

Observation 2: GC#1 generally exhibits the worst performance in terms of MAE. The larger the speed difference between the training data and the baseline, the higher the MAE tends to be.

Observation 3: GC#2 achieves performance comparable to the baseline and demonstrates the best generalization as compared to GC#1 and GC#3.

Observation 4: GC#3 performs below the baseline, but better than GC#1 and worse than GC#2.

Observation 5: Generalization across speed categories shows little variation among different configurations. This may be due to the limited simulation scenario, which may not induce significant measurement differences across speed settings and UE trajectories.

Observation 6: Using a random-walk UE trajectory inhibits the ML model’s ability to learn recurring patterns in the data.

Observation 7: Changing all the propagation random seeds across the drops makes it challenging to learn patterns in the RSRP time series, because in that case, UEs following identical trajectories would have vastly different RSRP time series.

And proposed the following:

Proposal 1: RAN2 to discuss the addition of Generalization Case #3 (GC#3), in which training dataset contains speeds S1 and S3, and the test dataset speed S2, where S1 < S2 < S3.

Proposal 2: RAN2 to consider a UE trajectory option where a pattern can be more easily learned by an ML model, e.g. UEs moving along a street.

Proposal 3: RAN2 to investigate additional generalization scenarios, such as the usage of different random seeds (e.g., LOS/NLOS scenarios), beyond speed categories.