3GPP TSG RAN WG2 Meeting #129	R2-2500963

Athens, Greece, 17 - 21 February 2025

Agenda Item:	7.2

Source:	Huawei, HiSilicon

Title:	MAC CR rapporteur summary

Document for: Discussion and Decision

# 1	Introduction

This document is a summary of proposals on TS 38.321 for LTM.

# 2	Discussion

The following documents include proposals for, or related to TS 38.321:

| R2-2500173 | Corrections related to random access for LTM | Samsung |
| --- | --- | --- |
| R2-2500728 | Discussion on LTM cell switch with CFRA | Google Korea LLC |

## 2.1	Release of CFRA resources

In R2-2500173, it is raised that:

-	upon RA completion, in TS 38.321 clause 5.1.6, dedicated CFRA resources are released;

-	upon T304 expiry, in TS 38.331 clause 5.3.5.8.3, dedicated CFRA resources are released;

-	upon successful completion of RACH-less LTM cell switch, there is no statement about dedicated CFRA resources.

Therefore, it is proposed to add the release explicitly in TS 38.331:

1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

2> release dedicated preambles provided in rach-ConfigDedicated if configured in the reconfigurationWithSync;

2> release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured in the reconfigurationWithSync;

The MAC CR rapporteur agrees that this is missing.

Proposal 1: Agree the 38.331 TP in R2-2500173 on release of dedicated CFRA resources upon RACH-less LTM cell switch completion.

In R2-2500728, it is raised that, in the reconfiguration with sync procedure in TS 38.331, MAC reset is called, which releases dedicated RACH resources and that it is not correct when these resources are to be used for LTM cell switch:

##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. <unrelated part omitted> 1> else (sl-PathSwitchConfig is not included): <unrelated part omitted> 2> else: 3> reset the MAC entity of this cell group;

and

## 5.12 MAC Reset If a reset of the MAC entity is requested by upper layers upon receiving RRCResume or RRCSetup, the MAC entity shall: <unrelated part omitted> Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall: <unrelated part omitted> 1> discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

However, the MAC rapporteur's understanding is that this reset is executed before the dedicated RACH resources in the LTM target configuration are applied, so the discard above does not discard such resources:

##### 5.3.5.5.2	Reconfiguration with sync

The UE shall perform the following actions to execute a reconfiguration with sync.

[...]

1>	if sl-PathSwitchConfig is included:

[...]

1>	else (sl-PathSwitchConfig is not included):

2>	If any DAPS bearer is configured:

[...]

2>	else:

3>	reset the MAC entity of this cell group;

[...]

3>	configure lower layers in accordance with the received spCellConfigCommon;

3>	if rach-LessHO is included:

4>	configure lower layers in accordance with rach-LessHO for the target SpCell;

3>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

[...]

Therefore, no change seems needed.

Proposal 2: The changes proposed in R2-2500728 are not pursued.

## 2.2	UL carrier for early RACH

In R2-2500173, it is raised that "it is not clear which EarlyUL-SyncConfig is selected by UE as there can be two EarlyUL-SyncConfig IEs, one each for SUL and NUL."

As a solution, it is proposed the following change:

5.1.1b	Selection of the set of Random Access resources for the Random Access procedure

:

1>	else if this Random Access procedure is initiated by PDCCH order for an LTM candidate cell:

2>	select the set of Random Access resources configured in EarlyUL-SyncConfig of the selected UL carrier, corresponding to the cell indicated by the field Cell indicator in the PDCCH order as specified in TS 38.212 [9].

The MAC CR rapporteur would like to raise that the following is specified in TS 38.212 clause 7.3.1.2.1:

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator - 1 bit.

-	If the Cell indicator field is absent or the Cell indicator field indicates serving cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1;

-	If the Cell indicator field indicates a candidate cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with ltm-EarlyUL-SyncConfigSUL in LTM-Candidate for the candidate cell, this field indicates which UL carrier in the candidate cell to transmit the PRACH according to Table 7.3.1.1.1-1;

-	Otherwise, this field is reserved.

And TS 38.331 includes:

LTM-Candidate field descriptions

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

From these descriptions, which EarlyUL-SynConfig to use seems clear, without the need of any other procedure text. Besides, in the TP, "the selected carrier" is misleading as the carrier is actually indicated in the PCCH order.

Proposal 3: The 38.321 TP in R2-2500173 is not pursued.

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1: Agree the 38.331 TP in R2-2500173 on release of dedicated CFRA resources upon RACH-less LTM cell switch completion.

Proposal 2: The changes proposed in R2-2500728 are not pursued.

Proposal 3: The 38.321 TP in R2-2500173 is not pursued.

# 4	References

- R2-2500173 Corrections related to random access for LTM	Samsung

- R2-2500728 Discussion on LTM cell switch with CFRA	Google Korea LLC