3GPP TSG-RAN WG2 #129	R2-2500973

Athens, Greece, 2025-02-17 - 2025-02-21

Agenda Item:	8.16

Source:	Ericsson

Title:	Remaining issues of BDS B2b Signal Addition

Document for:	Discussion, Decision

# Introduction

In Rel-19 as per 3GPP WI  a BeiDou Navigation Satellite System Signal B2b is being specified  [1]. The WID [2] of Rel-19 BDS_B2b is as follows:

Specify A-BDS enhancements in Rel-19, including: – Supporting new signal of BDS B2b in A-GNSS for both E-UTRA and NR [RAN2]; – Specify corresponding assistant information in dedicated signaling [RAN2].

In this contribution, we discuss the remaining issues/improvements for BDS LPP impacts.

# 2	Discussion

Field Description BDS-ClockModel2

BDS-ClockModel2 field descriptions

bdsTgdB1Cp Parameter TGDB1Cp Group delay differential of the B1C pilot component (seconds), see [39], 7.6.1 and [49], 7.6.1. In the case of BDS B2b_I, this field shall be ignored. Scale factor is 2-34 seconds.

bdsIscB1Cd Parameter ISCB1Cd Group delay differential between the B1C data and pilot components (seconds), see [39], 7.6.1. In the case of BDS B2b_I, this field shall be ignored. Scale factor is 2-34 seconds.

And field description of NavModel-BDS-KeplerianSet2

NavModel-BDS-KeplerianSet2 field descriptions

bdsIODE Parameter, Issue Of Data, Ephemeris (IODE), see [39], 7.4.1 and [49], 7.4.1. In the case of BDS B2b_I, this field shall be ignored.

From the above field description, the statement that “In the case of BDS B2b_I, this field shall be ignored” is ambiguous.

It is unclear as why the legacy field would be ignored when clock and navigation model are being provided from the new signal B2b.  The reason should be clarified.

Below we suggest via text proposal to solve this via Note as below

#### –	BDS-ClockModel2

The IE BDS-ClockModel2 is used for BDS B1C defined in [39], BDS B2a defined in [49] and BDS B2b defined in [XX].

-- ASN1START

BDS-ClockModel2-r16 ::= SEQUENCE {

bdsToc-r16		INTEGER (0..2047),

bdsA0-r16		INTEGER (-16777216..16777215),

bdsA1-r16		INTEGER (-2097152..2097151),

bdsA2-r16		INTEGER (-1024..1023),

bdsTgdB1Cp-r16	INTEGER (-2048..2047),

bdsIscB1Cd-r16	INTEGER (-2048..2047),

...,

[[	bdsTgdB2ap-r17	INTEGER (-2048..2047)				OPTIONAL,  -- Need ON

bdsIscB2ad-r17	INTEGER (-2048..2047)				OPTIONAL   -- Need ON

]],

[[	bdsTgdB2bI-r19	INTEGER (-2048..2047)				OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

BDS-ClockModel2 field descriptions

bdsTgdB1Cp Parameter TGDB1Cp Group delay differential of the B1C pilot component (seconds), see [39], 7.6.1 and [49], 7.6.1. This mandatory field is ignored by the device, when corresponding optional parameters indicated by B2a, B2b signals are present. Scale factor is 2-34 seconds.

bdsIscB1Cd Parameter ISCB1Cd Group delay differential between the B1C data and pilot components (seconds), see [39], 7.6.1. This mandatory field is ignored by the device, when corresponding optional parameters indicated by B2a, B2b signals are present. Scale factor is 2-34 seconds.

The next change

#### –	NavModel-BDS-KeplerianSet2

The IE NavModel-BDS-KeplerianSet2 is used for BDS B1C, BDS B2a and BDS B2b defined in [39], [49], [XX].

-- ASN1START

NavModel-BDS-KeplerianSet2-r16 ::= SEQUENCE {

bdsIODE-r16			INTEGER (0..255),

bdsToe-r16					INTEGER (0..2047),

bdsDeltaA-r16			INTEGER (-33554432..33554431),

bdsAdot-r16					INTEGER (-16777216..16777216),

bdsDeltaN0-r16			INTEGER (-65536..65535),

bdsDeltaN0dot-r16		INTEGER (-4194304..4194303),

bdsM0-r16					INTEGER (-4294967296..4294967295),

bdsE-r16					INTEGER (0..8589934591),

bdsOmega-r16			INTEGER (-4294967296..4294967295),

bdsOmega0-r16			INTEGER (-4294967296..4294967295),

bdsI0-r16					INTEGER (-4294967296..4294967295),

bdsOmegaDot-r16		INTEGER (-262144..262143),

bdsI0Dot-r16			INTEGER (-16384..16383),

bdsCuc-r16				INTEGER (-1048576..1048575),

bdsCus-r16				INTEGER (-1048576..1048575),

bdsCrc-r16				INTEGER (-8388608..8388607),

bdsCrs-r16				INTEGER (-8388608..8388607),

bdsCic-r16				INTEGER (-32768..32767),

bdsCis-r16				INTEGER (-32768..32767),

...

}

-- ASN1STOP

NavModel-BDS-KeplerianSet2 field descriptions

bdsIODE Parameter, Issue Of Data, Ephemeris (IODE), see [39], 7.4.1 and [49], 7.4.1. This field is not applicable for BDS B2b.

The ignoring of legacy fields is clarified as suggested in the text proposal.

# Conclusion

In the previous sections we made the following observations:

Observation 1	It is unclear as why the legacy field would be ignored when clock and navigation model are being provided from the new signal B2b.  The reason should be clarified.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The ignoring of legacy fields is clarified as suggested in the text proposal.

# References

BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2b (Version 1.0) , China Satellite Navigation Office, 2020-07

RP-242413, New WID on Introduction of BDS B2b Signal in A-GNSS, CAICT, CATT, 2024-09