3GPP TSG-RAN WG2 Meeting #129	R2-2500982

# Athens, Greece, 17th – 21st	February 2025

Agenda Item:	8.7.4.2

Source:	Ericsson

Title:	DSR enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to DSR:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In RAN2#127bis the enhanced DSR solution was further clarified that the buffered data is divided into multiple portions where the DSR indicates a buffer size and a shortest remaining time for each portion that has a buffer size larger than zero. Additionally, one bit will indicate if a further portion is reported for and no PSI information will be included in the DSR. Remaining topics to be agreed is the details on the DSR format, the triggering threshold configuration, how to handle inclusion of mixed data and the relation to Rel18 DSR. This paper will discuss these remaining aspects.

# 2 Discussion on DSR enhancements

## 2.1 New Rel19 DSR format

It was agreed in previous meetings that there can be multiple reporting thresholds configured for each LCG. For different traffic requirements and scheduler implementations there may be varying usefulness in the granularity of the reported information. It should therefore be up to network decision how many thresholds that are used for each LCG.

Network configures a number of reporting thresholds for each LCG.

It was also agreed that not all thresholds will be reported every time and that a bit in the DSR format will be used to convey if further thresholds for the LCG is present in the report. A straightforward and simple solution, that also doesn’t require any change to the structure introduced in the Rel18 DSR format, is to reuse the Rel18 reserved bit for this purpose. How this solution can work is shown in Figure 1. In this solution the n-bit has replaced the unused R-bit and is now used to indicate if a further threshold is being reported for the current LCG. In #127bis questions were raised if this solution would work to skip thresholds but as is shown in the example in Figure 1 that can be done easily. Since the remaining time field already provides an implicit indication for which threshold the information is being reported for (the remaining time value can only be between the upper and lower threshold values) there is no ambiguity in the reporting. For each LCG this solution enables to start at any threshold, jump over any threshold and report for any number of thresholds. If only one threshold is being reported for each LCG then the size of the MAC CE is not larger than the Rel18 DSR.

Figure 1: Mapping of data from the LCGs, with different remaining times and buffer sizes, to a new DSR format using multiple thresholds and a n-bit indicating if a further threshold is present for the current LCG.

For Rel19 DSR reuse the structure of the Rel18 DSR but change the R-bit to a n-bit to indicate if a further information pair is present for the current LCG.

Adopt the DSR format shown in Figure 1.

## 2.2 Reporting of data volume in the Rel19 DSR

### 2.2.1 Where to capture non-delay critical data

At RAN2#128 it was agreed that based on capability the UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for the enhanced DSR. An example how this is expected to work is exemplified in Figure 2.

Figure 2: UE that will transmit non-delay critical data before the delay critical data multiplex the size of the non-delay critical data into the buffer size of the delay critical data in the DSR.

There was a discussion on where to add the specification text for this enhanced calculation of the delay critical data for Rel19 DSR, i.e. in the PDCP or MAC specification. To make such decision there are a few aspects to take into consideration:

- In the PDCP specification it is written that PDCP SDUs shall be transmitted “in ascending order of the COUNT value”, meaning that PDCP PDUs shall be transmitted in the order of a FIFO queue. In the MAC specification there is nothing written about in which order packets shall be transmitted.

- Since delay critical and non-delay critical packets can be interleaved, it is possible that a number of non-delay critical packets need to be transmitted before delay critical packets with a certain remaining time. Therefore in each reporting threshold interval both delay critical packets, with a remaining time between the upper and lower threshold, need to be reported as well as non-delay critical packets that are ahead of the delay-critical packets (or non-delay critical packets that are interleaved with delay critical packets within a certain threshold interval). This is tightly connected to the identification of packets in the current PDCP specification for delay critical buffer calculation.

- Current PDCP data volume calculation anyway need to be updated to support changes in Rel19 DSR, i.e. the multi-threshold solution.

These aspects indicate that the handling of non-delay critical data should at least be captured in the PDCP specification. A straightforward approach for this is to add a new section on the calculation of the delay critical data for the Rel19 DSR, i.e. 5.15a Data volume calculation for multi-threshold delay reporting.

Multiple aspects point towards that the change in Rel19 DSR buffer size calculation should be captured in the PDCP specification.

Add a new section “5.15a Data volume calculation for multi-threshold delay reporting” in the PDCP specification.

### 2.2.2 New section for data volume calculation in PDCP specification

There are several changes to the new data volume calculation for Rel19 DSR. To support them, new parameters need to be defined. To start with thresholds for remaining time need to be in a list that is configured in RRC and used in PDCP and MAC. For that list there is also a need to configure the thresholds and agree on a maximum number of thresholds. These parameters could be called remainingTimeThresholdList and maxNumberOfRemainingTimeThresholds.

Call the list of remaining time thresholds remainingTimeThresholdList and the maximum number of thresholds maxNumberOfRemainingTimeThresholds.

The number of intervals in where to count delay critical and non-delay critical packets will be the same as the number of remaining time thresholds, where the first interval will go from 0 to the lowest value of remaining time thresholds. The elements in the list should be sorted in order according to their value, with the lowest numbers first.

The list remainingTimeThresholdList should be a list where the values in the list are sorted according to their value with the lowest numbers first.

In order to have a common understanding of the remaining time threshold intervals there is a need of a definition. Using the names above, intervals could be defined from one to maxNumberOfRemainingTimeThresholds, where interval i is the interval where remaining time is less than remainingTimeThresholdList[i] and larger or equal than remainingTimeThresholdList[i-1].

Define the remaining time interval i as is the interval where for i>1 remaining time is less than remainingTimeThresholdList[i] and larger or equal than remainingTimeThresholdList[i-1] and for i=1 the remaining time interval is less than remainingTimeThresholdList[1] and larger than 0.

In PDCP specification today there is a definition for delay critical PDCP SDUs in the definitions section. To be aligned with this approach, definitions for both delay critical PDCP SDUs and non-delay critical PDCP SDUs should be added in PDCP definitions section. For the delay critical PDCP SDUs the current definition states:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

This can be partly reused for defining delay critical PDCP SDUs in a specific interval:

Delay-critical PDCP SDU in interval i: if pdu-SetDiscard is not configured and i>1, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThresholdList[i] and larger or equal than remainingTimeThresholdList[i-1]. If pdu-SetDiscard is not configured and i=1, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThresholdList[1] and larger than 0. If pdu-SetDiscard is configured and i>1, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThresholdList[i] and larger or equal than remainingTimeThresholdList[i-1]. If pdu-SetDiscard is configured and i=1, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThresholdList[1] and larger than 0.

Adopt the text above as a definition for delay critical PDCP SDU in a specific remaining time threshold interval.

If the UE is capable of it the data volume that shall be reported for a certain interval shall contain both delay critical and non-delay critical packets. Also the Rel-18 DSR need to include PDCP control PDUs, AM DRBs (PDCP SDUs and PDCP PDUs to be retransmitted, as defined in the section 5.15 of the PDCP specification). The definition of the total data volume calculation in a specific interval the definition will not be as straight forward, considering that delay critical and non-delay critical packets may be interleaved in the buffers. The simplest solution is to refer to the order in which the packets will be transmitted. The total number of packets in interval i would then be the number of packets that will be transmitted when the last delay critical packet in interval i have been transmitted, but after all delay critical packets in interval i-1 have been transmitted.

Since there is no interval 0 there is a need of a different definition for interval 1: total number of packets in interval 1 would then be the number of packets that will be transmitted when the last delay critical packet in interval 1 have been transmitted.

Rel19 data volume calculation can include non-delay critical data based on UE capability in a multi-threshold data volume calculation.

In the multi-threshold data volume calculation, the total number of packets in interval i, where i>1, are defined as the number of packets that will be transmitted when the last delay critical packet in interval i have been transmitted, but after all delay critical packets in interval i-1 have been transmitted.

In the multi-threshold data volume calculation, the total number of packets in interval 1 are defined as the number of packets that will be transmitted when the last delay critical packet in interval 1 have been transmitted.

Instead if the UE capability only allows reporting delay critical data volume the calculation will be slightly different. Again there is a need to include PDCP control PDUs, AM DRBs (PDCP SDUs and PDCP PDUs to be retransmitted, as defined in the section 5.15 of the PDCP specification). In the PDCP specification it is not defined where in the buffer control PDUs and AM DRBs for retransmission should be, even though they should have high priority. Therefore it can be assumed they can be interleaved anywhere in the buffer.

Control PDUs and AM DRBs for retransmission can be interleaved anywhere in the buffer.

Also this case there can be a reference to the order in which the packets will be transmitted, but only delay critical SDUs, control PDUs and AM DRBs for retransmissions should be counted. The definition of the calculation of multi-threshold data volume will thus be a bit more complex.

In the multi-threshold delay critical data volume calculation the number of packets to be counted in interval i, where i>1, are defined as the number of delay critical packets, the number of control PDUs and the number of AM DRBs for retransmission that will be transmitted when the last delay critical packet in interval i have been transmitted, but after all delay critical packets in interval i-1 have been transmitted.

In the multi-threshold delay critical data volume calculation the number of packets to be counted in interval 1 are defined as the number of delay critical packets, the number of control PDUs and the number of AM DRBs for retransmission that will be transmitted when the last delay critical packet in interval 1 have been transmitted.

### 2.2.3 Change to MAC specification

In addition to the changes to PDCP specification also the MAC specification can be updated to capture the handling of the non-delay critical data. For Rel-18 triggering and creation of the MAC CE is described in section 5.4.9. Since multi-threshold DSR will be different in many ways from the Rel-18 DSR, a new section for multi-threshold DSR is needed in the MAC specification.

Create a new section in the MAC specification for enhanced DSR called “5.4.9a Multi-threshold delay status report”.

In addition to the reporting thresholds, a triggering threshold needs to be configured that can be used by the MAC layer. Such a parameter could be called e.g. delayStatusReportingTrigger.

Create a new network configured parameter for triggering the enhanced delay status report, called delayStatusReportingTrigger.

Current MAC specification states the following on the buffer size of the DSR MAC CE:

- 	Buffer Size: The Buffer Size field indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 and clause 5.6 in TS 38.323 for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBSR-TableAllowed and the amount of delay-critical UL data for an LCG is within the buffer sizes specified in Table 6.1.3.1-x, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-x to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits.

Instead of referring only to delay critical UL data for an LCG the MAC specification for the Rel19 DSR MAC CE could refer to all the data needed to transmit the delay critical data (following the same idea as for the changes to the Data volume calculation for multi threshold delay reporting in PDCP).

-	Buffer Sizei: The Buffer Size field i indicates the total amount of UL data that the UE needs to transmit for an LCG in order to have sent all data in the buffer for that LCG that has a remaining time i or lower, according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBS-TableAllowed and the amount of delay-critical UL data for an LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits.

MAC specification to be updated to capture that inside a time threshold interval the buffer size includes all the data the UE needs to transmit in order to have sent all the delay critical data in that remaining time interval.

## 2.3 Triggering of the DSR

It was also agreed in RAN2 #128 that network can configure the triggering and reporting thresholds without constraints. An example how this work with one trigger threshold, configured at 26ms remaining time, and one reporting thresholds, configured at 12 ms remaining time, is shown in Figure 4.

Figure 3: Example of a DSR configuration with multiple reporting thresholds. PDU Set Delay Budget (PSDB) is 30ms and triggering is based on the largest threshold (Threshold 1 = 26ms) while the second threshold is only used for reporting with higher granularity (Threshold 2 = 12ms).

## 2.4 Relation to Rel18 DSR

It has been suggested that the UE needs to be able to use a mix of Rel18 DSR and Rel19 DSR at the same time. The main claim being that the Rel19 DSR may be too large to fit in the UL grant. However, with the format proposed in this paper the size of the Rel19 DSR can be kept well under control. Furthermore, when a DSR is triggered it will have higher priority than data and thus no problem to fit in the grant. Generally, for all buffer reporting the only case when data is added to the grant before reports is when an opportunistic report is sent using any left-over padding bits. This is not the case with the DSR, as no padding DSR has been agreed. No truncated DSR was agreed in Rel18 and it is not foreseen that such is needed for Rel19 either.

There is no need to have both Rel18 and Rel19 DSR configured at the same time for one UE.

Network does not configure the Rel18 and the Rel19 DSR at the same time for one UE.

Nevertheless, if mixing the Rel19 with Rel18 DSR is really wanted by RAN2 any scheduler impacts need to be carefully considered. The main scheduler problem relates to how many thresholds that is configured. The concern with mixing reporting using both single and multiple thresholds is that it will create ambiguity for the network scheduler, i.e. mixing high granularity and low granularity reports. This needs to be avoided. However as long as the LCGs in the report only has one threshold configured, or data in only one threshold, it doesn’t matter which format that is used, as the granularity of the formats will be the same. Thus there need to be a restriction of reporting a mix of formats that is depending on the number of thresholds configured and data spread, i.e. any LCG that has configured multiple thresholds and data in more than one threshold will only use the Rel19 format.

If an LCG has multiple reporting thresholds configured using Rel19 DSR together with Rel18 DSR can create large problems of ambiguity to the network scheduler.

If mixing usage of Rel18 and Rel19 DSR, the Rel18 DSR format should not be used to report for a LCG that is configured with multiple reporting thresholds and has data in more than one threshold.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Multiple aspects point towards that the change in Rel19 DSR buffer size calculation should be captured in the PDCP specification.

Observation 2	Control PDUs and AM DRBs for retransmission can be interleaved anywhere in the buffer.

Observation 3	There is no need to have both Rel18 and Rel19 DSR configured at the same time for one UE.

Observation 4	If an LCG has multiple reporting thresholds configured using Rel19 DSR together with Rel18 DSR can create large problems of ambiguity to the network scheduler.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Network configures a number of reporting thresholds for each LCG.

Proposal 2	For Rel19 DSR reuse the structure of the Rel18 DSR but change the R-bit to a n-bit to indicate if a further information pair is present for the current LCG.

Proposal 3	Adopt the DSR format shown in Figure 1.

Proposal 4	Add a new section “5.15a Data volume calculation for multi-threshold delay reporting” in the PDCP specification.

Proposal 5	Call the list of remaining time thresholds remainingTimeThresholdList and the maximum number of thresholds maxNumberOfRemainingTimeThresholds.

Proposal 6	The list remainingTimeThresholdList should be a list where the values in the list are sorted according to their value with the lowest numbers first.

Proposal 7	Define the remaining time interval i as is the interval where for i>1 remaining time is less than remainingTimeThresholdList[i] and larger or equal than remainingTimeThresholdList[i-1] and for i=1 the remaining time interval is less than remainingTimeThresholdList[1] and larger than 0.

Proposal 8	Adopt the text above as a definition for delay critical PDCP SDU in a specific remaining time threshold interval.

Proposal 9	Rel19 data volume calculation can include non-delay critical data based on UE capability in a multi-threshold data volume calculation.

Proposal 10	In the multi-threshold data volume calculation, the total number of packets in interval i, where i>1, are defined as the number of packets that will be transmitted when the last delay critical packet in interval i have been transmitted, but after all delay critical packets in interval i-1 have been transmitted.

Proposal 11	In the multi-threshold data volume calculation, the total number of packets in interval 1 are defined as the number of packets that will be transmitted when the last delay critical packet in interval 1 have been transmitted.

Proposal 12	In the multi-threshold delay critical data volume calculation the number of packets to be counted in interval i, where i>1, are defined as the number of delay critical packets, the number of control PDUs and the number of AM DRBs for retransmission that will be transmitted when the last delay critical packet in interval i have been transmitted, but after all delay critical packets in interval i-1 have been transmitted.

Proposal 13	In the multi-threshold delay critical data volume calculation the number of packets to be counted in interval 1 are defined as the number of delay critical packets, the number of control PDUs and the number of AM DRBs for retransmission that will be tranmitted when the last delay critical packet in interval 1 have been transmitted.

Proposal 14	Create a new section in the MAC specification for enhanced DSR called “5.4.9a Multi-threshold delay status report”.

Proposal 15	Create a new network configured parameter for triggering the enhanced delay status report, called delayStatusReportingTrigger.

Proposal 16	MAC specification to be updated to capture that inside a time threshold interval the buffer size includes all the data the UE needs to transmit in order to have sent all the delay critical data in that remaining time interval.

Proposal 17	Network does not configure the Rel18 and the Rel19 DSR at the same time for one UE.

Proposal 18	If mixing usage of Rel18 and Rel19 DSR, the Rel18 DSR format should not be used to report for a LCG that is configured with multiple reporting thresholds and has data in more than one threshold.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024