3GPP TSG-RAN WG2 #129	R2-2500989

# Athens, Greece, 17th – 21st February 2025

Agenda Item:	7.2

Source:	Ericsson, MediaTek Inc.

Title:	Discussion on the fast RRC processing for LTM

Document for:	Discussion

# 1	Introduction

RAN2 has received a reply LS in [1] about the questions asked about the fast RRC processing capability for LTM. This document is to address the implications of the RAN4 reply for RAN2.

# 2	Discussion

According to the reply received by RAN4 it has been confirmed that as far as network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, the UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations. Therefore, it would be good for RAN2 to support this understanding:

RAN2 confirms that for the fast RRC processing capability for LTM, as far as network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, the UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations.

A further aspect that was asked to RAN4 has been whether it was possible for the network to configure a number of serving cell(s) + SpCell/SCell(s) within the LTM candidate configurations that can exceeds the UE reported maxNumberStoredConfigCells (which is one of the component of the fast RRC processing capability). The reply from RAN4 was affirmative and the main motivation is that when the number of total stored cell is exceeded, this does not mean that the UE cannot operate anymore, but rather that the UE will process the LTM candidate cell with an additional delay, which is the delay that is normally added when no fast RRC processing on the LTM candidate configurations is done. According to this, it would be good to clarify this aspects and thus we propose:

RAN2 confirms that it is possible for the network to configure a number of serving cell(s) + SpCell/SCell(s) within the LTM candidate configurations that can exceeds the UE reported maxNumberStoredConfigCells (which is one of the component of the fast RRC processing capability).

If the network configures a number of serving cell(s) + SpCell/SCell(s) within the LTM candidate configurations that exceeds the UE reported maxNumberStoredConfigCells, this means that UE will process the configured LTM candidate configuration without performing any fast RRC processing.

The last question it was asked to RAN4 it was whether the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration. The reply we got from RAN4 is the following:

RAN4: No. RAN4 has defined a specific rule for the feature of ltm-FastProcessingConfig-r18 to ensure that both UE and NW can be in sync regarding the early processing of the target LTM configuration. That said, the group finds it beneficial to provide such auxiliary information for efficient UE processing.

This means that even this information is not necessary, it would be beneficial for the UE to know such information as it could make the UE processing of the LTM candidate configurations (whether fast RRC processing is done or not) more efficient. Since the specification effort in RAN2 to add this information is minimal (just a new field within the LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration) we see no issue to support this. Therefore, we propose:

A new field is added within the LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration.

RAN2 to agree on the CR provided in the Annex.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 confirms that for the fast RRC processing capability for LTM, as far as network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, the UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations.

Proposal 2	RAN2 confirms that it is possible for the network to configure a number of serving cell(s) + SpCell/SCell(s) within the LTM candidate configurations that can exceeds the UE reported maxNumberStoredConfigCells (which is one of the component of the fast RRC processing capability).

Proposal 3	If the network configures a number of serving cell(s) + SpCell/SCell(s) within the LTM candidate configurations that exceeds the UE reported maxNumberStoredConfigCells, this means that UE will process the configured LTM candidate configuration without performing any fast RRC processing.

Proposal 4	A new field is added within the LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration.

Proposal 5	RAN2 to agree on the CR provided in the Annex.

# References

R4-2420218, Reply LS on the fast RRC processing for LTM, RAN4, RAN4#113 meeting, US, November 2024.

# Annex (TP to 38.331)

3GPP TSG-RAN WG2 #129	R2-25xxxxx

# Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | XXXX | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of additional information for fast RRC processing for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the reply LS received from RAN4 in R4-2420218 it would be beneficial for the UE to know such information as it could make the UE processing of the LTM candidate configurations (whether fast RRC processing is done or not) more efficient. Since the specification effort in RAN2 to add this information is minimal (just a new field within the LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration) we see no issue to support this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 6.3.2 - A new field is added within LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration Impact Analysis Impacted 5G architecture options: NR SA and NR-DC Impacted functionality: Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may start to process the LTM candidate configuration in advance (fast RRC processing) and figures out during the process that fast RRC processing on the configured LTM candidate configurations is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 6.3.2	Radio resource control information elements

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...,

[[

ltm-NrOfServingCells-r18                       INTEGER (1..maxNrofServingCells)                      OPTIONAL     -- Need M

]]

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.

ltm-NrOfServingCells If ltm-ConfigComplete is configured, this field indicates the number of serving cells (SpCell and SCell(s)) configured in ltm-CandidateConfig. Otherwise, this field indicates the total number of serving cells (SpCell and SCell(s)) configured in ltm-ReferenceConfiguration and ltm-CandidateConfig.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.

END OF CHANGES