3GPP TSG-RAN WG2 Meeting #129	R2-2501007

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.4.1

Source:	ITRI

Title:	Discussion on enhanced LCP for XR

Document for:	Discussion and Decision

1 Introduction

In RAN2 #128, some agreements of enhanced LCP had been reached as below.

  - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round.

  - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

There are still some issues needed to be discussed, such as the impact on SRB and intra-UE prioritization, the impact on PBR/Bj, and one or multiple additional priorities of the additional priority configuration. In this document, we proposed that the SRB impact can be avoided by a proper network implementation, and the additional LCH priority should also apply to intra-UE prioritization procedure. However, due to the high complexity and unconfirmed evaluation, the Bj adaptation is not needed and one additional priority per LCH is preferred for Rel-19.

2 Discussion

2.1 Impact on SRB and intra-UE prioritization

From our point of view, a proper network implementation is enough to avoid the SRB impact. That means the network can configure the LCH of SRB with a higher LCH priority than the additional LCH priority used for the LCH with LCH priority-adjusted data.

Proposal 1: A proper network implementation can avoid SRB impact caused by LCH prioritization-based LCP enhancement.

Following is the related description of intra-UE prioritization in TS 38.321, “For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2.” We think that if an LCH is adjusted to additional LCH priority due to LCH priority-adjusted data, it is straightforward to use the additional LCH priority as the current LCH priority in intra-UE prioritization procedure, so the LCH priority-adjusted data can be transmitted as soon as possible with a higher probability.

Proposal 2: The additional LCH priority also applies to intra-UE prioritization procedure.

2.2 Impact on PBR/Bj

Some companies proposed the Bj adaptation for delay-critical data. We analysed three options and argued the trade-off between the cost of Bj adaptation and the fairness of token bucket algorithm in the LCP procedure.

Option 1 is that the LCH with LCH priority-adjusted data is not restricted by PBR in the first round of resource allocation procedure. However, from our point of view, it would have more negative impacts on fairness caused by infinite PBR than the fairness issue caused by using additional LCH priority for LCHs with LCH priority-adjusted data because the possibility that important data expires due to the transmission of LCH priority-adjusted data will become severe. To reduce the negative impacts caused by infinite PBR, temporarily allowing UE to raise the rate limit (e.g. Bj) for delay-critical LCH is suggested to be considered. However, an additional cross-layer interaction may make it difficult for a UE to dynamically determine a proper value to raise Bj (e.g., data calculation, multiplexing and assembly.) Option 3 proposed that a delay-aware LCP solution uses a new counter variable to keep track of the scheduling of the LCH priority-adjusted data. We wonder the necessity to design a complicated solution (e.g., negotiation between the new counter and default counter) for the LCH priority-adjusted data. Therefore, since the Bj adaptation breaks the fairness of token bucket algorithm in the LCP procedure and the capacity improvement has not been confirmed, we propose not to consider the Bj adaptation for LCH priority-adjusted data for Rel-19.

Proposal 3: Not to consider the Bj adaptation for LCH priority-adjusted data for Rel-19.

2.3 One or multiple additional priorities of the additional priority configuration

We understand that there is an optimized design for multiple additional priorities of the additional priority configuration. However, it is difficult to evaluate the capacity performance if UE determines the additional priority for each LCH itself. For example, the same question will be raised that whether it has negative impact on the fairness and causes more loss of important data, just as we had been troubled when discussing the Bj adaptation. Therefore, we think that the optimization is not needed due to the complexity. Only one additional priority per LCH is preferred for Rel-19.

Proposal 4: For simplicity, only one additional priority per LCH is preferred for Rel-19.

3 Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: A proper network implementation can avoid SRB impact caused by LCH prioritization-based LCP enhancement.

Proposal 2: The additional LCH priority also applies to intra-UE prioritization procedure.

Proposal 3: Not to consider the Bj adaptation for LCH priority-adjusted data for Rel-19.

Proposal 4: For simplicity, only one additional priority per LCH is preferred for Rel-19.

4 References

- R2_128 R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM.docx