3GPP TSG RAN WG2 Meeting #129	R2-2501033

Athens, GR, Feb. 17 – 21, 2025

Agenda Item:	8.8.4

Source:	CMCC

Title:	Considerations on broadcast service for NR NTN

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In this contribution, we would like to provide some further considerations on the remaining issues for broadcast service in NTN system.

Discussion

Pointer for intended service area

- In last meeting, we agreed to introduce a new SIB to include a list of intended service areas and related pointer and specific format of the pointer is FFS. As known, for TN coverage, we introduced tn-AreaId-r18 which is related to a group of TN cells used for NTN-TN recelection, and in SIB4/5 indicates the specific TN coverage based on tn-AreaIdList-r18. However, for MBS intended service areas, we have agreed to to include a list of intended service areas, to be simple, we could just use the index in the service area list in anywhere requiring service area information.

Then to support both the circle and polygon solutions, a “CHOICE” signaling structure could be considered. And based on NW configuration to provide the intended service area information

Proposal 1: For the service area pointer, we could just use the index in the service area list in anywhere requiring service area information.

Proposal 2: It is proposed to use “CHOICE” signaling structure to support both the service area format solutions.

MCCH acquisition relaxation

To facilitate MCCH acquisition relaxation, a solution that provide the ISA ID in the MBS Service Announcement is discussed and no agreement is achieved.

Proposal 3 If ISA is sent in SIB, the ISA ID is provided in the MBS Service Announcement to facilitate MCCH acquisition relaxation. RAN2 sends an LS to SA2. - Apple wonders if this means that ISA ID should become a globally unique identifier - ZTE sees some benefit and is fine to further discuss this - CATT thinks it’s late to add more work to SA2/CT1 for this - Samsung thinks we can address the issue to avoid reading MCCH when not needed directly in RAN2 without involving other groups - Apple/Xiaomi think this would require coordination between RAN and CN and then involve other groups. Ericsson thinks the coordination will be there Continue the discussion in the next meeting

From our perspective, based on the WID scope, the enhancements for broadcast service for NTN only involve RAN work. Further MBS service announcement solution would have impact on SA2, while they only has limited remaining time for R19, which means SA2 is most likely running out of time to discuss and solve the issue. Therefore, facilitate MCCH acquisition relaxation based on MBS service announcement solution should not be pursued in R19.

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

Observation 1: Enhancements for broadcast service for NTN only involve RAN work based on the WID scope.

Observation 2: SA2 has little time in R19 to discuss the MCCH acquisition relaxation solution.

Proposal 3: Not pursue the solution that provide the ISA ID in the MBS Service Announcement to facilitate MCCH acquisition relaxation.

- Then for the RAN solution to allow UE skipping MCCH acquisition when UE is not within intended service area of any interested broadcast service. Per the current TS 38.331, it is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. Similarly, for skipping MCCH acquisition in NTN, it could be also up to UE implementation, when UE is not within intended service area of any interested broadcast service, it could relax MCCH acquisition based on the its own location and intended service areas provision by network.

- Proposal 4: When UE is not within intended service area of any interested broadcast service, it could be also up to UE implementation to allow UE skipping MCCH acquisition based on the intended service areas provision by network and its own location.

Service continuity

In RAN2 #127bis meeting, we have discussed on service continuity as following:

Proposal 4: Service continuity can be enhanced in two places by considering intended service area. • In MBSBroadcastConfiguration, the intended service area within each neighbor cell is provided for each MBS broadcast service session. • In SIB21, the intended service area is provided for each MBS broadcast service session, or for each FSAI (depending on common understanding selected). - Ericsson thinks we should focus on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement - ZTE thinks we currently support both and then supports p4 - QC also thinks both scenarios should be supported. - HW thinks this covers the case of UE movement. For the satellite movement there would be no difference wrt previous releases. HW is ok to reflect the second part of p4 - Ericsson disagrees with the proposal as there is no need for service continuity for this. Apple thinks this is needed for cell reselection. - CATT wonders whether we really need to provide the service area for service continuity, in a per neighbour cell way. - Ericsson thinks we need to clarify the UE behavior when receiving this information Come back in the next meeting

From our perspective, providing service area information for each MBS broadcast session of neighbour cells has some benefit for reselection and handover, it could help UE to select a more suitable cell to continue receiving the same broadcast services. As known, MCCH may also contain a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell. In order to guarantee service continuity better, providing service area information per neighbour cell in the MBSBroadcastConfiguration could help UE to reselect or handover to a neighbour cell including service areas association with the MBS session UE interested in or providing same broadcast service which UE is receiving currently.

Then SIB21 provides information related to service continuity, which contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). However, considering the different understanding about the relation between FSA and MBS session (i.e. one to one, one to multiple), to enhance service continuity, we could just consider to introduce intended service area for each MBS broadcast service session in SIB21.

Proposal 5: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 6: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

MBS interest indication

We discussed that whether a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area without convergence.

As current specification, the MBS interest indication procedure is to inform the network that the UE in RRC_CONNECTED is receiving or is interested to receive MBS broadcast service(s) and/or to inform the network about the priority of MBS broadcast versus unicast and multicast MRB reception. MBS Interest Indication can only be sent after AS security activation. And the trigger condition is described in section 5.9.4.2 of TS 38.331.

- 5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

In the contribution [2], it is mentioned that initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area could help NW to choose a suitable cell providing the broadcast service for this UE. However, per the exiting mechanism, UE already supports to initiate the MII procedure upon handover. And in NTN, UE is able to determine whether in the intended service area based its own GNSS and service area information provided by network. Further, network may have no acknowledge of the UE location, i.e., UE is in which service area may not be clear at network side. Then whether initiate the MII procedure could be just up to UE itself. Therefore, further enhancement for MBS Interest Indication initiation is not needed.

Proposal 7: It is proposed not to pursue that a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following observations and proposals:

Proposal 1: For the service area pointer, we could just use the index in the service area list in anywhere requiring service area information.

Proposal 2: It is proposed to use “CHOICE” signaling structure to support both the service area format solutions.

Observation 1: Enhancements for broadcast service for NTN only involve RAN work based on the WID scope.

Observation 2: SA2 has little time in R19 to discuss the MCCH acquisition relaxation solution.

Proposal 3: Not pursue the solution that provide the ISA ID in the MBS Service Announcement to facilitate MCCH acquisition relaxation.

- Proposal 4: When UE is not within intended service area of any interested broadcast service, it could be also up to UE implementation to allow UE skipping MCCH acquisition based on the intended service areas provision by network and its own location.

Proposal 5: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 6: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

Proposal 7: It is proposed not to pursue that a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area.

Reference

- R2-128 NR-NTN-IoT-NTN (Sergio)_ EOM