3GPP TSG-RAN WG2 Meeting #129       	                      R2-2501052

Athens, GR, 17th-21st February 2025

Agenda item:	8.3.2.2

Source:	CMCC

Title:	Discussion on model generalization

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed the model generalization for RRM prediction and achieved the following agreements.

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. 2 Companies can choose which case they compare with and should report it with simulation results. 3 Generalization issues on RRM measurement prediction are prioritized. 4 Start the study with generalization issue with RRM measurement prediction in temporal domain. Companies can chose to study frequency domain prediction cases and report what they have simulated. 5 Study generalization over UE speeds 6 The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h. FFS on combinations 7 The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h. FFS on combinations

1 Between GC#1 and GC#2, RAN2 focus on GC#2 2 simulation combination for FR1 generalization study: Training @Dataset: 30km/h; Training @Dataset: 60km/h; Training @Dataset: 90km/h; Inference @30km/h; Inference @60km/h; Inference @90km/h. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. 2a simulation combination for FR2 generalization study: Training @Dataset: 60km/h; Training @Dataset: 90km/h; Training @Dataset: 120km/h; Inference @60km/h; Inference @90km/h; Inference @120km/h. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes.

In this contribution, we will provide our simulation results on model generalization and observations.

- Discussion

- 2.1 Simulation results on model generalization

In our simulation, we use RRM sub case 2 as starting point. The measurements of L3 cell level RSRP are used as AI/ML model input to predict L3 cell level RSRP results.

Figure 1: RRM sub case 2

For model generalization, we focus on FR2 generalization study. The key simulation assumptions are shown in Table 1.

Table 1: The key simulation assumptions

| Report parameters |  | CMCC |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | UE speed (30,60,90,120 Km/h) | 90 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |
|  | Whether LOSsoft is modeled or not | Yes |
|  | spatial consistency option (A or B) | Procedure A |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | 240ms |
|  | Prediction window (Note3) | 80ms |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | ≈116K/29K |
|  | Testing | ≈29K |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |
|  | Model output(Note 6) | Cell level RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 780,150 |
|  | Model complexity in model size (e.g. Mbyte) | 8.963MB |
|  | Computational complexity [FLOPs] | 2138.85K |

In detail, we take UE speed 90km/h as baseline, i.e. inference dataset is for 90km/h, and both GC#1 and GC#2 are considered.

For GC#1-1, the dataset for 60km/h is used for model training, and for GC#1-2, the dataset for 120km/h is used for model training. For GC#2, the combination dataset for 60/90/120km/h is used for model training, where the dataset size for each speed is the same, accounting one third respectively.

The simulation results for model generalization are shown in Table 2.

Table 2: The simulation results on FR2 generalization

| Case | Training dataset(Km/h) | Inference dataset (Km/h) | RSRP_differ(dB) |
| --- | --- | --- | --- |
| Baseline | 90 | 90 | 0.538 |
| GC#1-1 | 60 | 90 | 0.554 |
| GC#1-2 | 120 | 90 | 0.540 |
| GC#2 | 60，90，120 | 90 | 0.551 |

Based on the above simulation results, it could be observed that the RSRP differences in each case are very close. That means the model for FR2 intra-frequency time domain prediction has good generalization ability on UE speed.

Observation 1: The model for FR2 intra-frequency time domain prediction has good generalization ability on UE speed, since the simulation results in each case are very close.

Furthermore, we observe that the RSRP difference for case GC#1-1 is larger than which for GC#1-2, i.e.  GC#1-2 shows a good model performance compared to GC#1-1. The reason may be that the model trained with dataset in high-speed can perform well with inference dataset in low-speed. However, the model trained with dataset in low-speed performs worse with inference dataset in high-speed.

Observation 2: The model trained with dataset in high-speed performs well with inference dataset in low-speed.

Observation 3: The model trained with dataset in low-speed performs not well with inference dataset in high-speed.

# 3	Conclusion

Here are the observations on model generalization.

Discussion on model generalization

Observation 1: The model for FR2 intra-frequency time domain prediction has good generalization ability on UE speed, since the simulation results in each case are very close.

Observation 2: The model trained with dataset in high-speed can perform well with inference dataset in low-speed.

Observation 3: The model trained with dataset in low-speed performs not well with inference dataset in high-speed.

# 4	Reference

- Draft_RAN2_128_Meeting_Report_v1