3GPP TSG-RAN WG2 #129 	R2-2501069

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Discussion on DL messages for Ambient IoT UEs

Document for:	Discussion/Decision

# 1	Introduction

In RAN#106, a new WI has been approved for Ambient IoT devices [1]. The following RAN2 related objectives are capture in the WID:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

In this contribution we discuss the downlink messages for A-IoT devices based on the scope highlighted above considering the outcome of the related SI as guideline.

# 2	Discussion

## 2.1 Reader awareness

In RAN2#127, it was agreed that the following information is considered useful if it is visible to the reader: the service type, e.g., inventory, command, whether the requested service is intended for one or multiple devices, and the approximate number of targeted devices. RAN2 sent an LS to SA2 in R2-2411232 and asked if it is feasible for CN to provide the following information to the reader:

-	the service type of A-IoT (e.g. inventory, command)

-	whether the service is targeted for a single or multiple devices;

-	the approximate number of target devices (if available).

In the reply LS, SA2 informed RAN2 in S2-2501241 that “CN can provide the information listed above to the reader if available”.

In RAN2, it was FFS whether such information should be mandatory or optional to provide. We think that the information on service type, and even command type, and whether service request is intended for a single device, or multiple devices should be provided to the reader mandatorily. This is so that it would be possible for the reader to configure the downlink message accordingly, e.g., contention based vs. contention free random access. However, for approximate number of targeted devices we think that it can be optional as it may not be always possible for the network to have such information.

RAN2 assumes that it is mandatory for the core network to inform the reader about the service type, command type (if the service type is a command) and whether the DL message is addressed to single or multiple devices.

RAN2 assumes that it is optional for the core network to inform the reader about the approximate number of targeted devices.

Based on the outcome of the discussion above, RAN2 should inform RAN3/SA2 which information needs to be always provided to reader, which one(s) can be optional, and ask if that would be feasible.

Send an LS to RAN3/SA2 regarding which information needs to be always provided to reader, which one(s) can be optional, and ask if that would be feasible

## 2.2 Multiple/subsequent A-IoT paging

In RAN2#127, it was agreed to support a mechanism for the reader to trigger multiple/subsequent A-IoT paging messages that are associated with the same service request from the CN. In RAN2#127bis, it was agreed that A-IoT paging message can include information to avoid duplicate response from the device to a reader. It is FFS how to indicate such information.

Should the ID be generated by the CN or the reader?

In RAN2#128, it was agreed that such information can be an ID generated either by the reader or CN, which can then be provided along with the paging request:

- Reader generated: when the reader receives a service request from the CN, it generates the ID associated with that service request and provides it in the paging/DL message. In the subsequent paging message(s) triggered by the reader, same ID is provided in the paging message.

- CN generated: ID associated with the service request is generated by the CN and provided to the reader along with that service request. The reader provides the ID in the paging message. In the subsequent paging message(s) triggered by the reader, same ID is provided in the paging message. It is also possible that the ID generated by the CN is provided in an encapsulated message provided by the CN, i.e., the ID is transparently transmitted by the reader to the device.

In this section we discuss the pros and cons for CN and reader generated IDs. If it is the CN that generates the ID, CN can coordinate so that same ID is provided to multiple readers, if the service request from the CN, e.g., inventory, is to be provided to multiple readers. This would make it possible to avoid receiving redundant responses from an A-IoT device in case it happens to be covered by multiple readers. Otherwise, i.e., if it is the reader that generates the ID, there may be a need for coordination between readers or additional information may need to be provided to identify the readers. Reader generating the ID can be beneficial since the device could determine whether to respond based on the ID provided in the paging message with no need to decode the message from the CN, if the ID is encapsulated in the message.

It may be better if the ID associated with the service request is generated by the CN and provided to the reader(s) along with that service request so that the reader(s) provide(s) the ID in the paging message. With this we can avoid coordination of ID generation between readers. We propose how the ID should be generated below and suggest sending an LS to RAN3 and SA2 to inform our preference and ask whether it is feasible for the CN to generate such ID.

RAN2 assumes that ID associated with the service request is generated by the CN and provided to reader(s) along with that service request.

Send an LS to RAN3/SA2 regarding the ID and ask whether it is feasible for CN to generate such an ID.

Device behaviour after receiving (subsequent) A-IoT paging

When an A-IoT device receives a paging message and observes that the information provided in the paging message is different from the one provided in the previous paging message, the A-IoT device performs the requested procedure regardless of whether it has completed the same procedure in the previous paging round. Otherwise, i.e., if the information provided in the paging message is the same as the one provided in the previous paging message, the device only needs to perform the requested procedure if it has not completed the procedure successfully in the previous paging round. Otherwise, it can ignore the paging message. Note that successful completion may be either due to receiving an acknowledgment explicitly or implicitly after transmitting Msg3 for 3-step contention-based RA. This is a separate discussion.

If the paging message is associated with a CN request different than the one associated with the paging message in the previous round, the device performs the requested procedure.

If paging message is associated with a CN request same as the one associated with the paging message in the previous round, the device performs the requested procedure only if it has not completed the procedure successfully in an earlier paging round associated with the same CN request.

Otherwise, the device can ignore the paging message.

## 2.3 Multiple reader scenario for A-IoT devices

Multiple readers may be deployed to serve an area, and their coverage can overlap. For example, in [1], a reader can transmit paging messages to A-IoT device(s) whose responses are received by another reader.

One may also need to consider the case where an inventory procedure started by one reader is interleaved by another inventory procedure started by another reader assuming that there is no coordination between readers in time and/or frequency domains. For example, an A-IoT device which is in the coverage of two readers as shown in Figure 1, may receive paging messages from both readers assuming that there is no coordination between the readers in time and/or frequency domains.

Figure 1 A-IoT device in the overlapping coverage.

In Figure 2, suppose Reader 1 transmits an A-IoT paging message to initiate the first paging round while Reader 2 transmits an A-IoT paging message to initiate the second paging round for the same service request before the first paging round initiated by Reader 1 is completed. Since both paging rounds are associated with the same service request, the A-IoT device should ideally only participate in one of the paging rounds. The device may dismiss the first request and participate in the second paging round initiated by Reader 2 unless it has completed the procedure for the paging round in initiated by Reader 1 successfully, or the device may ignore any other paging round received if it is participating in an ongoing paging round.

In case, the two paging rounds, that are going simultaneously, are associated with different service  requests, the device may participate in both paging rounds, provided that the device can maintain two sets of L1/L2 context, transmissions in the UL and DL do not overlap, and it is possible for the device to differentiate messages from different readers during data transmission phase (as defined in the TR).

We prefer to keep the design simple, so for the former case, i.e., same service request, the device should ignore any other paging round message received if it is participating in an ongoing paging round, and for the latter case, i.e., different service request, the device should not participate in multiple paging rounds. In short, for both cases the device should ignore any other paging round message received if it is participating in an ongoing paging round.

Device ignores any other paging message received if it is participating in an ongoing paging round.

Figure 2 A-IoT device receiving paging message from multiple readers in overlapping coverage area.

## 2.4 Indication of random-access type

In RAN2#127bis it was agreed that “From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. If multiple device IDs in single paging is supported and A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.”

A related discussion is if the reader should indicate explicitly whether a device responds to a paging message with a contention free or contention based random access. In our view there is no need to indicate such explicitly. For contention based random access, the reader only needs to indicate information that is required for a device to determine which slot to respond randomly, yet for contention free random access, the reader needs to indicate the dedicated resources for the targeted device(s). This means that the content of such paging messages will be different for contention free, and contention based random access.

From RAN2 perspective it is feasible to support paging multiple device IDs depending on TB size.

When A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.

There is no need to indicate explicitly whether a device should respond to the paging message with contention free or contention-based RA.

# 3	Conclusion

In this contribution we discussed downlink/paging messages for A-IoT devices. Based on the discussion in the previous sections we made the following observations:

Observation 1	From RAN2 perspective it is feasible to support paging multiple device IDs depending on TB size.

Observation 2	When A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.

Based on the discussion in the previous section, we propose the following:

Proposal 1	RAN2 assumes that it is mandatory for the core network to inform the reader about the service type, command type (if the service type is a command) and whether the DL message is addressed to single or multiple devices.

Proposal 2	RAN2 assumes that it is optional for the core network to inform the reader about the approximate number of targeted devices.

Proposal 3	Send an LS to RAN3/SA2 regarding which information needs to be always provided to reader, which one(s) can be optional, and ask if that would be feasible

Proposal 4	RAN2 assumes that ID associated with the service request is generated by the CN and provided to reader(s) along with that service request.

Proposal 5	Send an LS to RAN3/SA2 regarding the ID and ask whether it is feasible for CN to generate such an ID.

Proposal 6	If the paging message is associated with a CN request different than the one associated with the paging message in the previous round, the device performs the requested procedure.

Proposal 7	If paging message is associated with a CN request same as the one associated with the paging message in the previous round, the device performs the requested procedure only if it has not completed the procedure successfully in an earlier paging round associated with the same CN request.  Otherwise, the device can ignore the paging message.

Proposal 8	Device ignores any other paging message received if it is participating in an ongoing paging round.

Proposal 9	There is no need to indicate explicitly whether a device should respond to the paging message with contention free or contention-based RA.

# 4      References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

TR 22.840, V0.4.0, “Study on Ambient power-enabled Internet of Things (Release 19)”

TR 23.700-13, V1.2.1, “Study on Architecture support of Ambient power-enabled Internet of Things (release 19)”.