3GPP TSG-RAN WG2 Meeting #129	R2-2501181

Athens, Greece Feb 17th – 21st, 2025

Agenda item	:		8.8.4

Source	: 	Sharp

Title	: 	Remaining issues on intended service area

Document for	:	Discussion and Decision

Introduction

In the last meeting, RAN2 reached some agreements/decisions as follows [1];

Agreements: - The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. - The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. - The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) - A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). - RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. - Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) - The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In this paper, we’d like to discuss remaining issues on intended service area capable UE.

Discussion

[Issues on MCCH scheduling]

If information on the intended service area is to be included in the new SIB according to the agreement, it is necessary to consider how to schedule the MCCH (contains intended service area ID). The scheduling directions could be as follows.

(a) Include MCCH scheduling information in new SIB

(b) Include MCCH scheduling information in SIB20, extending existing MCCH-message

direction (a) :

This approach is reasonable in terms of separating information for intended service area capable UE and legacy UE respectively. However, in contrast to the existing specification where essentially one MCCH configuration information is transmitted for one cell, this direction results in two MCCHs being transmitted. In addition, the existing behaviour of the UE when receiving an MCCH needs to be changed, which will have some impact on the specification. For example, we need to consider which MCCH the UE needs to check when one or more MCCHs are received. Furthermore, it is necessary for the NW to select what information to be included in each MCCH, which may require coordination with the service area.

direction (b) :

In this direction, the new SIB is only used by intended service area capable UEs to know about the intended service area information. This can be achieved by combining the information contained in new SIB with the ID information contained in the MCCH received by the UE. This is also more reasonable as it has less impact on existing specifications, but it is necessary to consider whether the MCCH message content which includes intended service area IDs has any impact on the operation of the legacy UE.

Observation 1 : It is reasonable to include MCCH scheduling information in SIB20 with less impact on existing specifications.

[Issues on MCCH structure]

As mentioned above, with the introduction of direction (b), MCCH-messages that can be received and interpreted by both legacy UEs and intended service area capable UEs are required to be scheduled by SIB20. In previous meeting[2] it was agreed to include the intended service area ID in the MCCH-message, but the legacy UE cannot interpret this ID information. When considering this legacy UE, the legacy UE may receive the MBS session regardless of its associated with the intended service area if an existing SessionInfoLsit-IE is reused to send session information associated with an intended service area. In other words, if the reused MBS-SessionInfoList-r17 is sent to UE with sessions associated and not associated with the intended service area, the legacy UE may interpret all of sessions are receivable.

Observation 2 : The legacy UE cannot interpret intended service area IDs and may consider all sessions associated with intended service area as receivable.

To solve this problem, it is necessary to leave the MBS-SessionInfoList that legacy UE can understand (i.e., the list of session information not associated with the intended service area), and add information that the intended service area capable UE can understand somewhere.  However, the MBS-SessionInfoList-r17 has no extensible part, so that information has to be added outside of it.

Observation 3: There is no extensible part in MBS-SessionInfoList of current specification.

Therefore, it is more reasonable to add a new SessionInfoList associated with the intended service area to MBS BroadcastConfiguration-r17-IEs, since it is not possible to add ISA information to an existing SessionInfoList. This IE can extend lists that are not associated with ISA. The intended service area capable UE can receive the schedule of MBS session associated with intended service area correctly by checking both lists, and legacy UEs can receive their own receivable session information by checking only the legacy MBS-SessionInfoList.

Proposal 1 : The intended service area ID should be included in the IE of the MBSBroadcastConfiguration separately from the existing list of session information, together with the list of session information associated with the intended service area.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1 : It is reasonable to include MCCH scheduling information in SIB20 with less impact on existing specifications.

Observation 2 : The legacy UE may not be able to interpret intended service area IDs and may consider sessions associated with intended service area as receivable.

Observation 3: The existing specification does not allow extending MBS-SessionInfoList.

Proposal 1 : The intended service area ID should be included in the IE of the MBSBroadcastConfiguration, separately from the existing list of session information, together with the list of session information associated with the intended service area.

Reference

- Draft_RAN2_128_Meeting_Report_v1

- R2-2407902, Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China