3GPP TSG-RAN-WG2 Meeting #129	R2-2501193

Athens, Greece, 17th– 21st Feb, 2025

Agenda item	:		8.6.2

Source	: 	Sharp

Title	: 	Discussion on issues for supporting inter-CU LTM

Document for	:	Discussion and Decision

Introduction

WID [1] for mobility enhancement was approved and RAN2 started discussion on the WI. In RAN2#128, RAN2 made agreements as follows [2];

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In this paper, we discuss inter-CU LTM related issues.

Discussion

Inter-CU SCG LTM

RAN2 agreed that source SN initiates inter-CU SCG LTM preparation, source SN generates SCG part configuration and MN includes it into its MN RRC configuration message in RAN2#127.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

And, in RAN2#128, RAN2 assumes that source SN may send measurement results of candidate PSCells to the MN, the MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. These agreements and assumptions are captured in draft 37.340 CR discussion. Here, the above agreement is captured as “the candidate SN provides the SCG part configuration of each candidate PSCell” in 37.340 CR. We are ok with the thinking since it is wasteful if the agreement means that source SN receives LTM candidate configurations collected by MN, generates aggregated configuration based on the LTM candidate configurations and sends it to MN. It is reasonable that MN collects LTM candidate configurations, generates the aggregated configuration and indicates it to candidate SNs including source SN to support subsequent LTM.

Observation 1. The agreement made in RAN2#127 “The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message” means that “The candidate SN provides the SCG part configuration of each candidate PSCell.”

In the RRC CR discussion, Rapp suggested to use new IE (ltm-ConfigurationSCG) to indicate the SCG part configuration. We think there is no agreement to use new IE for SCG LTM configuration. If RAN2 don’t introduce the new IE, LTM candidate configuration is merged in MCG ltm-Config. It means that the maximum number of LTM candidate settings for MCG and SCG will be 8 in total (it may be too small). We think the maximum number should be 16 in total. So, we support to introduce new IE for SCG LTM configuration.

Proposal 1. ltm-ConfigurationSCG used in draft RRC CR discussion should be introduced.

Proposal 2. ltm-Config (which doesn’t include inter-CU LTM candidate configuration) for MCG and ltm-ConfigurationSCG are simultaneously configured.

Here, we’ll consider whether ltm-Config for SCG and ltm-ConfigurationSCG are simultaneously configured. RAN2 agreed that intra-SCG LTM candidate configuration can be mixed in MN RRC format, i.e., ltm-ConfigurationSCG. It means that 8 intra-SCG LTM candidate configurations can be provided by MN RRC format while 8 intra-SCG LTM candidate configurations are configured by SN RRC format. It seems a violation of the agreement “The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.” And it is difficult how to differentiate the configuration which is indicated in the LTM CSC/TCI (de)activation MAC CE. Therefore, we suggest to prohibit that ltm-Config for SCG and ltm-ConfigurationSCG are simultaneously configured.

Proposal 3. ltm-Config for SCG and ltm-ConfigurationSCG are not simultaneously configured.

“ltm-ConfigSCG” is signalled via MCG RRC, but there can be UE variables which are used for MCG LTM. And RAN2 already agreed to support intra-MN LTM and inter-SN LTM. Even if the inter-SN LTM candidate configuration is provided by MN RRC format, UE variables, i.e., VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID and VarLTM-ServingCellNoSecurityChange (ID), for SCG should be used for ltm-ConfigurationSCG same as ltm-Config for SCG. Otherwise, UE overwrites UE variables for MCG upon SCG LTM cell switch execution while inter-SN LTM candidate configuration is configured. And, we could consider introducing new UE variables for this purpose, but we believe that reusing existing UE variables would have less spec impact. So, for ltm-ConfigSCG, UE should use UE variables for SCG.

Proposal 4. UE uses SCG UE variables for inter-SN LTM candidate configurations, e.g., ltm-ConfigSCG.

Then, RAN2 needs to discuss how to treat the ltm-Config for SCG when the ltm-ConfigSCG is configured. According to the current RRC CR and some agreements, inter-SN LTM candidate configurations are provided by MCG and a MAC CE indicating inter-SN LTM cell switch is provided by SCG. If SN provides intra-SN LTM candidate configuration via RRC signalling for the UE, UE holds ltm-Config for SCG and UE variables associated with ltm-Config for SCG. Thus, UE needs to release or replace the configuration and UE variables. If UE needs to release these, two implementation methods can be considered. First one is that NW (SCG) sends RRC signalling to the UE to release the configurations and variables, second one is that UE autonomously releases the configurations and variables. It seems wasteful if UE releases it by explicit signalling from the SCG from the perspective of radio resource efficiency. That is, the UE needs to autonomously release the contents of ltm-Config for SCG.

Proposal 5. UE autonomously releases the contents of ltm-Config for SCG when the ltm-ConfigSCG is configured.

For the reference configuration, RRC CR Rapp suggest to discuss whether the field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG in case it contains both MCG and SCG configurations. In this regard, RAN2 agreed that SCPAC similar principles are applied, i.e. (1) The MN can request an SCG reference configuration from any of the involved SNs, (2) The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration, (3) The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part (FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.). Then, MN can determine the field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG. Another solution is the field ltm-ReferenceConfiguration included in ltm-Config for MCG includes MCG part and SCG part, and there’s no field ltm-ReferenceConfiguration in ltm-ConfigSCG. It may reduce signalling overhead in case that both of intra-MN LTM and inter-SN LTM is configured. However, if not, it seems wasteful that the field ltm-ReferenceConfiguration including both of MCG part and SCG part is always configured. And, even if there is no MCG part, ltm-ReferenceConfiguration should be included in ltm-ConfigSCG if needed. So, we support that the field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG regardless whether it contains both MCG and SCG configurations or not.

Proposal 6. The field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG regardless whether it contains both MCG and SCG configurations or not.

There is an editor’s note in the RRC CR as below:

Editor’s Note: FFS whether for the SRBs we use the field reestablishPDCP within the LTM candidate configuration (same as for the RLC re-establishment in Rel-18).

Basically, in case of intra-CU cell switch, there is no need to re-establish PDCP entity. And PDCP re-establishment always includes security key update since same COUNT value cannot be used for same security key. It means all RBs (i.e., not only SRBs) are re-established if the SRBs are re-established. Therefore, if PDCP re-establishment is needed while subsequent LTM is performed except in a case of inter-CU LTM execution, PDCP entity should be re-established by explicit signalling. Furthermore, if it is needed, it should be supported in Rel-18 LTM. However, RAN2 decided not to use the field for LTM configuration. So, the field reestablishPDCP for the SRB within the LTM candidate configuration should not be used.

Proposal 7. The field reestablishPDCP for the SRB within the LTM candidate configuration is not used.

Early DL/UL synchronization

RAN2 agreed that R18 TCI state activation/deactivation MAC CE is reused. However, it is not discussed which node sends the MAC CE. For inter-CU MCG LTM case, the MN-DU should send the MAC CE since MCG receives L1 measurement report for inter-CU MCG LTM and this MAC CE is used for early DL sync with the candidate PCell. For inter-CU SCG LTM case, RAN2 already agreed that source SN sends the LTM CSC MAC CE to the UE. Therefore, it is natural that source SN sends TCI state activation/deactivation MAC CE even if the inter-CU SCG LTM configuration is signalled via MN RRC format.

Proposal 8. Source MN sends TCI state activation/deactivation MAC CE for inter-CU MCG LTM, and source SN sends TCI state activation/deactivation MAC CE for inter-CU SCG LTM.

Security

In the draft RRC CR discussion, Rapp raised following discussion point:

Editor’s Note: FFS whether the list of sk-counters is part of the LTM-Config or is provided outside (same level as ltm-Config in the RRCReconfiguration message)

We don’t see any advantages if the sk-Counters list is provided outside of ltm-Config. The list is used for subsequent inter-SN LTM. So, it should be provided in the LTM-Config.

Proposal 9. The list of sk-Counters is provided in the LTM-Config.

And Rapp suggested to use following structure for the list of sk-Counters:

LTM-Config-r18 ::=   SEQUENCE {

-----------------------------------omit-------------------------------------------

ltm-SK-Counters-r19                         SetupRelease {SK-CounterList-r19}                                   OPTIONAL,   -- Need M

cltm-ServingCellExecutionConditions-r19     SetupRelease {CLTM-ExecutionConditions-r19}                         OPTIONAL    -- Need M

]]

}

SK-CounterList-r19 ::=   SEQUENCE {

sk-CounterList-r19              SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter

}

And candidate configuration has no sk-CounterList. It means that a common sk-CounterList is configured for all candidate SNs. In the SCPAC scheme, a list is associated with a securityCellSetId, i.e. a list is configured per CU. In a case that the sk-CounterList is configured per CU, target CU can predict sk-Counter which is used by the UE after cell switching based on the sk-CounterList. Then, target CU can decode UL packet before selectedSk-Counter is provided via MN. If there’s no association between sk-CounterList and securityCellSetId, target CU must wait the selectedSk-Conunter information. Therefore, we suggest that NW should configure the sk-CounterList per ltm-NoSecurityChangeID as with SCPAC.

Proposal 10. NW should configure the sk-CounterList per ltm-NoSecurityChangeID as with SCPAC.

And Rapp raised another issue too, for contents of RRCReconfigurationComplete:

Editor’s Note: Whether to have a new field or re-use the existing selectedSK-Counter is FFS.

The existing selectedSk-Counter is used for SCPAC. So, an issue to consider when reusing the parameter is collided with SCPAC. If there’s only information of selectedSk-Counter, when selectedSk-Counter received from the UE is not included in an sk-CounterList for SCPAC, NW cannot determine whether it is an error or inter-SN LTM. However, RRCReconfigurationComplete message has fields which indicate the selected CondReconfigId and the applied LTM-CandidateId. So, we don’t see any issues on reusing the existing selectedSK-Counter for inter-SN LTM.

Proposal 11. Re-use the existing selectedSK-Counter for inter-SN LTM.

LTM and L3 HO

In RAN2#127, it was agreed that L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE while inter-CU LTM is being configured (w/o DC). It was also agreed that when performing L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations. That is, the UE simultaneously keeps the RRC configuration of L3 mobility (HO and CHO) and inter-CU LTM. Depending on the triggering timing, the UE performs either L3 mobility or inter-CU LTM. However, it is the MAC entity of the UE that receives the LTM CSC MAC CE from the source DU to trigger the inter-CU LTM, while the RRC layer of the UE receives the L3 HO command from the source CU to trigger L3 mobility. Based on Rel-18 intra-CU LTM, when the intra-CU LTM is triggered, the MAC entity of the UE subsequently notifies the RRC layer of the UE. Therefore, we propose that when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE further notifies the RRC layer of the UE, by following the rationale in Rel-18. Afterwards, RAN2 further discuss the cases when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and the execution order of L3 HO and inter-CU LTM.

Proposal 12. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 13. RAN2 further discuss

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution order of L3 HO and inter-CU LTM.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1. The agreement made in RAN2#127 “The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message” means that “The candidate SN provides the SCG part configuration of each candidate PSCell.”

Proposal 1. ltm-ConfigurationSCG used in draft RRC CR discussion should be introduced.

Proposal 2. ltm-Config (which doesn’t include inter-CU LTM candidate configuration) for MCG and ltm-

Proposal 3. ltm-Config for SCG and ltm-ConfigurationSCG are not simultaneously configured.

Proposal 4. UE uses SCG UE variables for inter-SN LTM candidate configurations, e.g., ltm-ConfigSCG.

Proposal 5. UE autonomously releases the contents of ltm-Config for SCG when the ltm-ConfigSCG is configured.

Proposal 6. The field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG regardless whether it contains both MCG and SCG configurations or not.

Proposal 7. The field reestablishPDCP for the SRB within the LTM candidate configuration is not used.

Proposal 8. Source MN sends TCI state activation/deactivation MAC CE for inter-CU MCG LTM, and source SN sends TCI state activation/deactivation MAC CE for inter-CU SCG LTM.

Proposal 9. The list of sk-Counters is provided in the LTM-Config.

Proposal 10. NW should configure the sk-CounterList per ltm-NoSecurityChangeID as with SCPAC.

Proposal 11. Re-use the existing selectedSK-Counter for inter-SN LTM.

Proposal 12. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 13. RAN2 further discuss

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution order of L3 HO and inter-CU LTM.

Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4

- R2-24xxxx, Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA

- R2-2410928, [POST128][106][MOB] (China Telecom) 37.340 Running CR

Annex draft 37.340 CR (excerpt of R2-2410928)

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

Editor’s Note: RAN2 assumes that the figure should be anchored together with RAN3. The final figure is up to the progresses made in RAN3.

1.	The source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required (Editor’s Note: FFS up to RAN3.) to the MN providing a list of candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells to the MN.

Editor’s Note: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN also provides a list of KSN and associated sk-Counter values for each candidate SN, and forwards the received measurement results to each candidate SN(s). The MN may select one of the candidate SN(s) and request providing the reference SCG configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN.

4.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message.

7.	The MN transfers, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to each candidate SN.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared.

12.	Editor’s Note: FFS up to RAN3.

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN.

Editor’s Note: Details for TA information transfer procedure is up to RAN3 discussion.

15.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9.

16.	The source SN decides to execute cell switch to a target PSCell.

17.	The source SN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID including PSCell change.

18/19.	The source SN sends a notification message to the target SN via the MN, to indicate the triggering of LTM to the UE.

Editor’s Note: Details for Cell Switch Notification procedure is up to RAN3 discussion.

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.

NOTE X:	The steps 12-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of SN-initiated inter-SN SCG LTM, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.