3GPP TSG-RAN-WG2 Meeting #129	R2-2501194

Athens, GR,17th– 21st Feb, 2025

Agenda item	:		8.6.3

Source	: 	Sharp

Title	: 	Discussion on issues for supporting L1 event triggered measurement reporting

Document for	:	Discussion and Decision

Introduction

In the last meeting, RAN2 agreed below [1]:

Agreements on L1 event triggered MR

1.	TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

2.	TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

3.	TTT is applied to the leaving condition.

4.	Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

5.	Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

6.	SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

7.	For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

8.	For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

9.	A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

10.	Support the truncated measurement report MAC CE.

In this paper, we discuss some issued related to conditional LTM.

Discussion

Evaluation

Cell specific offset

In the current CR for MAC sublayer [2], there is a following FFS:

Editor’s NOTE: Whether/How to introduce a cell specific offset (Ocn) for the LTM candidate cell measurement needs further discussion. Similar as other events.

In our understanding, the gNB can control the event triggered L1 measurement reporting by using beam specific offset and the value of the beam specific offset can be decided by considering the value of cell specific offset. Therefore, a cell specific offset is not needed.

Proposal 1. The cell specific offset is not supported for event triggered L1 measurement reporting.

MR MAC CE

RAN2 WG need to discuss the format and contents for event triggered measurement reporting MAC CE. For some issues, L1 MR UCI mechanism is reused

L1 MR UCI is described as above. Base on this MR format, we consider the following issues.

Number of beams reported

In L1 MR UCI, reported beams included in UCI is configured as L×M, where L is the number of reported cells and M is the number of reported beams per cell. Here, max value of both M and L are 4. Thus, the max number of reported beams is 16. There is no motivation to change these values to support the event triggered reporting. Therefore, we think same mechanism and values should be reused for event triggered L1 MR MAC CE.

Proposal 2. The network configure the number of reported beam as L×M, where L is the number of reported cells and M is the number of reported beams per cells. Max value of both M and L are 4.

Differential RSRP

In the RAN2#128 meeting, it was discussed whether differential RSRP mechanism is used or not in event triggered MR MAC CE. If we use the differential RSRP value to indicate the quality of beams, we can reduce the size of MR MAC CE. Therefore, we propose to use the differential RSRP mechanism.

Observation 1. Using the differential RSRP allows for a smaller MAC CE than if absolute values were used to represent RSRPs for all beams.

Proposal 3. Differential RSRP is supported for event triggered measurement reporting.

Beam(s) not satisfying the event

In the RAN2#128, it was agreed that beams that do not fulfils condition can be reported as below:

For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

Based on this agreement, following proposals are provided for RAN2 WG’s consideration:

One L1 measurement event which triggering a MR MAC CE transmission would be configured with entering condition and leaving condition. RAN2 WG is suggested to confirm the satisfied beam/event can be associated with a fulfilled entering condition or a fulfilled leaving condition when RAN2 WG is discussing the beam(s) not satisfying the event.

Proposal 4: RAN2 WG is suggested to confirm whether the satisfied beam/event can be associated with a fulfilled entering condition or a fulfilled leaving condition when RAN2 WG is discussing the beam(s) not satisfying the event.

If a MR MAC CE includes the information for both of the beams that satisfied a triggering event and the beams not satisfying the event jointly, the network can’t know which beams triggered this MR. To avoid such situation, indication to which beam(s) is satisfied (i.e. which beam(s) triggered this MR) should be provided in the MAC CE.

Proposal 5. The information indicating the ‘satisfied beam(s)’ / ‘not satisfied beam(s)’ should be included in MR MAC CE.

RAN2 WG also needs to discuss how UE selects the beams that do not fulfil condition to include in MR MAC CE to report. To solve this issue, RAN2 WG needs to discuss the priority / sequence while the UE needs to report the beam(s) which satisfying/not satisfying of one triggering event while the UE is creating a L1 MR MAC CE.

Proposal 6: RAN2 WG is suggested to discuss the priority/sequence of ‘satisfied beam/event‘ and ‘not-satisfied beam/event’ while the UE is creating one MR MAC CE.

Proposal 7: RAN2 need to discuss how the UE selects the beams that not satisfying the event to be included in the MR MAC CE.

Triggering Event

In addition, it is also not clear whether one MR MAC CE can include multiple triggering events. If multiple triggering events can be included in one MAC CE, RAN2 WG is suggested to discuss the priority /sequence of “satisfied beam” /”not-satisfied beam” those triggering events.

Proposal 8: RAN2 WG is suggested to confirm whether one MR MAC CE can only include one triggering event or more than one triggering events.

Conclusion

Proposal 1. The cell specific offset is not supported for event triggered L1 measurement reporting .

Proposal 2. The network configure the number of reported beam as L×M, where L is the number of reported cells and M is the number of reported beams per cells. Max value of both M and L are 4.

Proposal 3. Differential RSRP is supported for event triggered measurement reporting.

Proposal 4: RAN2 WG is suggested to confirm whether the satisfied beam/event can be associated with a fulfilled entering condition or a fulfilled leaving condition when RAN2 WG is discussing the beam(s) not satisfying the event.

Proposal 5. The information indicating the ‘satisfied beam(s)’ /‘not satisfied beam(s)’ should be included in MR MAC CE.

Proposal 6: RAN2 WG is suggested to discuss the priority/sequence of ‘satisfied beam/event‘ and ‘not-satisfied beam/event’ while the UE is creating one MR MAC CE.

Proposal 7: RAN2 need to discuss how the UE selects the beams that not satisfying the event to be included in the MR MAC CE.

Proposal 8: RAN2 WG is suggested to confirm whether one MR MAC CE can only include one triggering event or more than one triggering events.

Reference

- R2_128_R16R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM

- R2-2500xxx_Running MAC CR for Mob Ph4