3GPP TSG RAN WG2 Meeting #129	R2-2501229

Athens, Greece, Feb 17th – 21th, 2025

Agenda item:	8.7.5

Source:	Quectel

Title:	Discussion on RLC retransmission for XR

Document for:	Discussion and Decision

- Introduction

In RAN2 #128 meeting, following agreement is achieved [1]:

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In the contribution, some aspects for RLC retransmission are discussed furtherly.

- Discussion

Since the RLC retransmission is designed for the XR session with critical PDB and PER, and every flow in XR session require different QoS configuration, so gNB shall determine whether to configure this feature for every flow/DRB.

Proposal 1: The feature of RLC retransmission enhancement shall be configured per flow/DRB.

In order to guarantee the PDB of a XR packet, it is reasonable to retransmit the RLC SDU before the discard timer expiry. For example, a threshold B in figure 1 shall be introduced in which the transmitter shall retransmit the packet even if it has not received the RLC acknowledge.

Figure 1: Autonomous RLC retransmission

Proposal 2: A new threshold shall be introduced to trigger RLC retransmission before the discard timer expiry.

The next issue is which entity shall be configured this new threshold. In the current PDCP procedure, the discard timer is maintained by PDCP entity. When a packet is discarded, and this packet has been delivered to lower layer, PDCP entity indicates to RLC. If this new threshold is configured to PDCP entity, it could determine the right moment to indicate RLC to trigger the RLC retransmission.

Proposal 3: PDCP maintains the new threshold, and indicates RLC to trigger retransmission before the discard timer expiry.

Some companies suggest the an RLC SDU shall be retransmitted after a configured number of HARQ transmissions [2]. However, this triggering could introduce unnecessary interaction between RLC and MAC. The reasonable alternative is using a new timer which is started when an RLC SDU is transmitted, and is stopped when the RLC acknowledge is received. If this new timer expires, this RLC SDU is retransmitted. This is the timer A in figure 1.

Proposal 4: A new timer shall be introduced to trigger RLC retransmission after the initial transmission.

- Conclusion

Based on the discussion above, the following proposals are suggested:

Proposal 1: The feature of RLC retransmission enhancement shall be configured per flow/DRB.

Proposal 2: A new threshold shall be introduced to trigger RLC retransmission before the discard timer expiry.

Proposal 3: PDCP maintains the new threshold, and indicates RLC to trigger retransmission before the discard timer expiry.

Proposal 4: A new timer shall be introduced to trigger RLC retransmission after the initial transmission.

- References

- RAN2 #128 minutes

- R2-2409558 Discussion on RLC enhancements Qualcomm Incorporated