3GPP TSG-RAN WG2 Meeting #129	R2-2501310

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	7.4.1

Source:	Ericsson

Title:	Remaining open issues in NR NTN

Document for:	Discussion, Decision

# 1	Introduction

This contribution addresses the following two open discussion items for Rel-17 NR NTN.

Can further check whether we need to clarify the UE behaviour for PDD reporting during the transition phase

RAN2 understands that if pci-List parameter is absent in an SSB-MTC4 in SIB2/SIB4, the UE behaviour regarding how to determine the relationship between SMTC and different PCIs is unspecified.

# 2	Discussion

## 2.1	PDD reporting in satellite switch with resync

According to TS 38.331, the Propagation Delay Difference (PDD) indicates the one-way service link propagation delay difference between the serving cell and each neighboring cell included in the neighCellInfoList. Rather than cell information, this list actually includes the ephemeris and epoch time of neighboring satellites. The PDD is used by the network to adjust the SMTCs and measurement gaps for a UE in RRC_CONNECTED mode. Currently, beyond a maximum number of entries, there are no restrictions in the specification regarding the (type of) satellites that can be configured for PDD. However, in RAN#125, RAN2 reached the following understanding:

- RAN2 understands that the NW is not expected to configure PDD reporting between serving and target satellites involved in the satellite switch

After extensive discussions, it has been concluded that the serving cell's SMTCs are not expected to change between the source and target satellites. The status of intra-frequency neighbours for the soft switch scenario is still under discussion. Given that the purpose of PDD reporting is to adjust SMTCs and considering that SMTCs remain unchanged, we believe there is no need for enhancements or PDD reporting with the target satellite. However, we are open to capturing the previous RAN2 understanding in the specification (stage 3) to clarify that the network is not expected to configure PDD reporting for the target satellite.

Enhancements to PDD reporting during satellite switch with resync are not pursued in Release 18.

## 2.2	Use of PCI list in SMTC4

Over the last three meetings, RAN2 has discussed the optionality of the PCI list included in SMTC4. Some companies have argued that its optional status is an error, and that the parameter should be mandatory. The following was agreed upon in the last meeting:

- RAN2 understands that if pci-List parameter is absent in an SSB-MTC4 in SIB2/SIB4, the UE behaviour regarding how to determine the relationship between SMTC and different PCIs is unspecified.

While we agree that the behavior is not specified, we believe the UE behavior is clear and does not necessitate the mandatory presence of the parameter. For inter-frequency neighbors, the network can configure the smtc4list without the pci-List, which remains optional. In such cases, the downlink carrier frequency (dl-CarrierFreq parameter) is always configured in SIB4, as outlined in TS 38.331.

-- ASN1START

-- TAG-SIB4-START

[...]

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

ssbSubcarrierSpacing                SubcarrierSpacing,

[...]

...

}

[...]

InterFreqCarrierFreqInfo-v1720 ::=  SEQUENCE {

smtc4list-r17                       SSB-MTC4List-r17                                            OPTIONAL     -- Need R

}

[...]

-- TAG-SIB4-STOP

-- ASN1STOP

SIB19 contains information about neighbor cell configuration and may include NTN configuration (e.g., ephemeris), PCI, and carrier frequency.

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

[...]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

[...]

}

-- TAG-SIB19-STOP

-- ASN1STOP

The linkage between SIB4 and SIB19 allows the UE to apply SMTC configuration appropriately. Thus, even if the pci-List is not included in SIB4, as long as the carrier frequency is specified in the neighbor cell information, the UE can use these two pieces of information to configure SMTCs correctly. Failing to provide either carrier frequency or PCI in NTN-NeighCellConfig would indicate a flawed network implementation. While it may aid in measurements to inform the expected number of cells within the SMTC, a UE can still conduct measurements without this prior knowledge, as defined in RAN4 specifications. In scenarios where multiple neighbor satellites use the same downlink carrier frequency, the UE could still apply the SMTC configuration to both satellites. Nevertheless, this is considered an unlikely situation, and in such cases, the network would provide the PCI list to facilitate measurements.

- A UE can utilize the link between SIB19 and SIB4 to configure SMTC4 appropriately without a PCI list.

Further changes to the PCI list in SMTC4 or SIB4 are not pursued.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A UE can utilize the link between SIB19 and SIB4 to configure SMTC4 appropriately without a PCI list.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Enhancements to PDD reporting during satellite switch with resync are not pursued in Release 18.

Proposal 2	Further changes to the PCI list in SMTC4 or SIB4 are not pursued.