3GPP TSG-RAN2 Meeting#129	R2-2501327

# Athens, Greece, Feburary 17th - Feburary 21st, 2025

Agenda Item:	6.1.3.2

Source:	OPPO

Title:	Left Issue on Maximum Number of UL Segments

Document for:	Discussion, Decision

- Introduction

This is to discuss the remaining issue of maximum UL segments.

- Discussion

In [POST128][601], one remaining issue for maximum UL segments was raised, i.e., whether the new rrc-MaxCapaSegAllowed flag can be treated in a similar way as for rrc-SegAllowed as follows.

5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon, rrc-SegAllowed and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, requestedCapabilityCommon and rrc-SegAllowed) as defined in TS 36.331 [10], where applicable.

Looking back into the history where rrc-SegAllowed was added (i.e., CR4625 in R2-2401753), for which the reason for change was described as follows, i.e., 125 agreed on the change considering the feasibility of separate enquiry message scenario.

RAN2 understands that when the network queries UE capabilities for different RATs multiple times, rrc-SegAllowed-r16 field shall be consistent among multiple UECapabilityEnquiry messages.

Similar issue has been identified for rrc-SegAllowed for the scenario of separate enquiry messages.

When it comes to rrc-MaxCapaSegAllowed, the logic seems to be similar, i.e.,

- If we can limit to joint enquiry message, there is no remaining issue(s). Some voices have suggested this approach.

- However, some infra-vendor(s) still holds the concern for ENDC, where the network needs to first acquire LTE capability to determine if UE supports ENDC, and then acquire MR-DC/NR capability. As a result, there are also some voices supporting separate enquiry.

Based on offline, it seems there is still split view regarding whether to support separate enquiry case.

- Proponents tends to avoid restriction on network behavior;

- Opponents see difficulty to reach FS-ID consistency without information of allowed segment number of the subsequent enquiry;

Furthermore, there is a need to clarify the definition of separate / joint enquiry, e.g., there are more cases seen as “joint enquiry”

- Tthe NW want to ask LTE only first for some reason, it can ask LTE first (with rrc-MaxCapaSegAllowed = 1) and then request LTE+NR+MR-DC (with rrc-MaxCapaSegAllowed = 3) assuming that the total size limitation is 3”

- “Firstly, NW enquires EUTRA capability, maybe together with other RATs such as utra, geran-cs, geran-ps, and so on.Secondly, NW enquires EUTRA, MRDC and NR capability with requestedFreqBandsNR-MRDC filter included.”

For rrc-MaxCapaSegAllowed, while all support to rely on joint enquiry, there are still split view on whether to exclude separate enquiry.

For rrc-MaxCapaSegAllowed, ‘joint enquiry’ in O2 is the enquiry behavior that: when MR-DC/NR capability is enquired, it has to be always enquired together with LTE capability, regardless of whether there is a preceding enquiry for LTE or not. Otherwise, it is the ‘separate enquiry’ for which some vendors are against.

Otherwise, in case there is a need to allow separate / multiple enquiry, the issue is similar to rrc-SegAllowed. I.e., if a first enquiry is with a large number of allowed segmentation but later a second enquiry is with a small number of allowed segmentation, it may lead to:

- Issue-1: the UE may report some FS:s which fail to have associated BC/FSC reported later due to message length restriction.

- Issue-2: the UE may report a high order BC/FSC reported in MR-DC container but only lower order BC/FSC reported in NR container and thus lead to network confusion.

Based on the offline, some solution candidates were discussed:

Solution-1: Allow separate enquiry for ENDC, and restrict the setting of rrc-MaxCapaSegAllowed in a way similarly to rrc-SegAllowed. Example in Annex. For this approach, chip-set vendor(s) expressed the concern on ‘look ahead’ implementation.

Solution-2: Allow separate enquiry for ENDC, and leave the setting of rrc-MaxCapaSegAllowed up to network implementation, meaning no restriction on rrc-MaxCapaSegAllowed but also relying on the network to handle if any consequences. For this approach, although supported by some infra-vendor(s), other infra-vendor(s) expressed the concern on additional effort required by network.

Solution-3: For ENDC, allow a separate enquiry for LTE first, and then a joint enquiry on MRDC+NR, with rrc-MaxCapaSegAllowed set. Based on this approach, there is no inconsistency between MR-DC and NR, and no double-reporting. Yet there could be a risk on inconsistency between LTE and MR-DC/NR (yet since LTE capability size is relatively smaller, hopefully the risk is not of major concern?). For this approach, some chip-set vendor(s) either see the difficulty to reach FS-ID consistency (as said above), or believe that the MRDC/NR capability has to be generated in a conservative manner.

Based on offline, for the negative impact (Issue-1/2 above), companies (who support separate enquiry) tend to leave it to inter-vendor coordination to solve the setting of rrc-MaxCapaSegAllowed by implementation.

For the case of separate enquiry, based on offline, it is not preferred to impose a static ‘same value’ restriction on rrc-MaxCapaSegAllowed in specification, but can leave it to inter-vendor coordination to set proper value of rrc-MaxCapaSegAllowed by implementation.

- For the usage of rrc-MaxCapaSegAllowed for EN-DC, R2 discuss whether MR-DC/NR capability has to be always enquired together with LTE capability or not.

- If Yes to P1, R2 discuss how to reflect the restriction.

- If No to P1, it is left to network implementation to set proper value of rrc-MaxCapaSegAllowed, e.g., by inter-vendor coordination.

- Conclusion

Proposal 1	For the usage of rrc-MaxCapaSegAllowed for EN-DC, R2 discuss whether MR-DC/NR capability has to be always enquired together with LTE capability or not.

Proposal 2	If Yes to P1, R2 discuss how to reflect the restriction.

Proposal 3	If No to P1, it is left to network implementation to set proper value of rrc-MaxCapaSegAllowed, e.g., by inter-vendor coordination.

- Annex: TP for solution-1

Start change

5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon, rrc-SegAllowed, rrc-MaxCapaSegAllowed and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, requestedCapabilityCommon, rrc-SegAllowed and rrc-MaxCapaSegAllowed) as defined in TS 36.331 [10], where applicable.

Stop change