3GPP TSG-RAN WG2 Meeting #129                                      R2-2501515

Athens, Greece, Feb 17th – 21th, 2025

Source:   ZTE Corporation

Title:      [AT129][406][POS] MAC spec impact for UTW (ZTE)

Agenda item:   7.1.6

Document for:  Discussion and Decision

- Introduction

This document is to trigger the following email discussion:

- [AT129][406][POS] MAC spec impact for UTW (ZTE)

Scope: Further discuss the CR in R2-2500604 and determine what MAC spec is needed to clarify the operation of the UTW.

Intended outcome: Agreeable CR in R2-25001428

Deadline: Wednesday 2025-02-19 1900 EET

The related CR is referenced as below:

R2-2500604	Correction on SRS hopping in positioning	ZTE Corporation	CR	Rel-18	38.321	18.4.0	2031	-	F	NR_pos_enh2

- Discussion

In Rel-18, RAN1#114-bis has made following agreement to introduce UTW (UL time window):

Agreement in RAN1#114-bis For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without or outside UL time window (i.e. option 1) - FFS: details on the collision rules Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group. Note: UE is not expected to be configured with a SRS for positioning hopping cycle partially overlapping with UTW.

In the RRC signaling, the UTW is configured per serving cell, as follows:

– SRS-PosTx-Hopping The IE SRS-PosTx-Hopping specifies the frequency hopping configuration for SRS for Positioning transmission. SRS-PosTx-Hopping information element -- ASN1START -- TAG-SRS-PosTx-Hopping-START SRS-PosTx-Hopping-r18 ::= SEQUENCE { srs-PosConfig-r18 SRS-PosConfig-r17, bwp-r18 BWP OPTIONAL, -- Need R inactivePosSRS-TimeAlignmentTimer-r18 TimeAlignmentTimer OPTIONAL, -- Need M inactivePosSRS-RSRP-ChangeThreshold-r18 RSRP-ChangeThreshold-r17 OPTIONAL, -- Need M srs-PosUplinkTransmissionWindowConfig-r18 SetupRelease { SRS-PosUplinkTransmissionWindowConfig-r18 } OPTIONAL, -- Need M ... } SRS-PosUplinkTransmissionWindowConfig-r18 ::= SEQUENCE { startSFN-r18 INTEGER(0..1023), windowPeriodicityAndOffset-r18 CHOICE { periodicityAndOffset-r18 SRS-PeriodicityAndOffset-r16, periodicityAndOffset-Ext-r18 SRS-PeriodicityAndOffsetExt-r16 }, duration-r18 ENUMERATED {s1,s2,s4,s6}, ... } -- TAG-SRS-PosTx-Hopping-STOP -- ASN1STOP SRS-PosUplinkTransmissionWindowConfig field descriptions. duration Indicates the duration of the uplink SRS for positioning transmission window with frequency hopping. Value s1 indicates 1 slot, s2 indicates 2 slot and so on.. periodicityAndOffset Indicates the Periodicity and slot offset for uplink SRS for positioning transmission window occurrence with frequency hopping.. startSFN Indicates the starting SFN of the uplink SRS for positioning transmission window with frequency hopping..

In the RAN1 specification, the UE behaviour towards UTW is as below:

TS38.214-i50 6.2.1.4.1 SRS frequency hopping for positioning <omitted> The reduced capability UE may be configured, via srs-PosUplinkTransmissionWindowConfig, subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with a SRS resource for positioning with transmit frequency hopping for which the transmission, which includes all the hops once and the switching time from/to active BWP required ahead of the first hop and after the last hop, is partially overlapped with an UL time window.

Base on the above, there are several facts/observations from the Rapporteur:

- UTW is designed to resolve the conflict between SRS hopping and other uplink signal/channels in the window, not to control whether to send the SRS hopping in the window;

- UTW is configured in RRC signaling with a start time, a window duration, a time offset and a time periodicity;

- RAN1 specification says UE will not transmit other signal/channels in the UTW except for SRS hopping;

- Neither RAN1 nor RAN2 introduces an uplinkTimeWindowTimer for SRS hopping in Rel-18 RedCap UE positioning.

From Rapp’s perspective, RRC configures the UTW, and then UE’s physical layer does not transmit other UL signal/channels in the UTW, so the UTW does not have MAC spec impact.

Similar case happens to PPW (PRS processing window), wherein MAC spec does not specify the PPW time location with a formula:

- 38.321-i40 - 5.24 Handling of PRS Processing Window When PPW is activated and PRS has higher priority than DL channel and signals, for the affected symbols within the PPW according to clause 5.1.6.5 in TS 38.214 [7], the MAC entity shall: 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.4a and 5.1.5. 1> else: 2> not receive DL-SCH; 2> not receive PDCCH.

So, the MAC CR R2-2500604 provides the following changes:

- In section 5.26, delete the SRS transmitting criteria of ‘uplinkTimeWindowTimer is running’;

- In section 5.32, delete the UTW window configuration/parameters, and delete the UTW time location formula;

- In section 5.32, delete the description related to the timer control, such as ‘MAC starts the uplinkTimeWindowTimer’, ‘MAC entity shall instruct the lower layer to transmit SRS for positioning Tx frequency hopping when the uplinkTimeWindowTimer is running’.

Rapporteur would like to confirm with companies about the following questions:

Q1: For the feature of RedCap UE SRS hopping, do you agree to delete the description related to ‘uplinkTimeWindowTimer’ in the MAC spec, including the MAC behavior towards the start/stop/expire of the ‘uplinkTimeWindowTimer’?

| Companies | Agree/Disagree | Comments |
| --- | --- | --- |
| Xiaomi | Agree | There is no explicit timer for the UTW according to the RAN1 agreements and RRC parameters. |
| CATT | Agree | We have the same understanding as Xiaomi and Rapporteur, we should follow RAN1’s agreements. |
| Ericsson | Agree | It appears RAN1 has captured the description (UE behavior) on UTW in their spec and thus we do not need any MAC impact. |
| Samsung | Agree |  |
| vivo | Agree |  |
| Lenovo | Agree | We understand that PHY will handle the UL time window configuration. |

Q2: Do you agree to delete UTW time location calculation formula in MAC specification?

| Companies | Agree/Disagree | Comments |
| --- | --- | --- |
| Xiaomi | Agree | Considering there is no uplinkTimeWindowTimer in the MAC spec, the UTW time location should be calculated in the physical layer based on the RRC parameters. |
| CATT | Agree | We have the same understanding as Xiaomi and Rapporteur, we should follow RAN1’s agreements. |
| Ericsson | Agree |  |
| Samsung | Agree | If we specify UTW calculation formula in MAC, MAC layer should check the start/end of UTW and indicate it to physical layer. However, in RAN1 specification, there is no corresponding procedure text to handle the UTW start/end indication from MAC layer. Therefore, to keep the UTW formula in MAC, we need additional work on RAN1 specification, which is not desirable to us. |
| vivo | Agree |  |
| Lenovo | Agree | We think that also the description of SRS-PosTx-Hopping in RRC can be revised a bit since currently there is no description of UTW at all except of the corresponding field descriptions, e.g. adding a pointer to RAN1 spec 38.214. But this can be done by a separate CR at next meeting. |

Q3: If you disagree with the above change, please provide an example TP that you want to have.

| Companies | Comments |
| --- | --- |
| Huawei, HiSilicon | The spec can be changed as follows? as a compromise |

Rapp Summary:

(6/7) companies agrees with the following changes:

- Delete the MAC behavior related to uplinkTimeWindowTimer;

- Delete the UTW calculation formula in MAC spec.

Considering the majority view and the opponent didn't provide convincing argument for keeping the formula, the decision is to keep the CR content as it is.

- Conclusion

TBD