3GPP TSG-RAN WG2 Meeting #129bis                                                               R2-2501901

Wuhan, China, 7th – 11th Apr, 2025

Source:	CATT

Title:	Consideration on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

In this contribution, we share our view on L3 measurement and MAC CE design for OD-SSB in Section 2. Our proposals are summarized in Section 3.

Discussion

OD-SSB measurement of SCell

- L3 RRM Framework

In the RAN2#127bis meeting, the details on L3 RRM framework were discussed and the following two options were summarized.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

During the previous RAN4#112, 113 and 114 meetings, the issue of OD-SSB based deactivated SCell measurement was discussed, and RAN4 have the following agreements:

Agreement of RAN4#112meeting: Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. Agreement of RAN4#113 meeting: Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Agreements of RAN4#114 meeting: Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

According to the above agreement, a fast time window is defined by RAN4. Within the fast time window, UE performs the OD-SSB based fast L3 measurement for deactivated SCell, i.e., the measurement period in fast time window is related to OD-SSB period regardless of measCycleSCell and DRX cycle.

Observation 1: A fast time window is defined by RAN4, during which UE performs the OD-SSB based fast L3 measurements for deactivated SCell within a measurement period associated with the OD-SSB period.

Furthermore, the RAN4 conclusions indicate that the measurement of OD-SSB in the fast time window is completely dependent on the period of OD-SSB, while the legacy deactivated SCell measurement requirement (i.e. N1*measCycleSCell) are applied outside this fast time window, as shown in Figure 1.

Observation 2: During OD-SSB based deactivated SCell measurement, OD-SSB measurements within a fast time window rely exclusively on the period of OD-SSB, and the legacy deactivated SCell measurement requirement (i.e. N1*measCycleSCell) are applied for the measurement of OD-SSB outside the fast time window.

Figure 1: The OD-SSB based deactivated SCell measurement procedure

Based on this observation, we recommend adopting Option 2 within the fast time window. This approach provides a simple implementation. If Option 1 is supported, the only benefit is that it follows the legacy L3 RRM framework, i.e., OD-SSB is measured within SMTC. However, there will be the following issues:

- Redundant Window Definition: RAN4 has already reached a consensus that the measurement period of OD-SSB within the fast time window is equal to the period of OD-SSB. Introducing an additional SMTC list would create unnecessary duplication;

- Unnecessary Parameter Dependency: This implementation requires establishing a connection between OD-SSB periodicity and SMTC configuration. Specifically, it would mandate dynamic SMTC updates based on OD-SSB period variations, introducing additional complexity and signalling overhead without corresponding performance benefits.

Proposal 1: In the fast time window, the RRM measurement of OD-SSB is performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

In addition, for OD-SSB measurements outside the fast time window or in the case of the known cell under Option 2 of Case# 2 of the above agreement, three potential approaches could be considered:

- Option 1: Perform measurements with N1*measCycleSCell measurement cycle while considering SSB transmission according to the corresponding OD-SSB period;

- Option 2: Perform measurements with N1*measCycleSCell measurement cycle while considering SSB transmission following an existing SMTC configuration in MeasObjectNR for one SSB frequency, no matter there are multiple possible OD-SSB periods within one frequency.

- Option 3: Perform measurements with N1*measCycleSCell measurement cycle while considering SSB transmission within an SMTC list (the SMTC list is related to different OD-SSB periods).

Based on the RAN4 agreements, we believe that option 1 is sufficient for OD-SSB measurements outside the fast time window. Option 2 and option 3 would split OD-SSB measurements into two methods during SCell deactivation. In order to align with the OD-SSB measurement in the fast time window, we suggest that OD-SSB measurements outside the fast window also follow the OD-SSB pattern.

Proposal 2: After the fast time window, OD-SSB is measured in measurement cycle based on OD-SSB pattern.

- Measurement performing

For Case #1, we had agreed UE does not expect to measure SSB when on-demand SSB transmission is deactivated. Therefore, the UE does not perform measurements until it receives an OD-SSB transmission indication, e.g., new MAC CE for OD-SSB transmission.

For Case #2, in the last RAN2 meeting, RAN2 reached the conclusion of "RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2”. And at RAN1#120 meeting, the relation of frequency location between the always-on SSB and on-demand SSB were discussed with the following agreements.

Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

Based on RAN1 agreements, when AO-SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. In the existing specification, a cell is associated with a single servingCellMO. In order to support the configuration of OD-SSB and AO-SSB in different frequency, an additional new servingCellMO for OD-SSB should be introduced.

Proposal 3：A new servingcellMO is introduced to support the case that OD-SSB and AO-SSB are in different frequency.

- L3 measurement report triggering

- Scenario #2

For Case #1, the legacy measurement report triggering can be configured, e.g, PeriodicalReportConfig and EventTriggerConfig. After the OD-SSB transmission is activated, the measurement is performed and the measurement reporting is triggered if the measurement reporting conditions are met.

Observation 3: For Scenario #2 with Case #1, there is no spec impact on measurement reporting.

According to RAN4 [2], if the SCell is an unknown SCell, after UE receives SCell activation command, UE has to perform AGC, synchronization and L1 measurement/report based on SSB, and then gNB activate TCI state for data transmission according to UE’s report. If the SCell is a known SCell, gNB can directly activate the TCI state for data transmission. Therefore, larger activation delay is required for the unknown SCell.

Observation 4: The latency of activating a SCell is different between a known cell and an unknown cell.

And according to RAN4 [2], for known cells, there is a limit to the time between the measurement reporting and UE receiving the SCell activation command.

8.3.2	SCell Activation Delay Requirement for Deactivated SCell

SCell in FR1 is known if it has been meeting the following conditions:

- During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified

in clause 9.2 and 9.3.

- the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

- During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time

when UE receives MAC-CE command for TCI activation

- During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

…

If OD-SSB and AO-SSB are at the different frequency, a measurement report triggering can be configured separately for AO-SSB and OD-SSB, e.g., the measurement report configuration can be associated with different MeasObjectId, and after OD-SSB transmission is activated, the measurement is performed and the measurement reporting can be triggered. It is similar as Scenario #2 with Case #1, legacy measurement reporting mechanism for OD-SSB frequency can be used after OD-SSB is activated.

Conversely, if OD-SSB and AO-SSB are at the same frequency, according to the existing specification, if the event triggered reporting is configured, UE may not trigger the measurement reporting after receiving the OD-SSB transmission indication, e.g. the measurement reporting for the cell has already triggered based on AO-SSB and no new applicable cell is fulfilled the event triggered condition. But the measurement reporting based on AO-SSB is triggered too early, and the SCell is an unknown cell. It is beneficial to quickly measure and report the measurement results after receiving the OD-SSB transmission indication, which can change the SCell from unknown cell to known cell to accelerate the activation of SCell.

Observation 5: For Scenario #2 with Case #2, if the AO-SSB and OD-SSB are at the same frequency, the SCell may be an unknown cell if UE does not trigger the measurement reporting after receiving the OD-SSB transmission indication.

Therefore, if AO-SSB and OD-SSB are at the same frequency, in order for UE to quickly measure and report measurement results after receiving the OD-SSB transmission indication, turn an unknown cell into a known cell, a new trigger condition of measurement reporting should be introduced, i.e., UE triggers measurement reporting after the OD-SSB is activated via MAC CE.

Proposal 4: For Scenario #2 with Case #2, if AO-SSB and OD-SSB are in the same frequency, a L3 measurement reporting is triggered after the OD-SSB is activated via MAC CE.

- Scenario #2A

Enhancement for multiple unknown SCells’ activation was discussed in R18. The RRC CR [3] for introduction of FR2 SCell enhancements is shown below.

| Reason for change: | This CR is to introduce the FR2 unknown SCell activation enhancement as requested by RAN4 in their LS in R2-2304636 and R2-2307039. In the enhancement of network triggering report (of L3 measurement result) during FR2 unknown SCell activation, when SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported on the unkown SCell, the RRC measurement report will be intiated which includes the SCell with beam level measurement result for that SCell. RAN2 agreed to reuse the existing RRM measurement framework with a new report type to support this enhancement. In RAN2#123bis meeting, RAN2 made the following agreements on multiple SCell activation case: - If the network activates multiple Sells within same MAC CE the UE may send only one measurement report. |
| --- | --- |

Based on the CR, after receiving the SCell activation command, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated. This can assist SCell activation.

Observation 6: During SCell activation procedure, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated.

Different from the scenario in R18, in Scenario #2A, the UE doesn’t have valid L3 measurement results. For example: in Scenario #2A + case #1, the UE doesn’t have available L3 measurement results for the SCell. Therefore, it is unclear if it is meaningful to introduce L3 measurement based on OD-SSB in Scenario #2A. Maybe RAN2 can discuss the issue or ask RAN4 if it is possible.

Proposal 5: RAN2 discusses whether L3 measurement based on OD-SSB is applied in Scenario #2A.

OD-SSB measurement of neighbor cell

According to the agreement of “the UE expects to measure SSB when on-demand SSB transmission is activated”, there is a question about whether UE can measure the OD-SSB of a neighbor cell when the OD-SSB transmission of the neighbor cell is activated. According to the WID [4], OD-SSB transmission is only applicable to SCell. Indicating the OD-SSB transmission of the neighbor cell to the UE is not supported.

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

- Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

- Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In addition, during RAN4 # 114 meeting [5], following agreements have been achieved on neighbour cell measurement.

Issue 7-2: OD-SSB based Neighbour cell measurement - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell Agreement: - For Scenario 1a: No RAN4 spec. change. - Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2.

Based on the agreement, RAN4 will not update the neighbour cell L3 measurement requirement due to the OD-SSB transmission. This means that UE will continue to measure SSB using the always-on SSB measurement configuration, even if OD-SSB transmission is activated. Therefore, OD-SSB SCell operation has no impacts to RRM measurement on neighbor cell.

Proposal 6: OD-SSB SCell operation has no impact to RRM measurement of neighbor cells.

Contents of OD-SSB MAC CE

In the RAN2#127bis meeting, it was agreed that NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. In the existing specification [6], the SCell Activation/Deactivation MAC CE support 1 octet (7 SCells) or 4 octets (31 SCells). Therefore, the new MAC CE should also support sending OD-SSB indication for 7 or 31 SCells simultaneously.

Proposal 7: The new OD-SSB MAC CE can indicate up to 7 or up to 31 SCells.

In the RAN1#119 meeting, following agreements have been achieved on OD-SSB deactivation:

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Based on the above agreement, explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication has been approved by RAN1. In the previous RAN2 meeting, a new MAC-CE for OD-SSB transmission indication is introduced. For simplicity, the new MAC CE for OD-SSB transmission indication should indicate OD-SSB activation and deactivation.

Observation 7: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission has been approved by RAN1.

Proposal 8: The new MAC CE for OD-SSB transmission indication can be used for OD-SSB activation and deactivation.

During RAN1#118 meeting, it was agreed that multiple candidate values of periodicity of the on-demand SSB can be configured by RRC and the applicable value can be indicated by MAC CE as follows.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB. - FFS: Any other relevant parameters.

RAN1 had agreed in the RAN1#119 meeting that the period of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. Hence, the following two options can be used to indicate the period of on-demand SSB:

- Option1: 3bit in the new MAC CE is used to indicate the activated period of OD-SSB (5ms, 10ms, 20ms, 40ms, 80ms and 160ms) explicitly;

- Option2: OD-SSB configuration by RRC includes the period of the on-demand SSB and other parameters. Several OD-SSB configurations for a SCell may be configured via RRC in advance. When an OD-SSB is activated, the corresponding OD-SSB configuration index is included in the new MAC CE.

In RAN1#120 meeting, RAN1 has agreed to provide several parameters for OD-SSB configuration by RRC as follows.

Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

And in the previous RAN1 #119 and 120 meeting, time domain location of on-demand SSB burst was discussed and RAN1 have the following agreements:

Agreement of 119 meeting - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. Agreement of 120 meeting - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists

Based on the above agreement, the time domain location of on-demand SSB is configured per on-demand SSB periodicity by RRC, including OD-SSB periodicity, SFN offset and half frame index. If option1 is supported, MAC CE also needs to indicate the time domain location of OD-SSB. So from the perspective of signaling overhead, it is more effective to include the period of on-demand SSB as a parameter of OD-SSB configuration by RRC. Option2 seems more appropriate.

Proposal 9: The different periods can be associated to different OD-SSB configuration index in RRC, and the activated OD-SSB configuration index is indicated in the new OD-SSB indication MAC CE.

From the above, the OD-SSB transmission MAC CE should indicate OD-SSB activation/deactivation state and the activated OD-SSB configuration index. For the content of MAC CE, two fields can be used to indicate OD-SSB transmissions:

- Field 1: A bitmap indicating the OD-SSB activation/deactivation state for each SCell;

- Field 2: The OD-SSB configuration index for each SCell that OD-SSB is activated.

During the previous meeting, some companies were of the view that the above design might lead to redundant transmission of the activation of the OD-SSB configuration index. For example, when the OD-SSB transmission of SCell 1 is activated at time t, and the OD- SSB transmission of SCell 2 is activated at time t + 1, it is also necessary to indicate the OD-SSB transmission of SCell 1 at time t+1. As a result, some companies proposed introducing an additional field to indicate which SCell the activation/deactivation status of OD-SSB applies to. However, compared to the above design, adding a dedicated field to indicate which SCell the activation/deactivation status of OD-SSB applies to will also inevitably introduce extra signaling overhead as each indication would require an additional field, which may not effectively reduce the overall overhead. Therefore, two fields indicating the transmission of OD-SSB are sufficient.

Proposal 10: In the OD-SSB MAC CE, the following two fields can be used to indicate OD-SSB transmissions:

- Field 1: A bitmap indicating the OD-SSB activation/deactivation state for each SCell;

- Field 2: The OD-SSB configuration index for each SCell that OD-SSB is activated.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: A fast time window is defined by RAN4, during which UE performs the OD-SSB based fast L3 measurements for deactivated SCell within a measurement period associated with the OD-SSB period.

Observation 2: During OD-SSB based deactivated SCell measurement, OD-SSB measurements within a fast time window rely exclusively on the period of OD-SSB, and the legacy deactivated SCell measurement requirement (i.e. N1*measCycleSCell) are applied for the measurement of OD-SSB outside the fast time window.

Observation 3: For Scenario #2 with Case #1, there is no spec impact on measurement reporting.

Observation 4: The latency of activating a SCell is different between a known cell and an unknown cell.

Observation 5: For Scenario #2 with Case #2, if the AO-SSB and OD-SSB are at the same frequency, the SCell may be an unknown cell if UE does not trigger the measurement reporting after receiving the OD-SSB transmission indication.

Observation 6: During SCell activation procedure, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated.

Observation 7: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission has been approved by RAN1.

OD-SSB measurement of SCell

Proposal 1: In the fast time window, the RRM measurement of OD-SSB is performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

Proposal 2: After the fast time window, OD-SSB is measured in measurement cycle based on OD-SSB pattern.

Proposal 3：A new servingcellMO is introduced to support the case that OD-SSB and AO-SSB are in different frequency.

Proposal 4: For Scenario #2 with Case #2, if AO-SSB and OD-SSB are in the same frequency, a L3 measurement reporting is triggered after the OD-SSB is activated via MAC CE.

Proposal 5: RAN2 discusses whether L3 measurement based on OD-SSB is applied in Scenario #2A.

OD-SSB measurement of neighbor cell

Proposal 6: OD-SSB SCell operation has no impact to RRM measurement of neighbor cells.

Contents of OD-SSB MAC CE

Proposal 7: The new OD-SSB MAC CE can indicate up to 7 or up to 31 SCells.

Proposal 8: The new MAC CE for OD-SSB transmission indication can be used for OD-SSB activation and deactivation.

Proposal 9: The different periods can be associated to different OD-SSB configuration index in RRC, and the activated OD-SSB configuration index is indicated in the new OD-SSB indication MAC CE.

Proposal 10: In the OD-SSB MAC CE, the following two fields can be used to indicate OD-SSB transmissions:

- Field 1: A bitmap indicating the OD-SSB activation/deactivation state for each SCell;

- Field 2: The OD-SSB configuration index for each SCell that OD-SSB is activated.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #RAN2 129

- 3GPP TS 38.133, “Requirements for support of radio resource management (Release 18)”, V18.4.0 (2023-12)

- R2-2313936, Introduction of FR2 SCell enhancements

- RP-242354 Revised WID: Enhancements of network energy savings for NR

- RAN4 #114 RRM session meeting report

- 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification (Release 18)”, V18.2.0 (2024-06)