3GPP TSG RAN WG2 Meeting #129-bis		     	R2-2501905

Wuhan, CN, Apr. 7th – 11th, 2025

Agenda item:	8.3.3

Title:	Discussion on measurement event prediction using large language model

Source:	Beijing Jiaotong University (BJTU)

Document for:	Discussion

1. Introduction

For measurement event prediction, the following agreement was made in RAN2#125bis:

Agreement (RAN2#125bis) - At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed. - Clarifications on what is being as input should be provided with results - Start with A3 as a baseline. - Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

For measurement event prediction, the following agreement was made in RAN2#127bis:

Agreement (RAN2#127bis) -Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement). FFS what is KPI. -Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing. Focus on temporal case B. -Companies will prioritize simulations on indirect method. Companies can bring simulations on direct method and should report what method is being used. -Measurement event prediction results are expected in RAN2#129

In this contribution, we propose an large language model (LLM)-based measurement event prediction method that leverages real-time UE mobility information to predict Event A3 parameters (TTT, HOM) under varying environmental conditions, integrating RRM measurement prediction to enhance measurement event prediction accuracy.

2. Motivation

In conventional indirect measurement event prediction, the RRM prediction model estimates measurement events based on predicted radio conditions. It takes cell- or beam-level measurements from serving and neighboring cells as input and outputs predicted cell-level results. A predefined rule-based mechanism (non-AI) then derives event predictions, such as Event A3, by checking if the predicted RSRP of a neighboring cell exceeds that of the serving cell for the TTT duration. This approach provides an estimated event occurrence time but remains limited by its fixed rule-based design.

The rule-based approach relies on fixed TTT and A3 offset values, which do not adapt to real-time variations in UE mobility states and dynamic channel conditions. This static configuration fails to account for the diverse handover requirements in different scenarios. For instance, high-speed UEs traverse multiple eNBs within a short period, necessitating a low TTT to prevent delayed handovers. Conversely, low-speed UEs experience stable signal conditions over shorter distances, requiring a higher TTT to avoid unnecessary early handovers. Moreover, factors such as UE movement direction and cell service quality further impact the optimal selection of handover parameters. A rigid rule-based mechanism that applies uniform values across all scenarios cannot effectively adapt to these real-world variations, leading to suboptimal handover decisions, increased handover failures, and potential link instability. Therefore, a more adaptive approach is needed to dynamically adjust TTT, A3 offset, and other handover control parameters based on the real-time environment. This would help minimize handover failures and enhance link stability by ensuring that handover decisions align with the UE’s mobility state, trajectory, and network conditions.

In this contribution, we propose a Large Language Model (LLM)-based measurement event prediction method. This approach enables real-time acquisition of user UE mobility information, including movement direction, velocity, and RSRP. These features are fed into the LLM to obtain key parameters for Event A3, such as TTT and A3-offset, under different environmental conditions. By integrating this with radio resource management (RRM) measurement prediction, our method enhances the accuracy of cvent A3 prediction, improving mobility management efficiency.

Proposal #1: Consider applying AI techniques to enable dynamic adjustment of TTT and A3 offset based on real-time network conditions and UE mobility states, ensuring adaptive handover control.

3. Scenarios description

Fig. 1 The scenario of HSR communications

Without loss of generality, we consider an HSR FR2 mmWave wireless communication scenario with a user equipment (UE) and seven base stations (BS) shown in Fig.1. Each BS is equipped with three half-wavelength spaced uniform plane arrays (UPAs) of 32 antennas with sector orientation of 30, 150 and -90 degrees, respectively, and we consider a downlink communication system with analog beamforming architecture for each UPA. Meanwhile, the UE is equipped with two half-wavelength spaced antennas without beamforming and moves according to the street model. The distance between base stations is set to 200m, and the channel model is referenced to 3GPP TR 38.901 UMa for LOS propagation. Detailed simulation parameters are shown in Table 1.

Table 1 Simulation setting

| Parameters | Values |
| --- | --- |
| Channel | UMa |
| Bandwidth | 80MHz |
| Subcarrier bandwidth | 120KHz |
| Number of subcarriers | 64 |
| Center frequency | 30GHz |
| Antenna setup of BS | （M, N, P, Mg, Ng）=（4, 8, 1, 1, 1） |
| Height of BS | 25m |
| Antenna setup of UE | （M, N, P, Mg, Ng）=（2, 1, 1, 1, 1） |
| Height of UE | 1.5m |
| UE distribution | 100%outdoor |
| UE speed | 60km/h、120km/h、350km/h、500km/h |
| UE motion trajectory | City street |
| Transmitting power of BS | 40dbm |
| Number of sites | 7 |
| Number of sectors in a site | 3 |
| Number of UE | 1 |
| Sampling interval | 40ms |
| Distance between BSs | 200m |

4 LLM-Based Measurement Event Prediction

Large Language Models (LLMs) are advanced deep learning models trained on vast amounts of text data using transformer architectures. These models leverage self-attention mechanisms and massive parameter scaling to understand, generate, and reason over complex textual and numerical information. Recent advancements have extended LLM capabilities beyond natural language processing (NLP) to multimodal applications, enabling them to process structured data, numerical inputs, and even spatial-temporal information.

Cross-modal learning refers to integrating multiple data types—such as text, images, and structured numerical data—within an AI model. For measurement event prediction, LLMs can process both textual network parameters and structured mobility features, enabling intelligent decision-making for dynamic handover control. Common paradigms for cross-modal integration in LLMs include: 1) Prompt Engineering & In-Context Learning (ICL): Encoding structured mobility data as text-based prompts for LLM inference. 2) Embedding-Based Fusion: Mapping numerical inputs (e.g., RSRP, velocity) into a shared latent space, allowing the model to correlate different feature modalities. 3) Adapter-Based Fine-Tuning (e.g., LoRA, Prefix-Tuning): Efficiently updating LLM weights using lightweight adaptation layers, reducing computational overhead while maintaining strong generalization.

Our proposed LLM-based measurement event prediction method begins by collecting real-time UE mobility information, including movement direction, velocity, and RSRP from both serving and neighboring cells. These raw features undergo patching and embedding processes to extract structured representations. To enhance the LLM’s comprehension, prompt engineering is introduced to encode prior knowledge about the simple mapping relationships between UE mobility information and key handover parameters, TTT and A3 offset. The extracted features and prompts are then combined, enriched with position embeddings, and fed into the LLM. The hidden states from the final LLM layer are further processed through a multi-layer fully connected (FC) network to derive the optimal TTT and A3 offset under varying environmental conditions. To improve task-specific adaptation, LoRA is applied to fine-tune a small subset of the model’s parameters efficiently. Finally, the LLM’s predicted handover parameters are integrated with the RRM prediction model’s output, enabling more accurate Event A3 prediction and enhancing mobility management efficiency.

Fig. 2 LLM-Based Measurement Event Prediction

Proposal #2: Leverage LLM to achieve multi-metric joint prediction of TTT and A3 offset by integrating UE mobility features, signal quality, and network conditions for more adaptive and accurate handover decisions.

5 Conclusion

In this contribution, we propose an LLM-based measurement event prediction method that leverages real-time UE mobility information to predict Event A3 parameters (TTT, HOM) under varying environmental conditions, integrating RRM measurement prediction to enhance measurement event prediction accuracy.

Proposal #1: Consider applying AI techniques to enable dynamic adjustment of TTT and A3 offset based on real-time network conditions and UE mobility states, ensuring adaptive handover control.

Proposal #2: Leverage LLM to achieve multi-metric joint prediction of TTT and A3 offset by integrating UE mobility features, signal quality, and network conditions for more adaptive and accurate handover decisions.