3GPP TSG-RAN WG2 Meeting #129bis	 R2-2502106

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.3.3

Source:	MediaTek Inc.

Title:	Discussion and preliminary results for direct and indirect event prediction

Document for:	Discussion, Decision

Introduction

In the previous #128 and #129 meeting, RAN2 made the following agreements for SLS evaluation for AI/ML mobility of event prediction and preliminary intermediate results for direct event prediction.

Agreements for system level simulation

1 	For measurement event prediction for temporal domain case B:

UE reports when A3 event is satisfied with actual measurements and predicted results.   And handover command will be received after handover preparation.

Remove T0 from the picture

2	For measurement event prediction for temporal domain case A, company focus on option 2 or option 3.

Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1))

Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

3	Handover Preparation time is 40ms and handover execution time: 40ms

Agreements on direct measurement event prediction - F1 score for direct measurement is very good based on the current simulation results.

In this paper, we focus on the evaluation on indirect and direct event prediction and provide preliminary results for intermediate KPI and system level performance.

Discussion

To improve the mobility performance, measurement event prediction can be utilized to enable proactive measurement reporting, thereby reducing handover latency; Or prevent the transmission of unnecessary measurement reports that could lead to unnecessary handovers. Two approaches could be applied to predict the measurement event, in the first approach, the UE performs continuous AI inference. In the second approach, the UE only performs AI inference when the A3 event criteria are met. As these are implementation-based approaches, the choice between these two methods does not have any impact on the specifications. Our focus should be on evaluating the performance of these two approaches.

Proposal 1: To evaluate the performance of measurement event prediction with following two options of UE inference operation:

- Option 1: UE does continuous inference, predicting if event is about to happen in the future/the prediction window.

- Option 2: UE starts AI inference when the entering condition of event is fulfilled, the UE predicts if the occurring event is valid in the future/ the prediction window. If the entering condition of the occurring event is remain satisfied in the future prediction window, the occurring is considered as valid, otherwise, it is invalid.

Indirect event prediction

Simulation Results of Case B

For case B, we need system-level simulation to verify the impact of HO performance when we apply the AI prediction to replace the real measurement. Our system-level simulation settings are given in the following table and other general settings can be found in the Appendix.

| Setting/Parameters | Value |
| --- | --- |
| TTT | 320ms |
| A3 offset | 2dB |
| # of UE | 8 |
| Running time for each UE | 400s |
| Max ETD | 80ms |

Table 1: Parameters of system level simulation

In system-level simulation (SLS), we compare three different approaches (1) legacy HO: this is the normal HO without any measurement reduction, which provides a baseline to compare the performance loss (2) sample and hold: the non-AI approach considers the measurement reduction based on MRRT, replacing the absent measurement with the last real measurement result. (3) AI: Using AI to predict the absent measurement. We consider two different MRRTs, e.g., 0.5 and 0.8, and the corresponding results are given in the following two tables.

| FR1 Temporal domain case B, UE speed = 30 km/hr, MRRT = 0.5 | Intermediate KPI | System level KPI |  |  |
| --- | --- | --- | --- | --- |
|  | L3 RSRP diff | HOF rate | HO number per UE per second | HOF number per UE per second |
| Legacy HO | NA | 0.094 | 0.059 | 0.0056 |
| Sample and hold | 0.124 | 0.089 | 0.062 | 0.0056 |
| AI | 0.059 | 0.094 | 0.063 | 0.0059 |

Table 1: System level result of temporal domain case B, MRRT=0.5

| FR1 Temporal domain case B, UE speed = 30 km/hr, MRRT = 0.8 | Intermediate KPI | System level KPI |  |  |
| --- | --- | --- | --- | --- |
|  | L3 RSRP diff | HOF rate | HO number per UE per second | HOF number per UE per second |
| Legacy HO | NA | 0.094 | 0.059 | 0.0056 |
| Sample and hold | 0.293 | 0.087 | 0.071 | 0.0062 |
| AI | 0.241 | 0.068 | 0.072 | 0.005 |

Table 2: System level result of temporal domain case B, MRRT=0.8

We observe that the HOF rate is quite low (less than 1%) in the FR1(UE speed=30km/hr) case. For MRRT=0.8, similar results are observed. Compared to the sample and hold, the AI approach has a similar # of HO attempts, but the total # of HOF reduces from 20 to 16. This is because, for MRRT=0.8, the sample and hold approach will repeat each real measurement 4 times which may lead to a wrong HO decision. However, in such low-speed FR1 scenario, the overall HO performance is still quite good, and the margin for AI improvement is limited.

Observation 1: The HOF happens with low probability for the setting given in temporal domain case B.

Observation 2: The AI approach can reduce the measurement overhead without significant system-level performance loss when MRRT=80%.

Proposal 2: To capture the observation that AI approach can reduce the measurement overhead without significant system-level performance loss when MRRT=80% for indirect event prediction temporal domain case B in the TR.

Simulation Results of Case A

In our AI-based approach for indirect event prediction, we utilize 5 previously available time samples measurement results (totalling 400ms, with each sample interval 80ms), i.e., L3 cell RSRP with sliding window, to predict the subsequent 5 future time samples (totalling 400ms, with each sample interval 80ms) cell level measurement result.

Table 1 present the summarized intermediate evaluation results for TTT setting of 160ms and 320ms at a speed of 120km/h. The results indicate that the average L3 cell RSRP difference (between prediction and genie-aided label) is smaller than 1 dB. Furthermore, the RSRP difference at the last predicted point is larger than the average of the predicted RSRP differences.

| TTT(ms) | F1 score | Precision | Recall | Average L3 cell RSRP difference (dB) | Last predicted point L3 cell RSRP difference (dB) |
| --- | --- | --- | --- | --- | --- |
| 160 | 0.915 | 0.964 | 0.87 | 0.64 | 1.22 |
| 320 | 0.874 | 0.941 | 0.816 | 0.64 | 1.22 |

Table 1 Intermediate results for temporal case A of indirect event prediction

Observation 3: For Measurement event prediction case A, the F1 score can exceed 0.85 when the TTT is set to either 160ms or 320ms.

Table 2 gives system level results for temporal case A of indirect event prediction. In the HO modelling of option 3, the HO command is sent in advance based on the prediction results, and the most intuitive system gain brought by AI is the increase in the time or probability of staying in the best cell, without increasing the occurrence of HOF and ping-pong or short ToS.

|  | TTT (ms) | HOF rate | Total number of HO attempts per UE per second | Total number of HOF per UE per second | Short ToS | Average ToS (ms) | Ping-pong | Camp on best cell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI | 320 | 26.6 | 0.225 | 0.06 | 46 | 4451 | 32 | 0.66 |
| Legacy | 320 | 23.4 | 0.266 | 0.0625 | 51 | 3726 | 30 | 0.62 |

Table 2 System level results for temporal case A of indirect event prediction

The results indicates that AI improves the efficiency of handover attempts, increases average ToS, reduces ping-pong/short ToS, and increases the probability of camping on the best cell meanwhile reduce the number of HOF per UE per second compared with legacy HO procedure, enhancing overall network performance and user experience. Therefore, we believe that AI is beneficial in this case for indirect event prediction temporal domain case A.

Observation 4: For indirect event prediction temporal domain case A with HO modelling option 3, the AI-based HO can provide ~ 11% gain (in terms of camp on best cell rate) and ~19% gain (in terms of average ToS) and 12% reduce of the total number of HOF per UE per compared to the legacy HO approach.

Proposal 3: To capture the observation that AI approach can improve the system-level performance of HO procedure for indirect event prediction temporal domain case A with HO modelling option 3 in the TR.

Direct event prediction

Labelling and output for Direct Event Prediction

For direct event prediction, RAN2 has agreed that the model output is the probability of event occurrence within a time window. And the occurrence window is equivalent to the prediction window.

Figure 1: occurrence window of direct prediction method

When training the direct event prediction model, the labelling is acquired based on legacy HO procedure with the agreed A3 event configuration. There are 2 possible options for labelling:

- Option 1: If no event happens, the time sample labelled as 0; otherwise, the time sample labelled as 1. The direct event prediction is a binary classification issue.

- Option 2: If there is no event happens, the time sample labelled as 0; otherwise, the time sample labelled as 1~N (target cell ID, e.g., N=21). The direct event prediction is a multiple-label classification issue.

Considering we utilize the method that UE starts the AI inference at t0 when the entering condition is fulfilled. We can get the target cell ID based on actual measurement at t0. Since the prediction of the target cell ID will introduce extra complexity, we will take option 1 to evaluate the direct event prediction.

Proposal 4: Direct event prediction can be considered as a binary classification issue, i.e. , if the event happen, the time sample is labelled as 0;otherwise, the time sample is labelled as 1.

The difference between indirect and direct is the model output and the usage of the output. For indirect, the model output is RSRP value, and we use the predicted RSRP to derive whether the event will happen. While the direct event prediction directly output is whether the event will happen. If we use probability of occurrence window as output, we always need a threshold (e.g., Pt) to compare with the model output probability (P0) to decide whether the event will happen (i.e., if Pt > P0, the event is predicted to be happen; otherwise, the event is predicted to not happen). Moreover, it is important to note that any chosen threshold would directly influence the intermediate results.

Observation 5: If the model output is the probability of event occurrence within the prediction window, there needs a threshold to determine the whether the event will happen. Moreover, the chosen threshold would directly influence the intermediate results.

Given the diverse implementations of channel models and AI algorithms across different companies, it’s challenging to establish a unified threshold that would be universally applicable, so RAN2 allows company to report probability threshold in previous meeting.

However, an easier approach is for the model to directly output a flag for each time instance within prediction window. In this way, there is no need to report the threshold, as the probability and threshold are obtained and compared by the AI model's internal calculation. This simplifies the process and ensures consistency in the evaluation of event predictions.

We take the model output the flag as AI implementation starting point. With cluster based RSRP (including serving cell and neighbouring cell) RSRP within OW as input, UE performs AI inference when the A3 event is fulfilled at t0, the model output flag of event happens for each time instance of PW. For example, we assume the prediction window = 320ms, there will has 4 time instances in PW, if the event is predicted to happen at the third time instance, so the model output would be [0, 0, 1, 0].

Figure 2: Option 2 that model output with flag within prediction window.

Proposal 5: In addition to the company report results with a probability threshold, the adoption of a model that directly outputs the flag at every time instance allows companies to report results without the need to specify/report a probability threshold.

Unified Simulation Assumptions and Derivation of Intermediate KPIs for indirect and direct event prediction

With model directly output flag (0/1) indicating whether the event happens at each time instance of PW during AI implementation, we can pinpoint the predicted exact timing of event happen. Then the max ETD defined for indirect event prediction can be reused to derive the intermediate KPIs.

Figure 3: Reuse the max ETD to derive intermediate KPIs.

Observation 6: With the model output is the flag (i.e., 0 for no event happen, 1 for event happen)) for direct event prediction, indirect event prediction and direct even prediction can have an unified simulation assumption and derive of metrics.

Proposal 6: For the direct event prediction, to conside the model output the flag (i.e., 0 for no event happen, 1 for event happen) of each time instance within the prediction window.

Preliminary Results for Direct Event Prediction for Temporal domain case A of System-Level Simulation

Following simulation assumptions are used for direct measurement event prediction case A:

Table 5.3.1-1: Additional simulation assumptions for measurement event prediction based

on intra-frequency temporal domain case A

| Parameters | Value |
| --- | --- |
| A3 event offset (dB) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 60/120 |
| OW length (ms, *) | 320 |
| PW length (ms,**) | 320 |
| Max ETD (ms, *) | 80 |

In our AI-based approach for direct event prediction, we utilize 5 previously available time samples measurement results (totalling 320ms, with each sample interval 80ms), i.e., L3 cell RSRP with sliding window, to predict the subsequent 5 future time samples (totalling 320ms, with each sample interval 80ms) cell level measurement result. Table 1 present the summarized intermediate evaluation results for TTT setting of 320ms at a speed of 60/90/120km/h. The results indicate that the F1 score are above 0.85 with speed 60km/h and 120km/h, and lower speed 60 km/h has slightly better intermediate results than higher speed 90km/h and 120km/h.

|  | Precision | Recall | F1 score |
| --- | --- | --- | --- |
| 60km/h | 0.917 | 0.846 | 0.880 |
| 120km/h | 0.902 | 0.833 | 0.866 |

Table 1 Intermediate results for temporal case A of direct event prediction

Observation 7: The F1 score are above 0.85 with speed 60km/h and 120km/h, and lower speed 60 km/h has slightly better intermediate results than higher speed 120km/h.

We further present system-level performance results. For the system-level simulation of the assumption on UE/NW behaviour with AI prediction for case A, we take option 3 as HO modelling procedures. For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and the HO command is sent after HO preparation, the target cell is determined based on the actual measurement when the event condition is fulfilled.

In the HO modelling of option 3, the HO command is sent in advance based on the prediction results, and the most intuitive system gain brought by AI is the increase in the time or probability of staying in the best cell, without increasing the occurrence of HOF and ping-pong or short ToS.

| Speed | Method | HOF rate (%) | HOF times per UE per sec | HO times per UE per sec |
| --- | --- | --- | --- | --- |
| 60km/h | AI (option 3) | 11.4 | 0.0325 | 0.285 |
|  | Legacy | 20.94 | 0.0498 | 0.238 |
| 120km/h | AI (option 3) | 13.89 | 0.0328 | 0.236 |
|  | Legacy | 23.4 | 0.0622 | 0.266 |

Table 2 System level results for temporal case A of direct event prediction

We can observe from the results that with AI prediction and HO modelling option 3, HOF times per UE per second reduced ~30% for 60km/h and ~45% for 120km/h, the HO attempts per UE per second increased for lower speed(60km/h) and reduce for higher speed(120km/h).

Observation 8: For direct event prediction, with AI prediction and HO modelling option 3, HOF number per UE per second reduced by~30% for 60km/h and ~45% for 120km/h, the HO attempts number per UE per second increased for lower speed(60km/h) and reduce for higher speed(120km/h) compared to the legacy HO procedure.

Furthermore, we conducted a comparative analysis of system-level simulation results for both indirect and direct methods under same settings (TTT=320ms, speed=120km/h). we observe that both direct and indirect event prediction methodologies demonstrate performance gains over the legacy HO. Notably, the direct event prediction method exhibits more pronounced improvements compared to indirect prediction.

|  | AI/legacy | HOF rate (%) | HOF times per UE per sec | HO times per UE per sec |
| --- | --- | --- | --- | --- |
| Indirect event prediction | AI (option 3) | 26.6 | 0.06 | 0.225 |
| Direct event prediction | AI (option 3) | 13.89 | 0.0328 | 0.236 |
|  | Legacy | 23.4 | 0.0622 | 0.266 |

Observation 9: Both direct event prediction and indirect prediction methodologies demonstrate a reduction in HOF number per UE per second when compared to the legacy approach at the system level performance.

Observation 10: The direct prediction method has more significantly gains compared to indirect on HO KPIs.

Our analysis suggests that the superior performance of direct event prediction can be attributed to its methodology for determining the target cell. In the direct event prediction implementation, the target cell is identified based on actual measurements taken at the moment when the A3 event entering condition is satisfied. This approach proves to be more accurate compared to the indirect event prediction method, which relies solely on predicted RSRP values for target cell determination. The distinction between direct and indirect event prediction methodologies pertains solely to UE implementation and does not impose additional implications on the specifications. Whether the UE anticipates event occurrences based on predicted RSRP values or flags, and subsequently reports Measurement Reports (MRs) in advance, these processes remain transparent to the network.

Proposal 7: To capture the observation that AI improves the system level performance compared to the legacy HO for direct event prediction temporal domain case A in the TR.

Proposal 8: To capture the observation that direct event prediction provide more gain on the system level performance compared to indirect event prediction for temporal domain case A in the TR.

Conclusion

Observation

Indirect event prediction for Case B

Observation 1: The HOF happens with low probability for the setting given in temporal domain case B.

Observation 2: The AI approach can reduce the measurement overhead without significant system-level performance loss when MRRT=80%.

Indirect event prediction for Case A

Observation 3: For Measurement event prediction case A, the F1 score can exceed 0.85 when the TTT is set to either 160ms or 320ms.

Observation 4: For indirect event prediction temporal domain case A with HO modelling option 3, the AI-based HO can provide ~ 11% gain (in terms of camp on best cell rate) and ~19% gain (in terms of average ToS) and 12% reduce of the total number of HOF per UE per compared to the legacy HO approach.

Direct event prediction

Observation 5: If the model output is the probability of event occurrence within the prediction window, there needs a threshold to determine the whether the event will happen. Moreover, the chosen threshold would directly influence the intermediate results.

Observation 6: With the model output is the flag (i.e., 0 for no event happen, 1 for event happen)) for direct event prediction, indirect event prediction and direct even prediction can have an unified simulation assumption and derive of metrics.

Observation 7: The F1 score are above 0.85 with speed 60km/h and 120km/h, and lower speed 60 km/h has slightly better intermediate results than higher speed 120km/h.

Observation 8: For direct event prediction, with AI prediction and HO modelling option 3, HOF number per UE per second reduced by~30% for 60km/h and ~45% for 120km/h, the HO attempts number per UE per second increased for lower speed(60km/h) and reduce for higher speed(120km/h) compared to the legacy HO procedure.

Observation 9: Both direct event prediction and indirect prediction methodologies demonstrate a reduction in HOF number per UE per second when compared to the legacy approach at the system level performance.

Observation 10: The direct prediction method has more significantly gains compared to indirect on HO KPIs.

Proposal

UE inference operation

Proposal 1: To evaluate the performance of measurement event prediction with following two options of UE inference operation:

- Option 1: UE does continuous inference, predicting if event is about to happen in the future/the prediction window.

- Option 2: UE starts AI inference when the entering condition of event is fulfilled, the UE predicts if the occurring event is valid in the future/ the prediction window. If the entering condition of the occurring event is remain satisfied in the future prediction window, the occurring is considered as valid, otherwise, it is invalid.

Indirect event prediction for case B

Proposal 2: To capture the observation that AI approach can reduce the measurement overhead without significant system-level performance loss when MRRT=80% for indirect event prediction temporal domain case B in the TR.

Indirect event prediction for Case A

Proposal 3: To capture the observation that AI approach can improve the system-level performance of HO procedure for indirect event prediction temporal domain case A with HO modelling option 3 in the TR.

Direct event prediction

Proposal 4: Direct event prediction can be considered as a binary classification issue, i.e. , if the event happen, the time sample is labelled as 0;otherwise, the time sample is labelled as 1.

Proposal 5: In addition to the company report results with a probability threshold, the adoption of a model that directly outputs the flag at every time instance allows companies to report results without the need to specify/report a probability threshold.

Proposal 6: For the direct event prediction, to conside the model output the flag (i.e., 0 for no event happen, 1 for event happen) of each time instance within the prediction window.

Proposal 7: To capture the observation that AI improves the system level performance compared to the legacy HO for direct event prediction temporal domain case A in the TR.

Proposal 8: To capture the observation that direct event prediction provide more gain on the system level performance compared to indirect event prediction for temporal domain case A in the TR.

Reference

[1] R2_129_ChairNotes_25-02-22_21-00_eom