3GPP TSG-RAN WG2 Meeting #129bis                           R2-2502109

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.1.3

Source:	Mediatek Inc.

Title:	Further Discussion on Network-Side Data Collection

Document for:	Discussion, Decision

Introduction

In the RAN2#129 meeting, following agreements were reached specifically for network-side training data collection pertaining to the use case of AI BM.

All agreements for NW side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

This paper delves into the specifics of data logging and reporting for data collection aimed at network-side model training, discusses the handling of logged data and measurements during RRC state transitions, and discusses the data content for both network-side and UE-side data collection.

Discussion

Configuration and Logged Data Handling

Normal Operation in RRC_CONNECTED

In RAN2#126 meeting, it was agreed that Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model. In the following meetings, RAN2 discussed how to support data logging and reporting on top of it and agreed that on-demand reporting of the logged data will be specified.

For logged MDT, the UE maintains only one RAT-specific logged measurement configuration at a time. When the network provides a new configuration, it completely replaces any previously configured logged measurement configuration. This means that only the new configuration will take effect. Additionally, any logged measurements associated with the previous configuration will be cleared. It is the network's responsibility to retrieve any relevant data before providing a new configuration.

Observation 1: In logged MDT, the UE maintains only one logged measurement configuration. It is the network's responsibility to retrieve any relevant data before providing a new configuration.

The same principle should be applied to data collection for network-side model training. Firstly, networks typically conduct data collection targeted to a specific RAN configuration or scenario, such as set A or set B configuration for AI BM use cases from large amount of UEs. Secondly, it simplifies the management of measurement configurations and logged data for both the network and the UEs. Otherwise, both the UE and the network would need to maintain the association between the logged measurement configuration and the logged data, and the network would need to differentiate under which logged measurement configuration the reported data is targeted. Thirdly, considering that there is no latency requirement for data collection for model training, supporting the reconfiguration of the measurement configuration before retrieving the logged data offers little benefit. Therefore, for data collection configuration for network-side model training, only one logged measurement configuration should be in effect. It should be noted that multiple measurement resources and measurement reports can be configured in the logged measurement configuration, such as CSI-MeasConfig.

Observation 2: It simplify the management of the data collection configuration and the logged data if the UE maintains only one logged measurement configuration for the data collection for network-side model training:

- When the network provides a new logged measurement configuration, it completely replaces any previously configured measurement configuration.

- When the network provides a new logged measurement configuration, any logged measurements associated with the previous configuration will be cleared.

- It’s up to the network to retrieve any relevant data before providing a new configuration for data collection.

In logged MDT, the release of a logged measurement configuration in the UE is achieved either by configuration replacement when the configuration is overwritten or by configuration clearance when a duration timer stops or expires. For data collection for network-side model training, since the data collection is conducted exclusively in the RRC_CONNECTED state and any control over the data collection configuration and logged data can be managed through network signaling, there is no need for a duration timer to handle the data collection operation. The duration timer was originally intended to control UE data logging in the IDLE/INACTIVE state. Therefore, explicit release of the measurement configuration needs to be supported for data collection for network-side model training. When the measurement configuration is released, the data associated to the configuration is also discarded.

Observation 3: Since the data collection for network-side model training is conducted exclusively in the RRC_CONNECTED state, any control over the data collection configuration and logged data can be managed through network signaling.

Proposal 1:The UE maintains only one logged measurement configuration for data collection for network-side model training. When the network provides a new logged measurement configuration, it completely replaces any previously configured logged measurement configuration.

Proposal 2: When the network provides a new logged measurement configuration for data collection for network-side model training, any logged measurements associated with the previous configuration will be discarded.

Proposal 3: It’s up to the network to retrieve any relevant data before providing a new logged measurement configuration for data collection for network-side model training.

Proposal 4: The logged measurement configuration can be released explicitly via RRCReconfiguration message by the network. The UE discards the associated data when the measurement configuration is released.

Handover

In the RAN2#129 meeting, it was agreed that the UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed.

There are possible scenarios in which the network may want to provide new configurations or release the configuration for the UE to collect data, such as:

- Targeting new scenarios or configurations: The target cell may want to collect data for new scenarios or configurations to improve performance in different environments, conditions, or network configurations.

- Inability of target gNB to continue data retrieval and data forwarding: If the target gNB is unable to continue data retrieval or to forward the data to the data collection entity at the network side (e.g. source gNB or OAM), the UE may be asked to discard the logged data.

Observation 4: There are scenarios where the target cell may need to provide new configurations or release existing ones for the UE to collect data. For example, the target cell may aim to collect data for new scenarios or configurations, or the target gNB may be unable to continue data retrieval and forwarding, requiring the UE to discard the logged data.

During the handover procedure, the UE remains in the RRC_CONNECTED state. The principles described in proposals 1, 2, 3, and 4 can still be applied, regardless of the scenarios mentioned above. It is assumed that the source cell will coordinate with the target cell for logged data retrieval. The network/target cell always has the flexibility to decide whether to continue retrieving the data or not after handover.

- Case 1-if the target cell can retrieve remaining logged data: No new logged measurement configuration should be provided in the HO command. In this case, the UE retains the logged data during handover and sends a data availability indication in the ReconfigurationComplete message to the target cell, which will retrieve the logged data later.

- Case 2-if the target cell intends to initiate data collection for specific configurations/scenarios/conditions: A new logged measurement configuration can be provided in the HO command. The UE replaces any previously configured measurement configuration with the new configuration and discards any logged measurements associated with the previous configuration.

- Case 3-if the target cell cannot retrieve or forward logged data from the UE: In abnormal cases where the target cell cannot retrieve or forward the logged data to the data collection entity at the network side (e.g., source gNB or OAM), the logged measurement configuration can be indicated to be released in the HO command. The UE should then discard the logged data.

Proposal 5: RAN2 assumes that the source gNB and the target gNB will coordinate the logged measurement configuration during the HO preparation phase. The specific details of this coordination are to be determined by RAN3.

Proposal 6: If no new logged measurement configuration is provided in the HO command, the UE retains the data as long as the existing logged measurement configuration is not released and the UE sends the data availability indication in the RRCReconfigurationComplete message.

Proposal 7: If a new logged measurement configuration is provided in the HO command, the UE replaces the old configuration with the new one and discards any previously logged data associated with the old configuration.

Proposal 8: If the existing logged measurement configuration is released in the HO command, the UE discards the logged data.

RRC Re-establishment

When the UE is in RRC_CONNECTED state and endures failures, e.g. RLF or handover failure, the UE will initated RRC reestablishement procedure. In this case, the UE can send data availability indication in the RRCReestablishmentComplete message to the cell where the RRC connection is sucessfully re-established. Meanwhile, the UE keeps the original logged measurement `configuration and the relevant logged data associated to the original configuration. In this case, the network can decide whether to retrieve the stored data from the recovered UE or release the logged measurement configuration and the logged data.

However, if the UE moves to the RRC_IDLE due to fail of RRC reestablishment, the UE should discard the log measurement configuration as well as the logged data.

Proposal 9: When the UE initiates the Re-establishment procedure, the UE keeps the original logged measurement configuration and the relevant data.  The UE sends the data availability indication to the network in the RRCReestablishComplete message if RRC connection is successfully re-established.

Proposal 10: If the UE transitions to RRC_IDLE due to a failed RRC reestablishment, it discards both the logged measurement configuration and the logged data.

State Transition

In logged MDT, under normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE. Given that on-demand requests from the network are supported for data collection for model training, the network should adhere to the same requirement, ensuring no residual data or logs are left behind when transitioning the UE to the RRC_INACTIVE or RRC_IDLE state. It is the network's responsibility to collect any stored data/logs before transmitting the RRCRelease message. In abnormal case that there is residual data or logs left when receiving the RRCRelease message, the UE discards the stored data or logs.

From complexity point of view, we also don’t see much benefits to make the UE store the measurement configuration and the logged the data when the UE is RRC_INACTIVE mode, since the collected data may targets to specific site/area. Considering the UE mobility, the UE may moves to another unrelavant area and the measurement configuration is not valid anymore. In order to simplify the data collection operation, the same operation is applied to RRC_IDLE and RRC_INACTIVE state.

Proposal 11: In normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE, which moves the UE from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state.

Proposal 12: When the UE receives RRCRelease message which moves the UE from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state, the UE releases the configuration for data collection and discard the logged data.

Data Logging

In RAN2#129meeting, it was agreed to support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.

For the use case of AI-BM, only the measurement of serving cell measurement needs to be considered, therefore, only Event A1 (Serving becomes better than threshold) and Event A2 (Serving becomes worse than threshold)  can be configured for the purpose of triggering measurement logging at the UE.

Proposal 13: Only A1 (Serving becomes better than threshold) and A2 (Serving becomes worse than threshold) events can be configured optionally to the UE for data logging.

If A1 or A2 event is configured for data logging, when the UE starts and stops the data logging needs to be discussed. One straightforward approach is that the the UE starts data logging when the entering condition of the event is fulfilled and stops data logging when the leaving condition is fulfilled. This ensures consistency in UE's measurement behavior if event-triggered measurement and reporting are configured. However, different RRM measurement and report that the UE removes the remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList, the UE needs to keep the logged measurement.

Proposal 14: If the L3 measurement event (A1, A2) is configured by the network, the UE starts data logging when the entering condition is fulfilled and stops data logging when the leaving condition is fulfilled.

Data Report

In RAN1, the discussion on data content and the corresponding report configuration primarily focuses on inference rather than model training.

In the RAN1 AI-BM FL summary[1], the following types of data content for network-side model training via higher layer signaling were discussed during the RAN1#119 and #120 meeting for BM-Case 1, but no agreement was reached. RAN1 had intended to send a LS to RAN2 to inform the data content for network-side data collection. There has been no discussion on BM-Case 2 yet. It is expected that RAN1 will continue the discussion, and the down-selection of different types of data content may take place.

Send LS to RAN 2, to inform RAN 2 that from RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, the following three types of contents for each instance can be considered: - Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE - Type 2: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information of K beams based on some resources (for Set A) configured to UE - Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE - Wherein the K beams are the beams with the largest K L1-RSRPs based on the resources (for Set A) in one instance - K is configured by NW.

The reason for discussing the three types of data content is that for AI-BM, generally only two types of AI algorithms are used: regression and classification. In both cases, the inputs are the L1-RSRPs of Set B.

- For regression algorithms, the outputs are the L1-RSRPs of Set A, corresponding to Type 1 data content.

- For classification algorithms, the outputs are the beam information (beam-IDs) of K beams (on resources for Set A), corresponding to Type 2 data content.

- If both regression and classification algorithms are used, Type 3 data content will be employed.

Since RAN2 already agreed that for data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress. Based on RAN1 discussion, there is no other data needs to be collected for AI-BM.

One advantage of Type 1 is that Set A/Set B information is not required. Therefore, the data content can be used to train AI models with different Set A/Set B configurations. Additionally, the data content can be utilized for both model training and model monitoring. However, the drawback is that it results in significant signaling overhead. The advantage of Type 2 is that it significantly reduces the signaling overhead, however, it requires the provision of Set A/Set B information.

Observation 5: For content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, Type 2/3 data content requires Set A/Set B configuration information, while Type 1 data content does not. However, RAN1 has not reached a conclusion regarding the data content.

Proposal 15: For the data content for both NW-side/UE-sdie model training, RAN2 waits for RAN1’s progress on BM-Case 1 and BM-Case 2.

Regarding whether to utilize the exsiting low priority SRB (e.g. SRB4) or a new SRB to report the logged data, a new SRB is preferred. A new SRB provides improved isolation, preventing interference with other signaling processes and leading to more flexible data transmission. It offers better scalability and extendability, which can be used for different use cases.

Proposal 16: A new SRB is used for logged data reporting for the network-side data collection.

Conclusion

Normal Operation in RRC_CONNECTED

Observation 1: In logged MDT, the UE maintains only one logged measurement configuration. It is the network's responsibility to retrieve any relevant data before providing a new configuration.

Observation 2: It simplify the management of the data collection configuration and the logged data if the UE maintains only one logged measurement configuration for the data collection for network-side model training:

- When the network provides a new logged measurement configuration, it completely replaces any previously configured measurement configuration.

- When the network provides a new logged measurement configuration, any logged measurements associated with the previous configuration will be cleared.

- It’s up to the network to retrieve any relevant data before providing a new configuration for data collection.

Observation 3: Since the data collection for network-side model training is conducted exclusively in the RRC_CONNECTED state, any control over the data collection configuration and logged data can be managed through network signaling.

Proposal 1:The UE maintains only one logged measurement configuration for data collection for network-side model training. When the network provides a new logged measurement configuration, it completely replaces any previously configured logged measurement configuration.

Proposal 2: When the network provides a new logged measurement configuration for data collection for network-side model training, any logged measurements associated with the previous configuration will be discarded.

Proposal 3: It’s up to the network to retrieve any relevant data before providing a new logged measurement configuration for data collection for network-side model training.

Proposal 4: The logged measurement configuration can be released explicitly via RRCReconfiguration message by the network. The UE discards the associated data when the measurement configuration is released.

Handover

Observation 4: There are scenarios where the target cell may need to provide new configurations or release existing ones for the UE to collect data. For example, the target cell may aim to collect data for new scenarios or configurations, or the target gNB may be unable to continue data retrieval and forwarding, requiring the UE to discard the logged data.

Proposal 5: RAN2 assumes that the source gNB and the target gNB will coordinate the logged measurement configuration during the HO preparation phase. The specific details of this coordination are to be determined by RAN3.

Proposal 6: If no new logged measurement configuration is provided in the HO command, the UE retains the data as long as the existing logged measurement configuration is not released and the UE sends the data availability indication in the RRCReconfigurationComplete message.

Proposal 7: If a new logged measurement configuration is provided in the HO command, the UE replaces the old configuration with the new one and discards any previously logged data associated with the old configuration.

Proposal 8: If the existing logged measurement configuration is released in the HO command, the UE discards the logged data.

RRC Re-establishment

Proposal 9: When the UE initiates the Re-establishment procedure, the UE keeps the original logged measurement configuration and the relevant data.  The UE sends the data availability indication to the network in the RRCReestablishComplete message if RRC connection is successfully re-established.

Proposal 10: If the UE transitions to RRC_IDLE due to a failed RRC reestablishment, it discards both the logged measurement configuration and the logged data.

State Transition

Proposal 11: In normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE, which moves the UE from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state.

Proposal 12: When the UE receives RRCRelease message which moves the UE from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state, the UE releases the configuration for data collection and discard the logged data.

Data logging

Proposal 13: Only A1 (Serving becomes better than threshold) and A2 (Serving becomes worse than threshold) events can be configured optionally to the UE for data logging.

Proposal 14: If the L3 measurement event (A1, A2) is configured by the network, the UE starts data logging when the entering condition is fulfilled and stops data logging when the leaving condition is fulfilled.

Data Report

Observation 5: For content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, Type 2/3 data content requires Set A/Set B configuration information, while Type 1 data content does not. However, RAN1 has not reached a conclusion regarding the data content.

Proposal 15: For the data content for both NW-side/UE-sdie model training, RAN2 waits for RAN1’s progress on BM-Case 1 and BM-Case 2.

Proposal 16: A new SRB is used for logged data reporting for the network-side data collection.

Reference

[1] R1-2410892, FL summary #5 for AI/ML in beam management,Samsung (Moderator)

[2] R1-2501595, FL summary #6 for AI/ML in beam management