3GPP TSG-RAN WG2 Meeting #R2-129bis		R2-2502122 Wuhan, China, Apr 07-11, 2025

Agenda Item:	8.6.2

Source: 	Rakuten

Title:  	Discussion on Inter-CU LTM

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

The WID objective mentions that for inter-CU LTM Rel-18 intra-CU LTM is the baseline. This implies that any enhancements for the inter-CU LTM use cases are going to be on top of the Rel 18 framework.

In this contribution, we describe and discuss some aspects involved in extending LTM to support inter-CU use-cases. We also propose a way forward for some of these.

# Discussion

## Support for mTRP with inter-CU LTM

RAN2#129 made the below agreement,

- Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

We think inter-CU LTM with MIMO 2TA should be supported, as Rel 18 LTM was also supported with mTRP.

However, in the mTRP case, the unified TCI state MAC CE could activate the two TCI states (one for TRP1, the other for TRP2). In addition, RRC configures the applied TCI state per TRP (first/second/both/none)

-	PDCCH could configured by first TCI state

-	PDSCH could be configured by second TCI state

In the context of inter-CU LTM, it is not clear which TCI state is mapped to the current beam, the PDCCH TCI state or the PDSCH TCI state.

R19 MIMO only focusses on the sTRP scenario but there is no such restriction in inter-CU LTM. In addition, R18 LTM already supports the mTRP. So it is reasonable the Rel-19 LTM also support the mTRP with LTM.

So, we propose a RAN2 discussion on this. The way forward from our side is that:

- RAN2 discusses the mTRP scenario for inter-CU LTM where mTRP means the UE in the source cell is configured with mTRP and two separate TCI states could be activated.

- RAN2 asks RAN1 which beam is used for the current beam (i.e. either indicated PDCCH beam or PDSCH beam if both beams are different)

RAN2 agrees to support the mTRP case with inter-CU LTM.

RAN2 asks RAN1 regarding which beam should be considered as current beam i.e the beam associated with the PDCCH or the one associated with PDSCH.

## Intra-gNB inter gNB-CU-UP relocation in LTM

In Rel 18, it was agreed that security key update is not supported for intra-gNB LTM scenarios.

The below agreement was made in RAN2 #125-bis which indicates that security key refresh is supported for inter-CU LTM.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

The scenario described in [5] is gNB-CU-UP change inside the same gNB. In this scenario, the PDCP anchor of the source CU-UP and the target CU-UP are different. Hence this intra gNB use case may also require security key refresh. In this section, we describe the inter gNB-DU LTM scenario and discuss the security aspects associated with it.

The most recent Reply LS from SA3 (S3-244316) for Rel 19 inter-CU LTM with security key change is given below:

# 1 Overall description SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases. SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

From the above, it is clear that whenever there is a PDCP anchor change, security key refresh is required.

Figure 2: Inter gNB-DU LTM HO with intra gNB gNB-CU-UP relocation

Let us consider the above figure which depicts a gNB with the following elements,

  - A gNB having a gNB-CU-CP and two gNB-CU-UPs serving a subset of gNB-DUs.

  - gNB-CU-UP1 serves gNB-DUs, DU1..DU5.

  - gNB-CU-UP2 serves gNB-DUs, DU6..DU10.

While the gNB internal architecture is not relevant to RAN2, it is interesting to notice here that when a UE undergoes LTM cell switch from a cell belonging to DU5 to a cell belonging to DU6, it also undergoes intra gNB inter-gNB-CU-UP relocation. When a UE undergoes intra gNB gNB-CU-UP relocation, it changes the PDCP anchor point and hence there is a need to refresh the security keys. This implies that new security keys generated have to be delivered to the UE. This also implies that the gNB-CU-UP has to be included while preparing such an LTM candidate cell.

Security key refresh is required when a UE undergoes an inter gNB-DU mobility that results in an intra gNB gNB-CU-UP relocation.

Security Key refresh is supported for inter gNB-DU LTM, when gNB-CU-UP relocation occurs inside the same gNB.

RAN2 discusses and agrees a way forward for the security impacts due to intra gNB gNB-CU-UP relocation scenario and sends an LS to RAN3 to discuss and address the same.

## Radio Resource aspects of inter-CU LTM

## Acquiring candidate cell TA and delayed resource reservation

The lead time (duration between LTM HO preparation and execution) in an LTM cell switch could be unpredictably long. Secondly, not all prepared target cell configurations end up in LTM cell switches. Radio resources are scarce and considering that a UE can be configured with a maximum of 8 LTM candidate cells (including intra-CU and inter-CU), resource reservation (including RACH resources) is always going to be an issue. Reservation of radio resources in all the configured LTM candidate cells for all UEs leads to sub-optimal performance and causes a huge overhead on the network.

Since UE is expected to perform RACH to acquire timing advance (TA) from the candidate/target cell before the LTM cell switch with the objective of enabling a RACH-less LTM cell switch, resource reservation is a serious issue considering the framework of LTM. For a successful LTM mass deployment, optimization of resource reservation is of utmost importance.

For a successful LTM deployment, optimization of radio resource reservation is crucial, especially considering the potential high lead times before LTM.

Based on the UE’s L1 measurements, the serving gNB-DU may ask the UE to perform RACH with the candidate/target cell to acquire UE’s candidate/cell TA. Hence, UE performing RACH with the candidate/target cell can be considered to indicate a likelihood of UE undergoing LTM HO to the given target cell.

To address the resource reservation issues, the following could be considered - at the time of preparing an LTM target cell, the gNB-CU could indicate to the candidate/target gNB-DU whether or not the UE is configured to perform RACH based on PDCCH order to acquire TA. Based on this, the target gNB-DU may decide not to reserve all radio resources immediately at the time of LTM candidate/target cell configuration preparation. At this time, the candidate/target gNB-DU may also defer reserving radio resources whose availability is limited.

Based on the UE performing a RACH operation, the candidate/target gNB-DU can anticipate a UE performing LTM cell switch and initiate resource reservation at the candidate/target cell including update of a RACH resource configuration corresponding to the UE’s best beam/beam-group at that point of time.

NOTE: This cannot be assumed to be an implementation-based solution as there are UEs which compute TA based on UE-capability and hence don’t perform RACH for TA acquisition.

This introduces multiple benefits.

- This ensures that the resource reservation at the target gNB-DU is performed very close to the LTM serving cell switch and hence optimized, as the likelihood of an LTM cell switch is strong. This also prevents unnecessary blocking of resources for a longer duration in case of long lead times.

- This also ensures that the right RACH resources corresponding to the best beam/beam-group for the UE can be reserved at the target gNB-DU.

- The UE performing RACH to acquire TA can be a good indicator of the likelihood of an LTM serving cell change. This enables the target gNB-DU to issue a target cell re-configuration (if required) based on the prevalent resource conditions at that time.

UE performing RACH (UL Sync) with the candidate/target gNB-DU, based on PDCCH order issued by the serving gNB-DU can be used as a trigger for delayed resource reservation at the candidate/target gNB-DU.

RAN2 discusses and agrees that using RACH for TA acquisition to the candidate/target gNB-DU is treated as a trigger for delayed resource reservation at the candidate gNB-DU.

## Reconfiguration of candidate cell configuration

The newly reserved radio resources including the new or modified PRACH preamble, resulting in a modified target cell configuration at the candidate/target gNB-DU could be delivered to the UE via the gNB-CU.

However, gNB-DU initiated LTM candidate cell re-configuration is not permitted by RAN3. This is a big limitation considering the fact that there could be changes in resource status and availability at the time of LTM execution, as compared to the preparation. As an example, a simple RACH preamble modification based on beam or beam-group change isn’t possible after the candidate cell configuration has been prepared. Once the candidate cell configuration is prepared, it is either executed or cancelled. This, in our view, is too inflexible and limiting genuine cases requiring re-configuration.

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM

RAN2 has made the above agreement in both Rel 18 and Rel 19 and the solution is not even usable as there is no possibility to (re)configure the CFRA resource for LTM cell switch.

RAN2 sends an LS to RAN3 to re-consider the gNB-DU initiated LTM candidate cell re-configuration in Release 19.

Triggering data forwarding from the serving gNB-DU, based on the L1 measurements requires awareness of target gNB-DU and its associated gNB-CU-UP (to determine whether or not there is a security domain change), which is an overhead. Hence, TA acquisition using UL sync procedure from the UE can also be used by the target gNB-DU to request and trigger downlink data forwarding from the target gNB-CU-UP. This will also ensure on-time data forwarding as it is very close to the LTM cell switch.

UE performing UL Sync with the target gNB-DU, based on PDCCH order issued by the serving gNB-DU can be used by the candidate/target gNB-DU to trigger data forwarding from gNB-CU-UP by the candidate/target gNB-DU.

For the observations 2 and 3 to be realistic, the serving gNB-DU has to restrict sending a PDCCH order for TA acquisition to the UE, only when there is a strong likelihood of an LTM cell switch.

RAN2 sends an LS to RAN3 to discuss the feasibility of target gNB-DU requesting data forwarding from the target gNB-CU-UP.

## Successful completion of a RACH-less Inter-CU LTM

## Signaling successful RACH-less LTM completion

In Release 18, in the context of a successful RACH-less LTM cell switch, the UE indicates a successful LTM cell switch to the network by sending a UL data packet to the target cell. Further, the UE determines successful reception of its first UL data at the network, based on the UE receiving a PDCCH addressing the UE’s C-RNTI in the new serving cell scheduling a new transmission after the first UL data is sent.

This is captured in TS 38.321 in section 5.3.1 and an excerpt is provided below.

Let us consider a scenario where

- Post execution of the RACH-less LTM cell switch, the source gNB-DU sends the RACH preamble and beam information delivered to the UE in the LTM cell switch command (MAC CE), to the target gNB-DU.

- Based on this information, the target gNB-DU schedules a PDCCH addressing the UE’s C-RNTI and provides dynamic UL scheduling grants.

- In parallel, the UE completes a RACH-less LTM cell switch and uses a Configured Grant (CG) to send the first UL data to signal a successful RACH-less LTM cell switch.

In such a scenario, the UE determining that there was a successful reception of the first UL data (sent using CG) at the network based on the dynamic grants being scheduled and concluding that the LTM cell switch is successful may not be correct as the dynamic grants were not scheduled based on the reception of the first UL data. In such a scenario, the first UL data packet sent by the UE was not received successfully, but the gNB allocating dynamic grants for pre-scheduling was successful.

Hence, there is probably a need for the UE to initiate a fresh scheduling request (despite receiving the dynamic grants) and ensure that the gNB has successfully received the UL data. If the scheduling request is answered by the gNB, the UE can conclude that the UL transmission was successful.

UE determining a successful reception of its first UL data at the gNB, based on the dynamic grants being scheduled and concluding the success of the LTM cell switch may not be appropriate as the dynamic grants may not have been scheduled based on the reception of the first UL data.

The UE initiates a fresh scheduling request with the new serving gNB-DU, despite allocation of the dynamic grants, to ensure that the first UL data was successfully received at the gNB.

If the new serving gNB-DU issues dynamic grant corresponding to the new scheduling request, the UE concludes that the UL transmission was successful and the RACH-less LTM is completed successfully.

# Conclusion

We have the following observations:

Observation 1	Security key refresh is required when a UE undergoes an inter gNB-DU mobility that results in an intra gNB gNB-CU-UP relocation.

Observation 2	For a successful LTM deployment, optimization of radio resource reservation is crucial, especially considering the potential high lead times before LTM.

Observation 3	UE performing RACH (UL Sync) with the candidate/target gNB-DU, based on PDCCH order issued by the serving gNB-DU can be used as a trigger for delayed resource reservation at the candidate/target gNB-DU.

Observation 4	UE performing UL Sync with the target gNB-DU, based on PDCCH order issued by the serving gNB-DU can be used by the candidate/target gNB-DU to trigger data forwarding from gNB-CU-UP by the candidate/target gNB-DU.

Observation 5	For the observations 2 and 3 to be realistic, the serving gNB-DU has to restrict sending a PDCCH order for TA acquisition to the UE, only when there is a strong likelihood of an LTM cell switch.

Observation 6	UE determining a successful reception of its first UL data at the gNB, based on the dynamic grants being scheduled and concluding the success of the LTM cell switch may not be appropriate as the dynamic grants may not have been scheduled based on the reception of the first UL data.

We have the following proposals:

Proposal 1	RAN2 agrees to support the mTRP case with inter-CU LTM.

Proposal 2	RAN2 asks RAN1 regarding which beam should be considered as current beam i.e the beam associated with the PDCCH or the one associated with PDSCH.

Proposal 3	Security Key refresh is supported for inter gNB-DU LTM, when gNB-CU-UP relocation occurs inside the same gNB.

Proposal 4	RAN2 discusses and agrees a way forward for the security impacts due to intra gNB gNB-CU-UP relocation scenario and sends an LS to RAN3 to discuss and address the same.

Proposal 5	RAN2 discusses and agrees that using RACH for TA acquisition to the candidate/target gNB-DU is treated as a trigger for delayed resource reservation at the candidate gNB-DU.

Proposal 6	RAN2 sends an LS to RAN3 to re-consider the gNB-DU initiated LTM candidate cell re-configuration in Release 19.

Proposal 7	RAN2 sends an LS to RAN3 to discuss the feasibility of target gNB-DU requesting data forwarding from the target gNB-CU-UP.

Proposal 8	The UE initiates a fresh scheduling request with the new serving gNB-DU, despite allocation of the dynamic grants, to ensure that the first UL data was successfully received at the gNB.

Proposal 9	If the new serving gNB-DU issues dynamic grant corresponding to the new scheduling request, the UE concludes that the UL transmission was successful and the RACH-less LTM is completed successfully.

# References

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RAN2 Chairman Notes R2_127bis_ChairNotes

RAN2 Chairman Notes R2_128_ChairNotes

TS 38.321 NR; Medium Access Control (MAC) protocol specification (Release 18)

RP-241106 : Rel-18 LTM correction for LTM with gNB-CU-UP change