# 3GPP TSG RAN WG2 Meeting #129b      	                           R2-2502127
Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.1.4

Source:	Apple

Title:	Further discussion on UE-side data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

The new Rel-19 SID of AI/ML for air interface Phase 2 (SID RP-243245) was agreed in RAN#106 [1]. The SI objective on UE-side data collection can be found below:

According to RAN#106 discussion, the following potential aspects on UE-side data collection were identified for further study in RAN2 in Rel-19:

Discuss any of the aspects identified in RANP WF

o	Study RAN aspects related to data transfer over UP

o	Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o	Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o	Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o	Discuss on the scalability aspects of CP

Based on our understanding, the above aspects can be classified into data collection configuration and dataset transfer. Thus, we discuss the above aspects according to below order:

- Data collection configuration

  - Remaining issue on configuration of UE side data collection

- Dataset transfer

  - RAN perspective to enable the data transfer to CN domain or OAM domain.

  - NG-RAN involvement in the data transfer, including visibility discussion

  - RAN aspects related to data transfer over UP

  - Scalability aspects of CP

# 2 Discussion

## 2.1 Data collection configuration

RAN2#129 extended the agreements on data collection in AI/ML based beam management to general case [2]:

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

Regarding the FFS on whether to allow the UE to request a suggested data collection configuration, we think it is usful because the UE may want to change the measurement configuration according to its local / internal informaion. For example, the UE may prefer to be configured with CSI-RS resource with smaller size when it is in low battery or memory state.

Observation 1: The UE may want to change the measurement configuration according to its local / internal informaion. For example, the UE may prefer to be configured with CSI-RS resource with smaller size when it is in low battery or memory state.

In RAN2#129 [2], some companies have concern on how the UE generate preferred CSI-RS resource configuraiton. We think the simplest way is that NW can provide candiadate parameter sets of measurement configurations for the UE to evaluate, which is similar to applicability reporting. And each candidate parameter set may include the RS configuration and associated, so that the UE doesn’t need to explcitly report associated ID. Thus, we propose:

Proposal 1: NW may provide several candidate data collection configurations where each includes at least RS configuration and associated ID FFS others. The UE request may indicate its preferred configuration from them.

Then, we discuss what signaling / message is used for UE request. As discussed in Observation 1, we think the intention to introduce UE request message is mainly to reflect UE’s local / internal information. Thus, it should at least support UE initiated signlaing. As UE-side data collection has no strict latency requirement, we think UAI is perfectly matched. Meanwhile, since the UE request is intended to reflect UE’s local information, we think it is up to UE implementation when to send the request. Thus, we propose:

Proposal 2: Introduce UAI message for UE request of data collection configuration. And it is up to UE implementation when to send the request.

Next, we discuss the following FFS of RAN2#128 [3]:

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In our understanding, this FFS is intended to discuss whether it is necessary to introduce NW initiated UE request signaling (e.g. via RRCReconfigurationComplete message as response to data collection configuration in RRCReconfiguartion).

If our Proposal 2 is agreed (i.e. UAI is introduced for UE request), it means the UE is allowed to send the request based on its implementation. Then, if the UE can’t perform data collection based on received configuration, it can send the request via UAI with indication of stopping data collection and suggestion on applicable CSI resource configuration. Thus, we think UAI should be sufficient and there is no strong need to introduce NW initiated UE request signaling on top of UE initiated UAI message. Thus, we propose:

Proposal 3: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

In RAN2#128 [3], some companies proposed some solutions without UE request (e.g. requested by UE-side server). However, we think the discussion on detailed solution is out of RAN2 scope, and it can be left to other WGs.

Proposal 4: On data collection configuration without UE request, RAN2 leaves the detailed solution discussion to other WG.

## 2.2 Data Transfer

In Reply LS to SA2 [4], RAN2 provide the following definition of data transfer:

- Data transfer refers to the sending/transfer of the collected data from the UE to Server for data collection for UE-side model training/OTT server (as capture in Table 7.2.1.3.2-1) in TR38.843.

Based on above definition, we discuss the issues identified in RANP#106.

### 2.2.1 RAN aspects to enable the data transfer to CN domain or OAM domain

In RANP#106, option 1b was precluded. Thus, the data transfer to CN domain is mapped to option 2 and the data transfer to OAM domain is mapped to option 3.

Observation 2: As option 1b was precluded in RANP#106, the data transfer to CN domain is mapped to option 2 and the data transfer to OAM domain is mapped to option 3.

According to the LS from RAN to SA2/SA5 in RANP#106 [5], SA2 will evaluate UP solution of option 2 and SA5 will evaluate UP solution of option 3.

From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements.  Transfer of data over UP for Solution 3 can be studied by SA5

RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer.  RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now.

Thus, it is still not clear whether / how UP solution of option 2 and option 3 can work before SA2/SA5 provide feedback. Furthermore, even if UP solution is adopted, it is also not clear whether RAN is transparent to option 2/3 and whether RAN can differentiate option 2 and option 3. Thus, we think it is difficulty for RAN2 to discuss detailed RAN impacts between option 2 and 3 at this stage, i.e. RAN2 can only discuss potential RAN impacts common to option 2 and option 3. Thus, we propose:

Proposal 5: Before SA2/SA5 provide feedback, RAN2 only discuss potential RAN impacts common to option 2 and option 3.

Meanwhile, please note that it is indicated that RAN will make a final decision on the way forward taking into account SA inputs in the LS to SA2/SA5 [5]. In a simple, RAN do not task RAN2 to discuss solution down-selection.

TSG RAN will make a final decision on the way forward taking into account SA inputs when available.  TSG RAN will have a checkpoint in Q2, 2025

Thus, we don’t think RAN2 need to compare between option 2 and option 3, and their down-selection.

Proposal 6: Before RANP provide guidance, RAN2 don’t compare between option 2 (i.e. transfer to CN domain) and option 3 (i.e. transfer to OAM domain) and discuss their down-selection.

### 2.2.2 NG-RAN involvement in the data transfer

We think it needs to discuss separately for CP solution and UP solution:

CP solution

We discuss the controllability and visibility of CP solution. On controllability, it is common understanding that NG-RAN have the full control because SRB transmission is under NG-RAN control.

Proposal 7: RAN2 conclude that NG-RAN have the full control for dataset transfer over CP solution.

On visibility, it is also common understanding that NG-RAN have full visibility for standardized data because it is ASN.1 coding.

Proposal 8: RAN2 conclude that NG-RAN have full visibility for standardized dataset transfer over CP solution.

UP solution

The existing UP protocol stack is between the UE and UPF. 5GC manages the UP traffic with the granularity of PDU session, and NG-RAN is responsible to map QoS Flow(s) of the same PDU session to DRB(s), which is illustrated in Figure. 1.

Figure. 1 5G UP framework

Thus, we understand that NG-RAN only has the controllability and visibility on configuration of PDU session and QoS Flow. Whether NG-RAN can be aware of UE-side data transfer depends on SA2 design (e.g. whether to introduce AI/ML specific PDU session or AI/ML specific QoS flow).

Observation 3: On UP traffic, existing NG-RAN only has the controllability and visibility on configuration of PDU session and QoS Flow. Whether NG-RAN can be aware of UE-side data transfer depends on SA2 design (e.g. whether to introduce AI/ML specific PDU session or AI/ML specific QoS flow).

Before SA2 provide feedback, we don’t think RAN2 can conclude the level of NG-RAN involvement. Thus, we propose:

Proposal 9: RAN2 wait SA2 feedback before discussion on level of NG-RAN involvement on dataset transfer over UP solution, including its controllability and visibility.

### 2.2.2 RAN aspects related to data transfer over UP

As we discussed in Observation 3, we think that RAN2 need to wait SA2 conclusion on:

- Whether NG-RAN can be aware of AI/ML data transfer

- If NG-RAN is aware of AI/ML data transfer, what is the granularity of management? For example:

  - Alt-1: Introduce AI/ML specific QoS flow

  - Alt-2: Introduce AI/ML specific PDU session

  - Alt-3: Introduce AI/ML specific protocol stack (e.g. data plane)

Thus, we propose:

Proposal 10: On data transfer solution over UP, RAN2 wait SA2 conclusion on whether/how NG-RAN can be aware of AI/ML dataset transfer (e.g. via AI/ML specific QoS flow, or AI/ML specific PDU session, or AI/ML specific protocol stack, etc.) to configure DRB accordingly.

### 2.2.3 Scalability aspects of CP

The existing RRC message only supports up to 16 segments in UL (i.e. up to data size of 144KB). Supporting larger UL segment number will put extra memory requirement on both UE and gNB. Thus, whether it works needs to be decided based on specific value of dataset payload size.

Observation 4: Supporting UL RRC message with larger than 16 segments put extra memory requirement for both UE and gNB. Thus, whether it works needs to be decided based on dataset payload size.

For Option 4-1: sharing {target CSI, CSI feedback} dataset:

  - Data size for target CSI depends on the format. There is no agreement on the format or necessary precision of the target CSI. Some examples based on companies’ evaluations are: eType-II format (up to ~1000 bits), eType-II-like format (~ a few 1000 bits), and float32 format (up to ~ 150K bits). The data size may also vary depending on the configuration, and the captured value indicates the order of magnitude of the typical data size per sample as a guideline.

  - There is no agreement on the CSI feedback size. Values in the order of eType II payload size may be assumed (up to ~ 1000 bits) for RAN2 discussion.

  - There is no agreement on the total number of samples N1 in the dataset. The range of the number of samples N1 used in RAN1 evaluations is 40K-1200K. As a representative number, RAN2 may assume that the total number of samples is around N1=600K.

Summing up the above, and assuming eType-II-like format of a size 2000 bits, the total dataset size for Option 4-1 may be roughly in the order of 600K * (2000 bits + 1000 bits) / (8bits/Byte) = 225 MB. The size may be smaller for eType-II format. The size may be larger if we consider the dataset consisting of supporting multiple configurations.

In Rel-18 study phase, RAN1 sent Reply LS (R1-2310681) to RAN2 with analysis on data payload size for different use cases [6]. However, as it was early study phase, RAN1 didn’t provide specific values for most use cases. Recently, RAN1 sent another LS (R1-2410922) [7] to request RAN2 to evaluate feasibility of identified solutions on two-sided model, which provides a specific value for option 1 of CSI compression (225MB in Option 4-1):

Proposal 10: Before discussion whether dataset transfer over CP solution can work, RAN2 need to wait RAN1 conclusion on dataset payload size.

If 225MB is the typical data size, it means that maximum UL segment number needs to increase to 25600, which is impossible to support.

Thus, we think CP solution may not work if data size is too large, and RAN2 need to discuss based on specific value of dataset payload size. Given RAN1 only provide data size of CSI compression in LS (R1-2410922) [7], we think it is necessary to send LS to request RAN1 to provide data size of other AI/ML use cases, including beam management, positioning and CSI prediction.

Proposal 11: On scalability of CP solution, RAN2 conclude that it may not work if data size is too large (e.g., 225MB in CSI compression). RAN2 send LS to request RAN1 to provide data size of other AI/ML use cases, including beam management, positioning and CSI prediction.

# 3 Conclusion

In this contribution, we further study UE-side data collection. Our observations are:

Observation 1: The UE may want to change the measurement configuration according to its local / internal informaion. For example, the UE may prefer to be configured with CSI-RS resource with smaller size when it is in low battery or memory state.

Observation 2: As option 1b was precluded in RANP#106, the data transfer to CN domain is mapped to option 2 and the data transfer to OAM domain is mapped to option 3.

Observation 3: On UP traffic, existing NG-RAN only has the controllability and visibility on configuration of PDU session and QoS Flow. Whether NG-RAN can be aware of UE-side data transfer depends on SA2 design (e.g. whether to introduce AI/ML specific PDU session or AI/ML specific QoS flow).

Observation 4: Supporting UL RRC message with larger than 16 segments put extra memory requirement for both UE and gNB. Thus, whether it works needs to be decided based on dataset payload size.

Based on observations, our proposals can be found below:

Configuration of UE side data collection

Proposal 1: NW may provide several candidate data collection configurations where each includes at least RS configuration and associated ID FFS others. The UE request may indicate its preferred configuration from them.

Proposal 2: Introduce UAI message for UE request of data collection configuration. And it is up to UE implementation when to send the request.

Proposal 3: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

Proposal 4: On data collection configuration without UE request, RAN2 leaves the detailed solution discussion to other WG.

RAN aspects to enable the data transfer to CN domain or OAM domain

Proposal 5: Before SA2/SA5 provide feedback, RAN2 only discuss potential RAN impacts common to option 2 and option 3.

Proposal 6: Before RANP provide guidance, RAN2 don’t compare between option 2 (i.e. transfer to CN domain) and option 3 (i.e. transfer to OAM domain) and discuss their down-selection.

NG-RAN involvement in the data transfer

Proposal 7: RAN2 conclude that NG-RAN have the full control for dataset transfer over CP solution.

Proposal 8: RAN2 conclude that NG-RAN have full visibility for standardized dataset transfer over CP solution.

Proposal 9: RAN2 wait SA2 feedback before discussion on level of NG-RAN involvement on dataset transfer over UP solution, including its controllability and visibility.

RAN aspects related to data transfer over UP

Proposal 10: On data transfer solution over UP, RAN2 wait SA2 conclusion on whether/how NG-RAN can be aware of AI/ML dataset transfer (e.g. via AI/ML specific QoS flow, or AI/ML specific PDU session, or AI/ML specific protocol stack, etc.) to configure DRB accordingly.

Scalability aspects of CP

Proposal 11: On scalability of CP solution, RAN2 conclude that it may not work if data size is too large (e.g., 225MB in CSI compression). RAN2 send LS to request RAN1 to provide data size of other AI/ML use cases, including beam management, positioning and CSI prediction.

# 4 References

[1] RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm.

[2] RAN2#129, Chair Notes.

[3] RAN2#128, Chair Notes.

[4] R2-2411152, Reply LS to SA2 on AIML data collection.

[5] RP-243316, LS on AI/ML UE sided data collection.

[6] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[7] R1-2410922, LS on signalling feasibility of dataset and parameter sharing.

[8] RP-250030, Reply LS on AI/ML UE sided data collection, SA5.