3GPP TSG-RAN WG2 Meeting #129bis                                 R2-2502139

Wuhan, China, April 7th – 11th , 2025

Agenda item:		8.3.5

Source:				Samsung

Title:					Discussion on LCM and spec. impact for AI/ML for mobility

Document for:		Discussion & Decision

1	Introduction

For study on the general spec. impact for AI/ML for mobility, the following agreement was made in RAN2#129.

Agreements on priority

- Further RRM prediction simulations are not expected.  For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction.   FFS if L3 beam level needs to be considered.  For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

In this paper, we would like to discuss the spec. impact for temporal and frequency domain prediction based on the agreed priority above.

2	Discussion

According to the agenda title “LCM and spec. impact for AI/ML for mobility”, we can discuss the common framework for LCM and the potential spec. impact for each use case together.

For the LCM framework, we can adopt the general LCM framework agreed in AI/ML for air as baseline. For the spec. impact part, we can focus on the signalling aspect between UE and gNB to support each LCM operation. For example, for inference of UE-side model, we can study the inference configuration from NW and the report of outcome of model inference from UE.

2.1 Capability/Applicability reporting

Definition:

In Rel-19 AI/ML for air WI session, RAN2 agreed the definition of “supported functionalities”, “applicable functionalities”, and “Activated functionalities” as below.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

To facilitate discussion by ensuring we have the same understanding on the definitions in AI/ML for mobility SI, we would like to propose the following.

Proposal. 1: To facilitate discussion in AI/ML for mobility SI/WI, RAN2 adopt the definition of supported/applicable/activated functionalities agreed in Rel-19 AI/ML for air WI.

UE Capability Reporting:

Regarding UE capability reporting, RAN2 made the following agreement in Rel-19 AI/ML for air WI.

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Since we do not see any issue specific to AI/ML for mobility in UE capability reporting procedure, we would like to propose the following.

Proposal. 2: For AI/ML for mobility, UE can report the supported AI/ML-enabled Features/FGs and supported functionalities to NW via UECapabilityInformation message.

Applicability Reporting:

Regarding applicability reporting, RAN2 made the following agreement for BM use case in Rel-19AI/ML for air WI.

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

Since we do not see any issue specific to AI/ML for mobility in applicability reporting procedure, we would like to propose the following.

Proposal. 3: For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

2.2 Inference

UE-side model inference:

For UE-side model inference, NW can provide some configuration for inference at UE-side. Then, the UE can perform the inference with the input data that is internally available at UE and report the outcome of the inference to NW. For the spec. impact, we can focus on the inference configuration from NW to UE and the report from UE to NW.

For the common part in the use cases (i.e., frequency/temporal domain RRM measurement prediction, measurement event prediction), NW can indicate the target MO/Cells for which the UE predicts RRM measurement/event prediction. Also, as in BM use case, for the consistency between training and inference environment, NW can provide the NW-side additional condition (e.g., associated ID) which can be used for applicability check. Based on the NW configuration, UE can perform the inference and report the outcome (e.g., Predicted L3 Cell measurement results).

For temporal domain Case A, NW can indicate how far the UE should predict the measurement results in time domain (i.e., prediction window length). Then, UE can report the predicted L3 Cell/Beam-level measurement results of multiple time instances within the prediction window via MeasurementReport.

For temporal domain Case B, one issue specific to Case B is how to determine MRRT (Measurement Reduction Rate in Time domain). Given the trade-off relation between measurement reduction and prediction accuracy, NW should have some control over MRRT used by UE to guarantee HO performance. Specifically, NW can either directly set the value and configure it for UE, or provide some requirement on prediction accuracy for UE to select the proper MRRT value that can meet the requirement on the accuracy. 
Meanwhile, with the same MRRT value, the measurement skipping pattern can vary depending on the UE’s implementation (i.e., depending on how UE-side server trains the AI model), and NW can not be aware of it. However, in our view, NW’s awareness on the skipping pattern is necessary to maximize energy savings at UE-side (i.e., to align the C-DRX cycle, measurement gap timing with the skipping pattern). To achieve this, UE can provide its measurement skipping pattern as an assistance data for NW.

For measurement event prediction, NW can indicate the target event (e.g., EventA3) to predict. The existing ReportConfig can be reused/enhanced for that. Based on the configuration from NW, the UE can predict whether/when the target event will occur within a certain time window (i.e., prediction window). If the UE predicts the event, it can report the expected occurrence time (for indirect method) or the probability of event occurrence within PW (for direct method).

Based on the discussion above, we would like to propose the following.

Proposal. 4: For UE-side model inference, RAN2 can study the following spec. impact

- Enhanced or new measurement/report configuration from NW to UE

  - Indication of the target MO/cell for which the UE predicts RRM measurement results

  - NW-side additional condition (e.g., associated ID per MO/cell) for applicability check

  - (Temporal Case A, Event prediction) Prediction window length

  - (Temporal Case B) configuration to control MRRT

  - (Event prediction) Indication of target event to predict

- Enhanced or new report from UE to NW

  - Predicted L3 Cell/Beam-level measurement results

  - (Temporal Case A) Predicted L3 Cell/Beam-level measurement results of N future time instances

  - (Event prediction) Expected occurrence time of measurement event, Probability of event occurrence within a time window.

- Assistance information from UE to NW

  - (Temporal Case B) Measurement skipping pattern for NW configuration (e.g., C-DRX, MG, …).

NW-side model inference:

For NW-side model inference, UE can provide the input data for inference at NW-side. Then, the NW can perform the inference with the input data and use the outcome of the inference for HO. How to use the outcome for HO is totally up to NW implementation. Therefore, for the spec. impact, we can focus on the input data report from UE to NW.

For temporal domain Case B, the motivation is to reduce UE’s measurement overhead (for saving energy). Specifically, with 50% reduction rate, UE can skip measurement once out of every two instances and predict the measurement values at the skipped time instance. In our evaluation in FR1, we assume that UE measures SSB every 40msec as baseline and for Case B with MRRT 50%, UE performs the measurement every 80msec. Then, we evaluate HO performance for the scenario where the UE detects measurement event and trigger MR based on both actual and predicted measurement results, which shows that the performance degradation is marginal. Now if we want to do the same thing with NW-side model, the UE should report the measurement results every 80 msec continuously and NW should predict the results at the skipped point. Considering that Case B is for goal 1(i.e., measurement overhead reduction for UE power saving), the frequent MR transmission from UE with very small interval does not make sense.  Therefore, for the Case B prediction, the use of NW-side model can be excluded.

Based on the discussion above, we would like to propose the following.

Proposal. 5: For temporal domain Case B, RAN2 exclude the NW-side model case.

For the common part in the use cases (i.e., frequency/temporal domain RRM measurement prediction, measurement event prediction), the UE should provide the input data for NW every time NW performs the inference. Even though it is unclear how often NW will perform the inference, it is evident that there should be the concern on the signalling overhead from the input data report. Thus, we need to study ways to reduce the signalling overhead (e.g., event-triggered reporting, adaptive reporting based on mobility status, …) .

For temporal domain Case A and measurement event prediction, the input data would be the historical L3 Cell/beam-level measurement results of multiple time instances. However, with the legacy measurement report, the UE can report only one latest measurement result for each cell/beam. Therefore, there should be some enhancement on measurement reporting to allow UE to log the measurement results and report them to network.

Proposal. 6: For NW-side model inference, RAN2 can study the following spec. impact

- Enhanced or new measurement/report configuration from NW to UE

  - (Temporal Case A, Event prediction) Indication to request the report of logged L3 Cell/beam-level measurement results of N recent time instances

  - (Temporal Case A, Event prediction) Observation window length

- Enhanced or new report from UE to NW

  - (Temporal Case A, Event prediction) Logged L3 Cell/Beam-level measurement results of N recent time instances

- Enhanced reporting mechanism to reduce signalling overhead

2.3 Performance monitoring & management

Model/functionality management:

NW-side model:

Regarding the model/functionality management for NW-side model, the following agreement was made in Rel-19 AI/ML for air WI.

Agreements

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

Since we do not see any issue specific to AI/ML for mobility for the management procedure of NW-side model, we would like to propose the following.

Proposal. 7: For AI/ML for mobility, for NW-side model, the model/functionality management decision can be up to NW implementation and transparent to UE.

UE-side model:

For UE-side model case, based on the discussions in the AI/ML for air WI so far, both RAN1 and RAN2 are focusing solely on functionality management for UE-side model case, assuming that model-level management is handled by the UE itself and is transparent to the NW. Specifically, for a particular AI-enabled functionality, the UE can simply report that it is applicable if there is at least one available model to use for that functionality. Consequently, the network can manage the functionality at this level without needing to know which AI model the UE employs for the functionality. The same principle can be adopted also in AI/ML for mobility.

Proposal. 8: For AI/ML for mobility, for UE-side model, the model management decision can be up to UE implementation and transparent to NW.

Regarding the functionality management for UE-side model, the following agreement was made in Rel-19 AI/ML for air WI.

Agreement (in RAN2)

- For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

Agreement (in RAN1) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In the RAN1 agreement on the BM use case, Type 1 refers to the method where NW makes the functionality management decision based on the monitoring results from UE or NW itself, while Type 2 refers to a method where UE makes the decision by itself based on the internal monitoring results. Given the remaining time, it is likely that only Type 1 will be supported at least for BM use case in Rel-19 AI/ML for air WI. On the other hand, for positioning use case, RAN2 is still discussing the UE autonomous fallback as it offers the advantage of reducing management delay compared to NW-side decision.

Based on this, for AI/ML for mobility, we think NW-decision can serve as the baseline, but we should remain open to discussing the UE-side decision.

Proposal. 9: For AI/ML for mobility, for UE-side model, the functionality management decision can be made by NW as a baseline. FFS. Whether to support UE-side decision.

Performance monitoring:

NW-side model:

Regarding the performance monitoring for NW-side model, RAN2 made the following agreement for BM use case in Rel-19 AI/ML for air WI.

Agreements for NW-sided model for Beam Management

4	The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

In our view, also in AI/ML for mobility, the gNB has the responsibility on monitoring its NW-side model performance. For the monitoring, gNB should have both predicted and the actual measurement results. For the predicted measurement results, gNB can obtain it as outcome of its NW-side model inference. However, the actual measurement results should be provided by UE and we think the existing measurement/report configuration can be reused for that. For example, for frequency domain prediction, gNB can predict the measurement results for the cell in the target frequency based on the serving frequency measurement results from UE. When the gNB need to perform the monitoring (i.e., need to obtain the actual measurement in the target frequency), it can configure UE to perform the measurement in that target frequency and to report the results with the legacy measurement configuration with measurement gap.

Proposal. 10: For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement.

UE-side model:

Regarding the performance monitoring for UE-side model, RAN1 made the following agreement for BM use case in Rel-19 AI/ML for air WI.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

For the UE-side model, the performance monitoring (i.e., calculation of performance metric) can be done by either UE (Option 2) or NW (Option 1) and the agreement above shows that RAN1 is focusing on the method to support UE-side monitoring. In our view, in case of UE-side model, both predicted and actual result are available at UE-side. Thus, it seems reasonable for UE to perform the performance monitoring and report it to NW for the management decision by NW. In that sense. we would like to propose that UE-side performance monitoring can be the baseline.

Proposal. 11: For AI/ML for mobility, for UE-side model, UE can perform the performance monitoring (e.g., performance metric calculation) and report the monitoring results to NW as baseline.

For the performance monitoring metric, “L3 RSRP difference” can be used for temporal and frequency domain prediction since we already evaluate the prediction accuracy with the metric. For measurement event prediction, the prediction accuracy is evaluated based on “F1 score” and thus we can use it as performance monitoring metric. One more point to think about is whether “L3 RSRP difference” can be also used as a metric for measurement event prediction, considering the relation that the lower “L3 RSRP difference” will lead to higher “F1 score” in the indirect method.

Proposal. 12: For temporal and frequency domain prediction, “L3 RSRP difference” can be used for performance monitoring metric.

Proposal. 13: For measurement event prediction, “F1 score” can be used for performance monitoring metric.
FFS. “L3 RSRP difference” can be also used for the indirect prediction.

Regarding the report of monitoring results by UE, we can consider the following two options.

- Option 1 (periodical reporting)
- UE periodically reports the calculated performance metric (e.g., L3 RSRP difference) to NW
- NW can make the management decision based on the calculated performance metric from UE.

- Option 2 (event-triggered reporting)
- UE reports the monitoring event (e.g., prediction accuracy becomes lower than some threshold) to NW once it is detected.
- NW can make the management decision based on the monitoring event report from UE.

In terms of signalling overhead, the option 2 has a benefit compared to the option 1, since with the option 2, the UE will trigger the monitoring report only when any event to report is detected. On the other hand, the option 1 can give more flexibility to NW from a management perspective while simplifying UE operations. Thus, RAN2 can study both options for the monitoring report.

Proposal. 14: For monitoring report from UE to NW in UE-side performance monitoring, RAN2 can study the following two options.

- Option 1 (periodical reporting)
- UE periodically reports the calculated performance metric (e.g., L3 RSRP difference) to NW
- NW can make the management decision based on the calculated performance metric from UE.

- Option 2 (event-triggered reporting)
- UE reports the monitoring event (e.g., prediction accuracy becomes lower than some threshold) to NW once it is detected.
- NW can make the management decision based on the monitoring event report from UE.

Another issue to address is how the UE can acquire the actual measurement results for performance monitoring in use cases involving measurement reduction (e.g., temporal domain Case B, frequency domain prediction). In such scenarios, the UE does not carry out actual measurements at certain times or frequencies to minimize its measurement overhead. Consequently, when the UE conducts inference, it cannot access the actual measurement results needed to compute the performance metric. Therefore, there must be a designated time duration (e.g., a monitoring window) during which the UE performs both actual measurements and inference for prediction concurrently.

For the specific spec. impact, we would like to propose the following based on the discussion above.

Proposal. 15: For UE-side performance monitoring for UE-side model, RAN2 can study the following spec. impact.

- Monitoring results

  - Option. 1: Performance monitoring metric (e.g., RSRP difference)

  - Option. 2: Event report (e.g., monitoring failure event)

- Enhanced or new measurement/report configuration from NW to UE

  - Indication to request UE to report the monitoring result

  - (Temporal Case B, frequency domain) Monitoring window

  - Monitoring report configuration 
(For Option. 1) monitoring report periodicity
(For Option. 2) monitoring event configuration

- Enhanced or new report from UE to NW

  - Report of monitoring results

2.4 Training data collection

NW-side model:

Regarding the training data collection for NW-side model, RAN2 made the following agreement for BM in Rel-19 AI/ML for air WI.

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

The agreements above are about training data collection for NW-side model in BM use case. According to the agreements, the baseline procedure for training data collection of NW-side model can be described as below.

- Data logging: UE can log multiple instances of L1 measurement result with periodic logging. Event-triggered data logging is supported.

- Data reporting: UE reports multiple instances of logged L1 measurement result to gNB when gNB requests it. Only on-demand reporting is supported. Periodic reporting is not supported.

- UE state report: UE can report the internal status (e.g., low power, low memory, …) that may impose restrictions on data logging.

From a training data perspective, RRM measurement prediction and Beam prediction are quite similar in that they require multiple instances of measurement results. The only difference lies in whether the measurement result is per cell or beam and whether they are L1 or L3 measurement results. Therefore, in our view, we can adopt the aforementioned baseline procedure also for RRM measurement prediction. Note that we do not need to consider the measurement event prediction here since the measurement event prediction with NW-side model is out of scope of this SI.

Based on the discussion above, we would like to propose the following.

Proposal. 16: For RRM measurement prediction, for NW-side data collection for NW-side model training, UE can log L3 measurement results and report multiple instances of logged L3 measurement results to gNB via RRC.

Proposal. 17: For RRM measurement prediction, for NW-side data collection for NW-side model training, the report of the logged data can be initiated by on-demand request from NW.

Proposal. 18: For RRM measurement prediction, for NW-side data collection for NW-side model training, periodic logging is supported and it can be triggered by RRM measurement event.

Proposal. 19: For RRM measurement prediction, for NW-side data collection for NW-side model training, UE can report the internal status (e.g., low power, low memory, …) that may impose restrictions on data logging.

UE-side model:

Regarding the training data collection for UE-side model, the following agreement was made in Rel-19 AI/ML for air WI.

Agreements

Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration:

1.	Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

2.	For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).  FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

3.	The network can provide or release the data collection configuration (at any point in time), with or without UE request.

4.	The following methods for network control of the initiation and configuration for data collection:
a. The network can decide when to start/stop the data collection and send configuration.
b. The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

For the training data collection for UE-side model, we can discuss the data collection part and the data transfer part, separately.

In RAN2, the data collection part has been discussed under Rel-19 AI/ML for air WI. Here, the data collection part is about how the UE collects the training data based on the configuration from NW. The agreements above are made under the discussion on the data collection part, which can be summarized as below.

- For UE to collect the training data, there should be some training data collection configuration from NW. 
- UE can send a request for the data collection configuration to NW.
- NW can provide or release the data collection configuration with or without UE request.

For BM use case, NW can provide the configuration needed for UE to measure CSI-RS to collect the beam measurement data. Similarly, in the case of RRM measurement prediction, NW can provide the configuration (e.g., MeasConfig) for UE to measure SSB to collect cell measurement results for training data.

Proposal. 20: For AI/ML for mobility, for UE-side data collection for UE-side model training, gNB can provide data collection configuration (e.g., MeasConfig) with or without UE request.

Meanwhile, for the data transfer part, the issue is how to transfer the training data from UE to UE-side server. Although RAN2 has discussed the three options (Option 1a, 2, 3), we were not able to reach a consensus on which option to support in Rel-19 AI/ML for air WI. Consequently, RAN decided to continue studying those options under Rel-19 AI/ML for air SI. Thus, we can postpone the discussion in AI/ML for mobility SI and adopt the outcome from Rel-19 AI/ML for air SI.

Proposal. 21: For AI/ML for mobility, for UE-side data transfer for UE-side model training, RAN2 can postpone the discussion and adopt the outcome from Rel-19 AI/ML for air SI.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Capability/Applicability reporting:

Proposal. 1: To facilitate discussion in AI/ML for mobility SI/WI, RAN2 adopt the definition of supported/applicable/activated functionalities agreed in Rel-19 AI/ML for air WI.

Proposal. 2: For AI/ML for mobility, UE can report the supported AI/ML-enabled Features/FGs and supported functionalities to NW via UECapabilityInformation message.

Proposal. 3: For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

Inference:

Proposal. 4: For UE-side model inference, RAN2 can study the following spec. impact

- Enhanced or new measurement/report configuration from NW to UE

  - Indication of the target MO/cell for which the UE predicts RRM measurement results

  - NW-side additional condition (e.g., associated ID per MO/cell) for applicability check

  - (Temporal Case A, Event prediction) Prediction window length

  - (Temporal Case B) configuration to control MRRT

  - (Event prediction) Indication of target event to predict

- Enhanced or new report from UE to NW

  - Predicted L3 Cell/Beam-level measurement results

  - (Temporal Case A) Predicted L3 Cell/Beam-level measurement results of N future time instances

  - (Event prediction) Expected occurrence time of measurement event, Probability of event occurrence within a time window.

- Assistance information from UE to NW

  - (Temporal Case B) Measurement skipping pattern for NW configuration (e.g., C-DRX, MG, …).

Proposal. 5: For temporal domain Case B, RAN2 exclude the NW-side model case.

Proposal. 6: For NW-side model inference, RAN2 can study the following spec. impact

- Enhanced or new measurement/report configuration from NW to UE

  - (Temporal Case A, Event prediction) Indication to request the report of logged L3 Cell/beam-level measurement results of N recent time instances

  - (Temporal Case A, Event prediction) Observation window length

- Enhanced or new report from UE to NW

  - (Temporal Case A, Event prediction) Logged L3 Cell/Beam-level measurement results of N recent time instances

- Enhanced reporting mechanism to reduce signalling overhead

Performance monitoring & management:

Proposal. 7: For AI/ML for mobility, for NW-side model, the model/functionality management decision can be up to NW implementation and transparent to UE.

Proposal. 8: For AI/ML for mobility, for UE-side model, the model management decision can be up to UE implementation and transparent to NW.

Proposal. 9: For AI/ML for mobility, for UE-side model, the functionality management decision can be made by NW as a baseline. FFS. Whether to support UE-side decision.

Proposal. 10: For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement.

Proposal. 11: For AI/ML for mobility, for UE-side model, UE can perform the performance monitoring (e.g., performance metric calculation) and report the monitoring results to NW as baseline.

Proposal. 12: For temporal and frequency domain prediction, “L3 RSRP difference” can be used for performance monitoring metric.

Proposal. 13: For measurement event prediction, “F1 score” can be used for performance monitoring metric.
FFS. “L3 RSRP difference” can be also used for the indirect prediction.

Proposal. 14: For monitoring report from UE to NW in UE-side performance monitoring, RAN2 can study the following two options.

- Option 1 (periodical reporting)
- UE periodically reports the calculated performance metric (e.g., L3 RSRP difference) to NW
- NW can make the management decision based on the calculated performance metric from UE.

- Option 2 (event-triggered reporting)
- UE reports the monitoring event (e.g., prediction accuracy becomes lower than some threshold) to NW once it is detected.
- NW can make the management decision based on the monitoring event report from UE.

Proposal. 15: For UE-side performance monitoring for UE-side model, RAN2 can study the following spec. impact.

- Monitoring results

  - Option. 1: Performance monitoring metric (e.g., RSRP difference)

  - Option. 2: Event report (e.g., monitoring failure event)

- Enhanced or new measurement/report configuration from NW to UE

  - Indication to request UE to report the monitoring result

  - (Temporal Case B, frequency domain) Monitoring window

  - Monitoring report configuration 
(For Option. 1) monitoring report periodicity
(For Option. 2) monitoring event configuration

- Enhanced or new report from UE to NW

  - Report of monitoring results

Training data collection:

Proposal. 16: For RRM measurement prediction, for NW-side data collection for NW-side model training, UE can log L3 measurement results and report multiple instances of logged L3 measurement results to gNB via RRC.

Proposal. 17: For RRM measurement prediction, for NW-side data collection for NW-side model training, the report of the logged data can be initiated by on-demand request from NW.

Proposal. 18: For RRM measurement prediction, for NW-side data collection for NW-side model training, periodic logging is supported and it can be triggered by RRM measurement event.

Proposal. 19: For RRM measurement prediction, for NW-side data collection for NW-side model training, UE can report the internal status (e.g., low power, low memory, …) that may impose restrictions on data logging.

Proposal. 20: For AI/ML for mobility, for UE-side data collection for UE-side model training, gNB can provide data collection configuration (e.g., MeasConfig) with or without UE request.

Proposal. 21: For AI/ML for mobility, for UE-side data transfer for UE-side model training, RAN2 can postpone the discussion and adopt the outcome from Rel-19 AI/ML for air SI.