3GPP TSG-RAN WG2 Meeting #129bis												     R2-2502175

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.2.4

Source:	Apple

Title:	A-IoT MAC design for data transmission

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our views on the Ambient IoT MAC design focusing on data transmission in accordance with the WID [1] objectives and SI conclusions [2], while also covering a couple of general MAC design principles and issues.

# 2   	Discussion

In RAN2#129 a number of agreements relevant to this discussion have been achieved, specifically:

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

These agreements, in particular “aim to design simple MAC PDU format design” reinforce what we’ve argued before - an Ambient IoT device is expected to have an extremely low complexity “modem”, no CPU and in general very rudimentary processing capabilities. While it may not be true for all devices, it is certainly true for many. As ambient IoT MAC design should cater to all device types, including extremely low complexity ones (such as stickers for products in a warehouse), our main guiding principle should be low cost and simplicity, specifically processing simplicity. This is a radical departure from the way 3GPP normally design our systems, where performance is key. We think that in Ambient IoT design, when there is a tradeoff to make between simplicity and performance, we should prefer the former. Otherwise, we are likely to end up with a great Ambient IoT system which no one would deploy.

Observation 1: Ambient IoT MAC design should support devices with extremely limited rudimentary processing capabilities.

### 2.1 UL and DL data PDUs

With this understanding we now proceed to discuss DL and UL data frame formats. It must be noted that in the following discussion, the functions we propose are not exclusive. That is, what we propose for now is the “bare minimum” and we are generally open to consider additional proposals. However, we must start somewhere and so let’s start with the following high-level DL and UL data frame formats. Furthermore, what’s important for now is to agree on design principles – once it is done, adding new features would be relatively easy.

Below we illustrate a general DL and UL data frame structure.

| AS ID. Control. UL grant. Data length. Data. Figure 1: DL data PDU | Control. Data Length (FFS). Data. Figure 2: UL Data PDU |
| --- | --- |

In both UL and DL, the proposal is to have a frame structure with fixed control part (which is always present),an optional data part (which may or may not be present, see below for more detail) and other fields (which may differ between DL and UL).

Let’s first focus on the “Control” field. In NR (and LTE) we are used to the concept of MAC CE, which is very convenient and flexible, and it is natural that some people propose to reuse this legacy mechanism in A-IoT. Such flexibility, though, comes at a cost which is likely to be prohibitive for cheap A-IoT devices. Furthermore, we don’t think such extreme flexibility is even needed for A-IoT. The number of control functions which would require bits in the control field is very limited and we don’t expect it to grow significantly even in the next release. Finally, if we ever run out of the spare bits in the control field, there is always an option to extend the protocol (as we explain further down below. Hence the proposal: UL and DL data frames contain a fixed-length control field which is always present.

Proposal 1: UL and DL data frames contain a fixed-length control field which is always present. MAC CEs are not used in A-IoT.

The length of the control field can be further discussed. It’s quite clear that 1 byte should be more than sufficient for the first release of A-IoT and is likely to cover the next release (through the remaining spare bits) as well. However, 2 bytes can also be considered. Anyway, we don’t need to discuss the exact MAC control header length (e.g. 1 or 2 bytes) at this stage, even though in our view 1 byte is probably sufficient. We can first agree on the control bits and based on this (and the number of reserved bits we would want to have) decide the MAC control header length.

Furthermore, as we explain below, there is also the proposal to have fixed control bit assignment.

As we already demonstrated, the primary motivation for the Ambient IoT MAC design should be [extreme] device simplicity. Since such devices can’t be expected to have a real CPU implementing complex logic, there is benefit in defining a fixed-length MAC control header where each control (e.g. ACK/NACK) indication is always in the same bit.

We understand that such design may have limitations as it is somewhat less extensible (however, there are ways to make it extensible in future releases, as we demonstrate below). Furthermore, it is evident that more flexible design would be somewhat more efficient in terms over the air interface transmissions. However, such minor optimizations (saving a few bits here and there) hardly justify substantial increase in Ambient IoT device complexity and cost. Hence the proposals: MAC PDU contains a fixed length (e.g. 1 or 2 bytes) “control header” with fixed control bits assignment.

Proposal 2: Fixed control bit assignment in MAC PDU control header.

Similar considerations (i.e. the appeal of fixed size headers) apply to other fields as well, naturally the “AS ID” and perhaps even “UL grant”. As the discussion on these is conducted in a different agenda item and also depends on other WGs (SA2 and RAN1) we skip this discussion for now.

Proposal 3: it would also be beneficial to design fixed-length “AS ID” and “UL grant” fields in the DL MAC PDU.

In the last meeting, RAN2 have agreed to support segmentation retransmission through the mechanism in which a reader indicates an offset, i.e. the number of bits successfully received so far:

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

One way to support this agreement while also keeping the fixed size (and fixed bit assignment) of the control field is to define an optional “retransmission offset” field which would only be present if NACK is signaled.

Proposal 4: define an optional “retransmission offset” field which would only be present if NACK is signaled.

Assuming it is agreeable that the MAC control header with fixed length is adopted, we then need to discuss the “content”, i.e. what kind of control information we need to have in DL and UL MAC PDUs and how many bits to assign for each one.

As we seem to have agreed to support retransmissions (of segments) in UL, it’s obvious that ACK/NACK bit is needed in the DL MAC PDU to indicate success/failure of the last received segment. Furthermore, it is equally obvious that we need to support multiple types of DL PDUs (e.g. data, paging, msg2, etc) we also need a “message type” field. While in our view it is sufficient to allocate 2 bits for the message type field, we acknowledge that it would be better to park the question of how many bits to use for the time being till we agree on the number of different DL MAC PDUs.

RAN2#129 have already agreed to introduce “NACK based mechanism in D2R”, which naturally requires a bit in the DL control header. Furthermore, RAN2 have agreed to define DL message type and to consider the following types:

- R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF))

- D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not)

- R2D MAC PDU (MSG2)

- D2R MAC PDU (MSG3 and data)

- R2D MAC PDU (R2D data)

- Other message types are FFS.  The message types may evolve based on functionality agreements.

With this we propose to agree that DL MAC control header contains at least the following: “message type” and “ACK/NACK” (1 bit).

Proposal 5: DL MAC control header contains at least the following: “message type” and “ACK/NACK” (1 bit).

How many bits to assign for the message type indication is not yet clear. 2-3 bits are likely to be sufficient, but we can postpone this decision till later.

Turning now to the UL control, we first note that there is the following FFS in our agreements: FFS whether for D2R we need message type field, any length and need for padding. On one hand, message type in UL/D2R may seem redundant, as everything is triggered by the reader and therefore the reader would know what kind of message type to expect in the UL grant it provides to the device. That being said, we think there are at least two reasons to consider message type in UL/D2R:

- Future extensibility – this would allow adding new non-backwards compatible UL messages in the future

- More robust design (i.e. for the case of multiple reader scenario, etc)

Hence the proposal to adopt message type in UL/D2R as well.

Proposal 6: UL MAC control header contains “message type” field

With regards to UL data length indication, our initial thinking was that L1 pre/post-amble, if introduced in RAN1, may make it redundant. However, since in DL we decided to introduce the data length anyway, we might as well be consistent and follow suit for UL design.

Proposal 7: UL MAC header contains length indication.

Furthermore, the following functionality would require at least the following indications in the UL control header:

- Energy status indication (if needed)

- Last segment/more data

The main point of the arguments and the proposal below is the concept of how we introduce control bits for such features (i.e. we are not necessarily advocating for energy status indication, and indeed we are skeptical that it’s needed). What’s important is to address the fact that we shouldn’t expect all devices to support such features. At the same time, we should also avoid defining capability exchange signalling. Therefore, the main question is how to design signalling for fuch features so that a device has the flexibility to support the feature or not, and yet the reader would be able to react accordingly without any prior knowledge of whether the device supports the feature.

Proposal 8: segmentation and energy status indication (if needed) are optional features.

The question then becomes how to realize optionality. The usual 3GPP way of UE capability signaling is obviously out of the question, as we have agreed not to use RRC (and in fact agreed to only define MAC).

Observation 2: as there is no RRC in Ambient IoT, optional features cannot be realized through capability signalling.

Our proposal is to define the signalling (i.e. MAC control header bits and the associated device behavior) is such a way, that devices supporting the feature would signal the relevant indication while the devices not supporting it would signal a fixed value, e.g, 0.

For example, the energy status 1-bit indicator can be defined as follows:

- 1 indicates an alert that the device has no energy for more than the current message  exchange (or if the feature is not supported)

- 0 otherwise (i.e., the device has energy for more transmissions, or in other words no engery shortage alert);device not supporting the feature always signals 0

And so the devices that support the feature, would indicate 1 or 0 in accordance with their energy status alert and the devices that don’t would always indicate 0. The reader would then be able to handle the situation even without knowing which devices support it. So the reader behavior (this is just an example for illustration, we do not actually propose to specify reader behavior) would be:

- If the reader gets an UL message with energy status alert indication set to 0, it will immediately proceed to the next message (if needed). If it fails (which can happen anyway), the reader will handle it as it would for many other reasons a message might fail.

- If the reader gets an UL message with energy status set to1, it will deploy whatever corrective actions the reader implementation finds useful (e.g. wait for the device to accumulate more energy, etc.).

We would like to further note, that even if a device does support the energy status alert indication feature, such estimate would be imprecise as the device may not know the next command, how much data it will need to send, etc. So the feature is an optimization which might work in some cases, but for sure will not work in 100% cases.

Similar logic can be defined for another optional feature of last segment/more data indication as follows:

- 1 indicates device has more (beyond the current grant) data to transmit

- 0 otherwise (i.e. this is the last segment); device not supporting the feature always signals 0

Proposal 9: since there is no capability signaling, optional features are defined in such a way that a device not supporting a feature always signals a specific value in the corresponding control bit.

Proposal 9a: for last segment/more data indication, 1 indicates device has more (beyond the current grant) data to transmit and 0 otherwise (or if the device doesn’t support the feature).

### 2.3 Extensibility

With fixed MAC control header design, there is the inevitable question of extensibility – that is, how new features are added in e.g. the next release.

First off, we are likely to end up having a few spare/reserved bits, which presents an easy way to add more functionality in the future. As we’ve demonstrated above, in many (if not all) cases it should be possible to design new features in such a way that the legacy devices not supporting it would always signal a specific value (e.g. 0) in the spare/reserved bits.

Observation 6: spare/reserved bits can be used for future extensions.

However, as Rel-19 is the first Ambient IoT release with very limited functionality, it is likely that we won’t be able to get away with just the reserved bits for future extensions (even though we could reserve a whole spare byte to be on the safe side). There are then two ways to ensure future extensibility without the limitations of using a handful of spare/reserved bits:

- Protocol version indication, or

- Extension bit

Assuming 1 byte MAC control header, 2 bits version field (V), 2 bits message type (T) and 1 bit for ACK/NACK (A) in the UL header (these numbers are up to further discussion, we use them just to illustrate the concept here) the header could look as in Figure 3. Fields marks as R indicate spare/reserved bits.

Figure 5: Illustration of UL control header with protocol version

With the version field, in the next release we would increase the version number, which would allow us unrestricted freedom of design for the next extensions.

With regards to the existing features and their control bit assignment there would be two options:

- Complete redesign of the whole header, both the legacy and the new parts

- Keep the legacy part as is, and only add new features and their corresponding control bits in e.g. the second byte

Either way, in this approach both the device and the reader would indicate their supported version in the header and the legacy devices would ignore MAC PDUs with protocol versions they don’t support. This would probably require the reader to support both versions and if (it expects multiple versions in the population of devices they need to communicate with), the reader would need to integrate and communicate with devices of different versions separately. For example, the reader would first inventory/communicate with devices of version 0 and then proceed to do the same with devices supporting version 1, etc.

Alternatively, we can allocate 1 (e.g. the last bit) to indicate that there is an extension. This is illustrated in Figure 4 below.

Figure 6: Illustration of UL control header with extension bit

In this alternative, in the first version the extension bit X would be set to 0, and to 1 in the next release. When a legacy (e.g. Rel-19) devices sees that X is set to 1, it would assume that there is an extra byte in the control header that it is not expected to comprehend and process.

Proposal 10: to discuss whether to support future extensibility using “protocol version” or “extension bit”.

# 3	Proposals

Observation 1: Ambient IoT MAC design should support devices with extremely limited rudimentary processing capabilities.

Proposal 1: UL and DL data frames contain a fixed-length control field which is always present. MAC CEs are not used in A-IoT.

Proposal 2: Fixed control bit assignment in MAC PDU control header.

Proposal 3: it would also be beneficial to design fixed-length “AS ID” and “UL grant” fields in the DL MAC PDU.

Proposal 4: define an optional “retransmission offset” field which would only be present if NACK is signaled.

Proposal 5: DL MAC control header contains at least the following: “message type” and “ACK/NACK” (1 bit).

Proposal 6: UL MAC control header contains “message type” field

Proposal 7: UL MAC header contains length indication.

Proposal 8: segmentation and energy status indication (if needed) are optional features.

Observation 2: as there is no RRC in Ambient IoT, optional features cannot be realized through capability signalling.

Proposal 9: since there is no capability signaling, optional features are defined in such a way that a device not supporting a feature always signals a specific value in the corresponding control bit.

Proposal 9a: for last segment/more data indication, 1 indicates device has more (beyond the current grant) data to transmit and 0 otherwise (or if the device doesn’t support the feature).

Observation 6: spare/reserved bits can be used for future extensions.

Proposal 10: to discuss whether to support future extensibility using “protocol version” or “extension bit”.

# 5	References

[1]		RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2]		TR 38.769 Study on solutions for ambient IoT (Internet of Things)