3GPP TSG-RAN WG2 Meeting #129bis													     R2-2502176

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.3.2.2

Source:	Apple

Title:	On generalization across cell configurations

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our evaluation results for the study of model generalization across cell configurations

# 2   	Discussion

### 2.1		Background

This is the customary boilerplate background information, which you can safely skip and go straight to the next subsection 2.2.

RAN2#129 agreed [1], based on the email discussion [2], to further conduct the study of model generalization (using the representative case of RRM prediction in temporal domain) based on the following assumptions as agreed:

- Two sets of parameters (ISD, BS antenna height, BS Tx power)

- FR1 is the primary focus

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 1: simulation assumptions

One potentially important parameter (UE speed) wasn’t explicitly agreed, and so based on offline discussions we decided to set it to 30km/h and to focus on case B. Here is the list of additional assumptions we’ve used in our evaluation:

- Speed: 30 km/h

- Mobility pattern: option 1

- MRR: 50%

- Measurement period: 200ms

Previously (in RAN2#128) we’ve agreed on a general approach to the generalization study, which essentially mimics the approach adopted in RAN1:

- Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

In subsection 2.2 below we provide our evaluation results, along with conclusions and some preliminary considerations on how these results affect the normative phase.

### 2.2	Evaluation Results

In this subsection we provide the evaluation results. If you are interested in the analysis and the conclusions only, proceed to subsection 2.3 below.

In our evaluation, we’ve considered four cases, two of which correspond to two variants of Generalization Case #1 (GC#1) and two to the Generalization Case #2 (GC#2):

- Training data set – UMi, testing data set – UMa

- Training data set – UMa, testing data set – UMi

- Training data set – mixed (50%/50%) UMi and UMa, testing data set – UMa

- Training data set – mixed (50%/50%) UMi and UMa, testing data set - UMi

As in our previous contributions, we’ve evaluated 4 AI/ML models with the intention to cover very simple models (e.g. Autoregression), moderately complex models (e.g. Random Forest, XGBoost) and relatively high complexity models (e.g. Feedforward NN):

- Autoregression (AR)

- Random Forest

- XGBoost

- Feedforward NN

We’ve used similar models in [3] and [4] (and other contributions), and therefore model description is omitted here for brevity. Below we provide the evaluation results for the 4 cases mentioned above and then proceed to discuss the conclusions. The metric used in these results is the Mean Absolute Error (MAE). The time axis (x) is the prediction window.

Case 1: Training data set – UMi, testing data set – UMa

Figure 1: Training data set – UMi, testing data set – UMa

Case 2: Training data set – UMa, testing data set – UMi

Figure 2: Training data set – UMa, testing data set – UMi

Case 3: Training data set – mixed UMi and UMa, testing data set – UMa

Figure 3: Training data set – mixed UMi and UMa, testing data set – UMa

Case 4: Training data set – mixed UMi and UMa, testing data set - UMi

Figure 4: Training data set – mixed UMi and UMa, testing data set - UMi

At the first glance, it is already rather obvious that there is generally (pun intended) no generalization problem, as in all the cases the MAE is below 1, often much lower – around 0.5. However, it is of course beneficial to have a reference to compare the generalization results. Fortunately, there is a simple way to do so in the present study of generalization across cell configurations – as this (generalization across UMa and UMi) is the main purpose of the study, a simple and useful reference is the case of:

- Training and testing with UMa, and

- Training and test with UMi

Observation 1: a simple way to assess whether the generalization (across cell configurations) is good enough is to compare the results of various generalization cases with a baseline, which uses only one cell configuration (UMa or UMi).

We would like to further note that in this case using a baseline (as opposed to system level KPIs) is more than enough, as the intention is to study whether generalization degrades performance – not necessarily whether AI/ML models deliver acceptable performance (which had already been established in the previous meetings).

These baseline results used for reference are provided below.

UMa baseline

Figure 5: UMa baseline

UMi baseline

Figure 6: UMi baseline

A few immediate (and relevant for the following analysis) observations should be noted about these baseline results. These observations are not new as similar results have been shown before, but it is worth keeping them in mind in the context of the follow up discussion:

- AI/ML performs better for UMa rather than UMi (this is intuitively obvious, as we should expect more variants with UMi); below we would use the following baseline MAE values

  - 0.46 for UMa

  - 0.66 for UMi

- AR and Feedforward NN perform best

With this we now proceed to analyze the results.

### 2.3		Analysis and Conclusions

Perhaps the most interesting question to answer (and the most consequential in terms of standards impacts) is whether there is indeed an issue with AI/ML model generalization across cell configurations. In order to answer it, we need not consider all the results (we’ll do it later anyway), but instead it would be more than sufficient to compare the best performing generalization configuration to the corresponding baseline.

To answer this question, we use the results from cases 3 and 4, where mixed (UMa and UMi) data set is used for training. In case 3 (testing with UMa configuration), the best models (AR and FNN) provide accuracy of about 0.45-0.46 MAE for the baseline which is actually very close to the generalization results. In case 4 (testing with UMi configuration), there is a small gap (between the baseline and the generalization results) – the best models in the baseline delivery accuracy of about 0.66 MAE whereas the generalization results are in the area of 0.68. Having said that, even in this (UMi) case the gap is negligible.

This leads us to the first and arguably the most important conclusion – under the simulation assumptions we’ve agreed on, there is no generalization across cell configuration issues. In other words, a model trained on a mixed data set covering multiple cell configurations delivers similar accuracy to a cell configuration specific model.

Observation 2: a model trained on a mixed data set covering multiple cell configurations delivers similar accuracy to a cell configuration specific model; in other words, under the simulation assumptions we’ve agreed on, there is no generalization across cell configuration issues.

As we show in our companion contribution [5], this has significant consequences for the discussion of LCM for AI/ML mobility.

The remaining observations are unlikely to have significant impact on the normative specifications work, but may be interesting, nonetheless.

As we’ve alluded to already, there is a significant difference in generalization performance between GC1 and GC2:

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

This is especially evident with the results where the testing data set is UMi – for example, using OW of 800ms and AR and FNN models as an example, GC#1 accuracy (in this example) is 0.74-0.77 whereas the GC#2 accuracy is 0.66-0.67 MAE. Similar pattern is observed with other examples. Hence the observation that GC#2 outperforms GC#1.

Observation 3: GC#2 (training on mixed data sets) outperforms GC#1 (training in configuration-specific data set).

And yet another observation is that the generalization problem is harder when going from UMa to UMi, then the other way around. For instance, using the same example as above (800ms and AR and FNN models), we observe the following difference in results:

- UMa -> UMi: 0.47-0.48

- UMi -> UMa: 0.74-0.77

This result is both not surprising (as obviously there more variations in the UMi data compared to UMa) and not particularly important, as what matters is that there is generalization strategy (to use mixed data sets for training) that works.

Observation 4: generalization problem is harder when going from UMa to UMi, then the other way around; this is not particularly important as long as there is an approach ((to use mixed data sets for training) that works.

And so the proposal is to capture the observations listed above in the TR.

Proposal 1: to capture the observations listed above in the TR.

# 3	Proposals

### 3.1 	Observations

Observation 1: a simple way to assess whether the generalization (across cell configurations) is good enough is to compare the results of various generalization cases with a baseline, which uses only one cell configuration (UMa or UMi).

Observation 2: a model trained on a mixed data set covering multiple cell configurations delivers similar accuracy to a cell configuration specific model; in other words, under the simulation assumptions we’ve agreed on, there is no generalization across cell configuration issues.

Observation 3: GC#2 (training on mixed data sets) outperforms GC#1 (training in configuration-specific data set).

Observation 4: generalization problem is harder when going from UMa to UMi, then the other way around; this is not particularly important as long as there is an approach ((to use mixed data sets for training) that works.

### 3.1 	Proposals

Proposal 1: to capture the observations listed above in the TR.

# 5	References

[1]				RAN2#129 meeting minutes

[2]				R2-2500559	Email discussion summary for [Post128][018][AI Mob] generalization  (Apple)	Apple

[3]				R2-2409201, RRM measurement prediction results using field and simulated data, Apple

[4]				R2-2406664, RRM measurement prediction results for case B, Apple

[5]				R2-2502177, On LCM for AI/ML mobility, Apple