3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502193

Wuhan, CN, April 07-11, 2025

Agenda Item:	8.17

Source:	THALES

Title:	Discussion on support of IoT-NTN TDD mode

Document for:	Discussion

# 1	Introduction

The IoT-NTN TDD mode Work Item approved in RAN#105 in September 2024 was revised in RAN#106 [1] following a study phase in RAN1. It states the following objectives for RAN2:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2]

The RAN1 study phase agreements can be found in the WI status report [2].

In this contribution, we present the definition of TDD pattern as discussed in RAN1, and the possible adaptations and associated UE procedures related to the new IoT-NTN TDD pattern:

- SI scheduling

- Paging (H-SFN)

- Kmac extension

- Random Access Procedures

- Cell Barring

# 2	Discussion

RAN1 started the normative phase at RAN1#120. The following agreements were made for the frame structure and how to handle NPSS/NSSS/NPBCH/NPUSCH/NPDCCH and NPDSCH (not containing SI):

Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [48…50] ms at the ULSRP - NOTE: This corresponds to the largest separation between DL and UL Iridium slot pairs. Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA Agreement The downlink subframes within D=8 are fixed to: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) Agreement NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - FFS: handling for SIB1-NB Agreement The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications - NPUSCH is mapped as per current specifications. - NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. - FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes” - FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD. - FFS: handling of DL transmission gap - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

Figure 1 - The periodic transmission pattern

The periodic transmission pattern is defined by RAN1 as illustrated above with 8 downlink NB-IoT subframes followed by a fixed guard period (in the specifications) to be [48..50] ms, followed with 8 uplink NB-IoT subframes in a periodic transmission of 9 radio frames equal to 90 ms. All the other subframes where no transmission occurs are named “non-D NB-IoT subframes” and “non-U NB-IoT subframes”.

Regarding UE behavior during the DL-to-UL transmission gap, the last RAN2 meeting, it was proposed by some companies to reuse the discontinuous mechanisms. That is UE should perform a transmission to RRC Idle mode. In our view, the UE is not required to go to RRC IDLE as for discontinuous coverage. Indeed, the time scales between the IoT NTN TDD guard time and the discontinuous coverage are not the same, 50ms in regard to a few seconds. So the transition RRC CONNECTED to RRC IDLE mode is less justified for IoT NTN TDD.

The UE is not required to go to RRC IDLE as for discontinuous coverage during the DL to UL guard period.

The modified frame structure adopted by RAN is illustrated in the following figure with the set of subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames, implying that subframes #1 and #2 are never transmitted.

Figure 2 - Modified frame structure for the new IoT-NTN TDD mode in bottom for SIB1-NB with 16 repetitions

## 2.1	System Information Scheduling

RAN2#129 agreements suggested waiting for further RAN1 progress before discussion of the SI message transmission.

Proposal 4: When operating in IoT-NTN TDD mode, there is no need to update the SIB-1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time. Proposal 5: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time. - QC thinks that it’s possible to either postpone or drop some transmission and thinks we could leave this to RAN1. Nokia agrees. Vivo/CATT agree We wait for further RAN1 progress before further discussing this Proposal 6: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity. We wait for further RAN1 progress before further discussing this Proposal 7: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window. We wait for further RAN1 progress before further discussing this Proposal 8: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length. We wait for further RAN1 progress before further discussing this Proposal 9: In IoT-NTN TDD mode, by making modifications to the SI scheduling mechanism, all SI messages can be delivered to the UE. This can be achieved by updating 36.331 §5.2.3a as follows: We wait for further RAN1 progress before further discussing this

Meanwhile, RAN1#120 concluded that the SI message transmission in the presence of non-D subframes has to be solved by RAN2 specifications:

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

RAN2 to discuss how to handle SI message scheduling and transmission. If any RAN1 impacts on the RAN2 solution, send a LS to RAN1.

The SIBs other than SIB1-NB that are required for the NB-IoT NTN system are (including those for the mobility):

- SIB2-NB : contains common radio resource configuration

- SIB5-NB : contains neighbouring cell related information for inter-frequency cell re-selection (in IDLE mode)

- SIB22-NB : contains common radio resource configuration information for paging

- SIB31-NB : contains NTN configuration information

The SIBs-NB are transmitted over NPDSCH in SI-Message within a periodically occurring SI-Window. Up to 8 SI-Message can be scheduled in SIB1-NB. The SIBs-NB transmitted in a SI-Window are indicated with the schedulingInfoList and the sib-MappingInfo. Note that SIB2-NB is always present in the first SI message listed in schedulingInfoList

The time-domain related configurations are:

- si-WindowLength (160 to 1600 ms) that indicates the SI-Window length where the SI-Message is transmitted

- si-RadioFrameOffset (0..15 radio frames) indicates the radio frame offset to calculate the SI-Window w.r.t. SFN

- si-Periodicity (640 to 40960 ms) indicated the periodicity of the SI-window in the SIB1-NB periodicity of 4096 radio frames

- si-RepetitionPattern ({everyRF,every2ndRF,every4thRF,every8thRF,every16thRF,every32ndRF,every64thRF,spare1}) indicates the repetition pattern within the SI-Window, e.g. every 2nd RF means that every 2 radio frames within the SI-Window, the SIB is transmitted. For a SI-WindowLenght of 160 ms, it implies 8 repetitions within the SI-Window of the SI-Message

- si-TB (56 to 680 bits) is used to determine the Transport Block Size of the SI-Message. It is related to the time domain : when the TBS <= 120 bits, the SI-Message is transmitted over the 2 first consecutive valid subframes SF; otherwise, the SI-Message is transmitted over the 8 consecutive valid subframes in the radio frame.

| SI configuration Parameters | value |
| --- | --- |
| si-Periodicity-r13 ENUMERATED {rf256,rf512,rf1024,rf2048,rf4096,rf8192,rf16384,spare} | 256 |
| si-RepetitionPattern-r13 ENUMERATED {everyRF,every2ndRF,every4thRF,every8thRF,every16thRF,every32ndRF,every64thRF,spare1} OPTIONAL -- Need OP | 0 |
| si-WindowLength-r13 ENUMERATED {ms160,ms320,ms640,ms960,ms1280,ms1600,spare2,spare1} | 160 |
| si-RadioFrameOffset-r13 INTEGER (1..15) OPTIONAL -- Need OP. not present = 0 | 0 |

The SI-Window starts at subframe #0 in the radio frame, from which 
(H-SFN*1024 + SFN) mod T = FLOOR(x/10) + Offset
where T is the si-Periodicity, and Offset is the si-RadioFrameOffset [5]-§5.2.3a.

After that, the NB-IoT NTN UE receives and accumulates SI message transmissions on DL-SCH over the usable NPDSCH downlink subframes until the end of the SI-window according to the si-RepetitionPatternp. The usable downlink subframes correspond to subframes not used for NPSS, NSSS, MIB-NB and SIB1-NB. Note that if SI-message was unable to decode from the accumulated SI-message transmissions by the end of the SI-Window, the reception can continue on the next SI-window occasion.

In IoT NTN TDD, The non D subframe is not an usable NPDSCH usable downlink subframe

The above configurations with example values are depicted below within the TDD pattern (note that any configuration of SI-WindowLength is aligned with the DL-UL-Periodicity of 90 ms):

Figure 2 - SI-Window within the TDD pattern with configurations : SI-WindowLenght=160ms; SI-Periodicity=64rf; repetitionPattern=every2ndRF

In the configuration above, on 2 consecutive SI-windows, only one SI-Message radio frame is transmitted in a usable DL (in yellow). RAN1 still discusses the maximum TBS that can be decoded by the UE for a given configuration that determines the number of available repetitions within a BCCH modification period (see [5]-§5.2.1.3). However, it is possible to find a configuration that ensures a minimum transmission within the IoT-NTN TDD transmission pattern.

There are SI-Window configurations that ensure transmission of the SI-Message

Another issue raised by the SI-message transmission related to the TDD pattern comes from the TBS. Indeed, when the TBS <= 120 bits, 2 SFs within the SI-message radio frame are needed for transmission. TDD pattern can afford this even with 8 contiguous usable SF. But for TBS>120 bits, 8 SFs are needed. Only a maximum of 6 are available according to the DL subframe option selected. Therefore, a solution could be to : introduce a postponing mechanism for SI-message, i.e. for maximum up to 680 bits (which may contain several SIBs-NB), describe a UE behavior where the UE receiving less than 8 SF in an active DL frame waits for the next active DL frame inside the SI-Window to receive the remaining SF.

The solution is illustrated below:

Figure 3 - Postponement of SIBs subframes principle

In the example given above, we see that the 8 SFs corresponding to a SI-message transmission could not fit in a DL subframe. Moreover, there is an offset A between the beginning of the SFN and the beginning of the transmission. It is assumed that the subframe #0 for the start of the SI-Window corresponds to the subframe#0 of the SFN#2 that is not transmitted. The UE will start to receive and accumulate at the subframe#3 instead of the subframe#1. The offset A is then implicit by the fact that subframe#1 and #2 are not a valid D subframe. It is totally feasible in the specifications since the postponement is made in a contiguous subframe excluding the non-D subframe gap. To limit the UE search space for SI-message, the SI transmissions cannot be postponed beyond the end of the corresponding SI-Window.

The SI-message transmission can be postponed to the next valid D subframe within the SI-Window

The problem is, the postponed SI-message subframe could collide with the transmission of a repetition according to the si-RepetitionPattern within the SI-window. Two options are possible:

- Option 1 : If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped

- Option 2 : If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the repetitions corresponding to the second radio frame are dropped and the repetitions corresponding to the first radio frame are not dropped

In option 1, possibly only 6 SFs of the first repetitions are transmitted and in the other option, it is the transmissions of the second repetition that are dropped.

Down select one of the transmission options when SI messages overlap:

- Option 1: If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped

- Option 2: If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the repetitions corresponding to the second radio frame are dropped and the repetitions corresponding to the first radio frame are not dropped

## 2.2	Paging

The IoT-NTN UE is expected to be configured with a discontinuous reception (DRX) for paging for energy saving purposes, i.e. Idle DRX (I-DRX) and/or extended DRX (eDRX). The eDRX is supported for IoT NTN TDD system.

Proposal 2: RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. - DT thinks the expected performance for this constellation should be similar to others and then supports this Agreed

During (e)DRX, the UE is still reachable by the network if a notification at the AMF is received for an MT data through paging. In I-DRX and eDRX, the paging is made through NPDCCH with Type 1 common search space. Paging occurs on a limited number of occasions during Paging Occasions (PO) within Paging Frames (PF). One Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). The paging message can contain up to 16 paging records (paging UE identity, e.g. IMSI). When DRX is used, the UE needs only to monitor one PO per DRX cycle. The paging occasions are configured through SIB2-NB in PCCH-Config-NB with:

- defaultPagingCycle-r13 (T) : 128, 256, 512, 1024 radio frame

- nB-r13 (nB for Paging frame) : (4T, 2T,… 1/1024T) paging cycle

- npdcch-NumRepetitionPaging-r13 (R) : number of repetitions to be applied, (1..2048) repetitions

From which are derived the 3 scheduling parameters (in addition to the UE_ID):

- N the number of PFs within DRX cycle, i.e. min(T, nB)

- Ns the number of POs within PFs, i.e. max(1, Nb/T)

- i_s the index of PO table

The Paging frames are separated by the derived paging cycle (minimum between SIB2-NB default paging cycle and UE-specific paging cycle) and the paging occasions are determined from the Paging Occasion table in [8]-§7.2-1 (FDD NPDCCH). For Ns=1 (one paging occasion per paging frame), the PO is located on the SF#9, For Ns=2 the PO is located on SF#4 or 9 according to i_s and on SF#0, #4, #5, #9 for Ns=4.

The Paging Occasion(s) subframes within a Paging Frame are transmitted, in SF#0, #4, #5, #9 that are valid subframe in the IoT NTN TDD radio frame structure.

No adaptation of the PO subframes is needed

After, there is the eDRX that was introduced for longer sleep of NB-IoT UEs. It is configured by the Core Network with 2 parameters [9]-§10.5.5.32 controlling the eDRX paging cycle:

- Paging Time Window (PTW) : 2.56 seconds (256 radio frames) to 40.96 seconds (4096 radio frames)

- eDRX value or T_eDRX : 10.24 seconds (1024 radio frames) to 10485.76 seconds (1024x1024 radio frames). The eDRX uses the H-SFN instead of SFN.

The eDRX is also used for system information update notification. So, for NB-IoT, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0. In the case of de-synchronization between the UE and the eNB, the UE may reacquire H-SFN through MIB to avoid repeated paging failure. RAN1 agreed not to modify the H-SFN timing, the UE may derive the TDD structure based on observation of downlink signals to determine the offset between the 90 ms periodic pattern and the H-SFN.

Conclusion For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals. No specification impact.

The H-SFN is not modified for the IoT NTN TDD frame structure

I-DRX and eDRX cycles are depicted in the Figure below:

Figure 13 - I-DRX and eDRX with IoT-NTN TDD pattern with, Paging cycle = 128, nB = 1T, R = 1, PTW = 256, eDRX cycle = 1024, a given UE_id and TDD pattern N=9 (note that PF does not necessarily match with the beginning of the PTW)

We see that a paging frame does not necessarily match with the TDD pattern. There is no configuration that ensures always aligning the 90 ms IoT-NTN TDD pattern with the paging cycle.

There exists a set of configurations that allow transmission of paging but no configuration can align IoT-NTN TDD pattern N=9 and paging cycle

In [8], it is defined the following:

“In case of P-RNTI transmitted on NPDCCH, PO refers to the starting subframe of NPDCCH repetitions unless subframe determined by PO is not a valid NB-IoT downlink subframe then the first valid NB-IoT downlink subframe after PO is the starting subframe of the NPDCCH repetitions.”

The mechanism of postponement discussed for other SIBs-NB scheduling can also be applied for the Paging Occasion. If the first subframe determined by PO is not a valid NB-IoT downlink subframe e.g. in the TDD guard time period, the PO is postponed to the next valid NB-IoT downlink subframe.

RAN2 to discuss how to clarify the UE behaviour on the postponement of a Paging Frame (so the Paging Occasion(s)) in the 3GPP TS 36.304

## 2.3	Kmac extension

According to the actual satellite constellation deployments that are likely to adopt the IoT-NTN TDD pattern and the constraints on the scheduling due to rare DL subframes, it would be beneficial to increase the Kmac to allow more flexibilities to cope with extra latency. It is also beneficial for upcoming field deployments where the reference point is located in the serving satellite but where, for a question of gateway diversity, the gNB can be located on the ground at several hops from the serving satellite. With a one-bit extension of Kmac field, we would be able using up to 1024 ms RTD between the base station and the reference point of uplink sync, which would be sufficient for the future deployments.

RAN2#129 agreements:

Proposal 11: Increase Kmac, effectively either by increasing defined value from 9 to 10 bits, or by defining another optional parameter “Kmac_extension”, and the two together would allow for larger delay. - QC supports a new parameter - IRDM thinks this could also apply to other satellite deployment - Thales also thinks this could apply to other cases - Samsung thinks we should also check if there is any other implication due to the multi-hop scenario. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

The Kmac range was agreed in RAN1#107-e [11]. The 9 bits was a compromise between the bit size of the SIB and the range of the considered maximum RTD to support all the transparent scenarios including the GSO. There were no further issues raised to have 276 or 1024 ms for Kmac. The Kmac is mainly used for UE to estimate the UE-eNB RTT (38.321 definition) i.e. NPDCCH UE-specific search space monitoring after Msg3, NPUSCH transmission using PUR, NPUSCH (HARQ feedback), EDT, NB access preamble for SR or DLoffset for HARQ RTT timers, RA response window start, MAC contention resolution timer, PUR response window timer start, DRX Active Time on SR pending. All these procedures are not subject to a time limitation that could avoid the extension of the Kmac.

Support k-Mac-r19 with a value range of 1 – 1023 ms

## 2.4 	Random Access procedure

Random Access procedure is critical for a workable, deployable and scalable NB-IoT NTN system. RAN2 agreed to discuss the RAR window (and contention resolution window) in regard to the modified frame structure. RAN2#129 agreements:

Proposal: RAN2 to study aspects such as PF determination, SI message scheduling, UL gap, starting point of PUR response window in this WI. - Nokia thinks we should discuss RACH procedure in general, not just PUR. Xiaomi agrees - Samsung wonders in PUR is useful RAN2 can continue the discussion also on RAR window in the next meeting

The NPRACH transmission timing is mainly driven by the nprach-Periodicity, the nprach-StartTime and the number of repetition per npreamble attempt (i.e. numRepetitionsPerPreambleAttempt). [36.213]-§10.1.6 indicates that a “NPRACH transmission can start only [nprach-StartTime] ms after the start of a radio frame fullfiling nf mod ([nprach-Periodicity]/10) = 0”. The issue is that the nprach should not occur on a U NB-IoT subframe because of the configurations of the above parameter. Note that RAN1 states the issue and while decided between the below options:

Agreement For NPRACH, further consider the two options until RAN1#120bis: - Option 1: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD. - Option 2: NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s) - FFS: The unit of postponement (e.g. PRU)

Option 2 requires coordination with RAN2 because the eNB and the UE need to have the same understanding on the transmission subframe for the Msg1 preamble.

It is up to RAN1 to decide how to adapt NPRACH transmission in the IoT NTN TDD frame structure. FFS if postponement option 2 is adopted in RAN1

If postponement option 2 is adopted in RAN1, there is a clarification needed for the calculation of the RA-RNTI associated with the NPRACH :

RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id, where SFN_id is the index of the first radio frame of the specified PRACH (for NB-IoT UEs in FDD). The MAC entity may stop monitoring the Random Access Respons(s) (RAR) after successful reception of a RAR containing Random Access Preamble identifiers that match the transmitted Random Access Preamble. The SFN index should match between the UE and the eNB. If there is a postponement, the preable index selected by the UE for the transmission, based on the configuration transmitted in the SIB2-NB, may lead to a different effective transmission on a valid U-subframe. So a clarification is needed to avoid a mismatch between the RA-RNTI calculated by the UE and the eNB.

The SFN_id used to calculate the RA-RNTI is based on the SFN of the NPRACH occasion even if the NPRACH occasion is postponed to a valid U-subframe

Figure 4 - RACH procedure with IoT NTN TDD procedure

The Random Access procedure is initiated by the UE after the reception of the necessary cell and network information in MIB-NB, SIB1-NB, SIB2-NB and SIB31-NB to connect to the network. As illustrated in Figure 4, after the NPRACH (msg1), the UE expects the msg2 after the estimated UE-gNB RTT in a Random Access Response (RAR) window configured by ra-ResponseWindowSize-r13 (RACH-Info-NB-r13). The window size is configured in the number of NPDCCH periods from 2 to 10 periods.

Figure 12 – NPDCCH Period with TDD pattern N=9, withn Rmax=4, G=4

The NPDCCH period is driven by the Rmax (npdcch-NumRepetitions-RA-r13 [1, 2, ...2048]) and G (npdcch-StartSF-CSS-RA-r13 [1.5, 2, 4, 8, 16, 32, 48, 64] in FDD mode) values. The value adaptation of the NPDCCH periodicity is still discussed in RAN1:

- FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD.

RAN1 still discusses if new periodicities for NPDCCH monitoring are introduced

However, we can affirm that a wisely chosen configuration of Rmax and G allows the RAR window to cover multiple DL transmission opportunities and, therefore, multiple opportunities for the eNB to transmit the msg2 within a time window where the UE is waiting for.

The same reasoning can be applied to the mac-ContentionResolutionTimer-r13 for msg4 (from 1 to 64 NPDCCH periods).

The RAR window and the MAC contention resolution timer can be configured to up to 10 NPDCCH cycles and 64 respectively. According to the NPDCCH cycles configuration it is sufficient to ensure multiple DL subframe transmissions w.r.t. the IoT-NTN TDD pattern

From RAN2 perspective no enhancements on RAR window and the MAC contention resolution timer configurations are needed.

## 2.6 	Cell Barring

RAN2#129 discussed the cell barring procedure for IoT NTN

Proposal 1: The IE cellBarred in the SIB1-NB message can be reused to control whether a UE supporting NB-IoT NTN TDD mode is allowed to access an NB-IoT TDD NTN cell. - vivo supports this. Nokia agrees Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

Since the IoT NTN TDD as defined in the WI is only applicable to NTN, the cellBarred in SIB1-NB is set to “barred” and the cellBarred-NTN is used for barring as legacy. How a UE not supporting IoT NTN TDD mode can be prevented connecting to a NTN cell in IoT NTN TDD mode is left for further progress in RAN1. Either configuration information can be used to determine the system is in IoT NTN TDD mode (e.g. the DL-UL-gap fixed ber band) and so, by implementation, the UE can detect the cell is operating in IoT NTN TDD mode or the UE behaviour is left to the implementation.

The cellBarred-NTN bit is used for cell barring. FFS if an indication is added for UE to avoid connecting to a cell operating in IoT NTN TDD mode

# 3	Conclusion

In this contribution, the following observations and proposals was provided:

<UE behaviour during gap>

- The UE is not required to go to RRC IDLE as for discontinuous coverage during the DL to UL guard period.

<Other SIBs>

RAN2 to discuss how to handle SI message scheduling and transmission. If any RAN1 impacts on the RAN2 solution, send a LS to RAN1.

- In IoT NTN TDD, The non D subframe is not an usable NPDSCH usable downlink subframe

- There are SI-Window configurations that ensure transmission of the SI-Message

The SI-message transmission can be postponed to the next valid D subframe within the SI-Window

Down select one of the transmission options when SI messages overlap:

- Option 1: If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped

- Option 2: If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the repetitions corresponding to the second radio frame are dropped and the repetitions corresponding to the first radio frame are not dropped

<Paging>

The Paging Occasion(s) subframes within a Paging Frame are transmitted, in SF#0, #4, #5, #9 that are valid subframe in the IoT NTN TDD radio frame structure.

No adaptation of the PO subframes is needed

The H-SFN is not modified for the IoT NTN TDD frame structure

There exists a set of configurations that allow transmission of paging but no configuration can align IoT-NTN TDD pattern N=9 and paging cycle

RAN2 to discuss how to clarify the UE behaviour on the postponement of a Paging Frame (so the Paging Occasion(s)) in the 3GPP TS 36.304

<k-Mac extension>

Support k-Mac-r19 with a value range of 1 – 1023 ms

<Random Access procedure>

It is up to RAN1 to decide how to adapt NPRACH transmission in the IoT NTN TDD frame structure. FFS if postponement option 2 is adopted in RAN1

The SFN_id used to calculate the RA-RNTI is based on the SFN of the NPRACH occasion even if the NPRACH occasion is postponed to a valid U-subframe

RAN1 still discusses if new periodicities for NPDCCH monitoring are introduced

The RAR window and the MAC contention resolution timer can be configured to up to 10 NPDCCH cycles and 64 respectively. According to the NPDCCH cycles configuration it is sufficient to ensure multiple DL subframe transmissions w.r.t. the IoT-NTN TDD pattern

From RAN2 perspective no enhancements on RAR window and the MAC contention resolution timer configurations are needed.

<Cell Barring>

The cellBarred-NTN bit is used for cell barring. FFS if an indication is added for UE to avoid connecting to a cell operating in IoT NTN TDD mode

# References

RP-243293, Revised WID on introduction of IoT-NTN TDD mode, RAN#106, 2024-12, Madrid, ES.

RP-242652, Status Report - WID on introduction of IoT-NTN TDD mode, Iridium, RAN#106, 2024-12, Madrid, ES.

R1-2409440, Discussion on IoT-NTN TDD mode, Thales, RAN1#119, 2024-11, Orlando, US.

3GPP TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”, v18.3.0.

3GPP TS 36.331 “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”, v18.4.0.

3GPP TS 36.306 “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities”, v18.4.0.

3GPP TS 36-212 “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding”, v18.1.0.

3GPP TS 36.304 “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”, v18.3.0.

3GPP TS 24.008 “Mobile radio interface Layer 3 specification; Core network protocols; Stage 3”, v18.7.0.

3GPP TS 36.300 “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”, v18.4.0.

R1-2200002, Report of RAN1#107-e meeting, RAN1#107-bis-e, 01-2022.