3GPP TSG-RAN WG2 Meeting #129bis	           R2-2502240

Wuhan, China, April 7th – April 11th, 2025

Agenda item:	8.1.2.3

Source:	China Telecom

Title:	Performance monitoring for positioning use case

Document for:	Discussion

- Introduction

RAN2 has not discussed performance monitoring so far. From our point of view, RAN2 can discuss the control of performance monitoring based on RAN1’s progress. In this paper, we mainly discuss performance monitoring for UE-sided model case 1 for positioning accuracy enhancement.

- Discussion

RAN1 has the following agreements about performance monitoring for UE-sided model case 1:

RAN1#116bis: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. For performance monitoring for functionality-based LCM, further study on details of type 1,2 and 3, e.g., potential specification impact, pros/cons aspects. - To clarify the boundary between type 1 and type 3 - To clarify definition of monitoring output and performance metric RAN1#118: Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. RAN1#119: For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

According to RAN1 agreements, for case 1, two types of performance monitoring are discussed, i.e. target UE performs monitoring metric calculation, or LMF performs monitoring metric calculation. Target UE performs monitoring metric calculation is supported by RAN1, and LMF performs monitoring metric calculation is FFS. We will discuss the network control of performance monitoring based on target UE performs monitoring metric calculation. From our point of view, RAN2 can discuss the network control of performance monitoring from the following aspects:

- When to report monitoring result

- Who determine whether the monitoring performance meets the requirement or not

- Performance feedback

Generally, we think it is up to LMF to decide the required performance metric. As for target UE performs monitoring metric calculation, target UE is aware of the calculated performance metric. It is possible that the LMF is only interested in the situation where the performance metric calculated by target UE is low. It is beneficial that target UE reports monitoring result when the calculated performance metric is lower than a threshold, even though the LMF has not released or deactivated the monitoring configuration, so that the network can react when the performance metric is low enough. In that way, the LMF is able to configure target UE to stop monitoring in advance, which is good for UE power consumption and the overhead of monitoring result reporting. If target UE is not triggered to report the calculated performance metric in advance. The LMF can request target UE to report monitoring result when the monitoring duration is long enough.

Proposal 1: For case 1, UE can be triggered or requested by LMF to report monitoring result.

In our understanding, the performance requirement is determined by the LMF, e.g. prediction accuracy. It is possible the LMF determines whether the calculated performance metric meets the requirement or not based on the monitoring result (i.e. calculated performance metric) reported by target UE.

Figure 1

Another potential option is that target UE can determine whether the calculated performance metric meets the requirement or not by comparing the calculated performance metric and the required performance metric, given that the LMF can configure the required performance metric to UE in advance. Then target UE can report the monitoring result as an indication of whether the required performance is satisfied or not to the LMF.

Figure 2

Proposal 2: For case 1, there are two alternatives for the content of monitoring result:

- Option 1: calculated performance metric

- Option 2: indicator of whether required performance is satisfied or not

In previous discussion related to applicable functionality for UE-sided model, it is assumed that UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. In our understanding, UE need to be aware of whether the monitoring performance metric meet the requirement or not, in order to confirm whether the model be monitored is still available or not. Furthermore, UE can confirm if there is other available model and determine whether the corresponding functionality is still applicable or not. Based on the above reason, for option 1 listed above, UE need to be notified when the LMF determines that the monitoring performance metric does not meet the requirement based on monitoring result report.

Proposal 3: For case 1, UE need to be notified if LMF determine that the monitoring performance metric does not meet the requirement.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: For case 1, UE can be triggered or requested by LMF to report monitoring result.

Proposal 2: For case 1, there are two alternatives for the content of monitoring result:

- Option 1: calculated performance metric

- Option 2: indicator of whether required performance is satisfied or not

Proposal 3: For case 1, UE need to be notified if LMF determine that the monitoring performance metric does not meet the requirement.