3GPP TSG-RAN WG2 Meeting #129bis	R2-2502364

Wuhan, China, April 7 – April 11, 2025

Agenda item:	8.7.4.2

Source:		Lenovo

Title:			Enhanced delay status reporting for XR

Document for:	Discussion and Decision

- Introduction

In this contribution we discuss delay status reporting enhancements for XR traffic.

- Discussion

In RAN2#129 the following agreements were reached on DSR enhancements:

- One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

Last meeting, RAN2 has discussed whether New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. For the New DSR MAC CE format, if one LCG is configured with reporting thresholds, UE should use the new DSR format for simple handling since this new MAC CE format with extension flags is also compatible for the Rel-18 delay status reporting. For example, the EXT field associated with a pair of delay status information is set to zero, which indicates there is no further pair of delay status information follows for the LCG with R18 DSR configured.

Proposal 1: If at least one LCG is configured with Rel-19 DSR reporting thresholds and there is data to be reported for an LCG in a triggered DSR, UE should use the new DSR format for all LCGs, wherein the EXT field is set to zero.

For the enhanced DSR MAC CE format, if there’s multiple pair of delay status information for an LCG, it’d better place the delay status information for an LCG in increasing order of the associated reporting thresholds. In other words, UE places the delay status information in increasing order of the shortest remaining time associated with the pair of delay status information, so that it is beneficial for network to analyse and handle if the content of the MAC CE would be more deterministic.

Proposal 2: UE places the delay status information in increasing order of the value of the remaining time filed.

One of the open issues is the reporting of PDCP control PDUs, the PDCP SDUs to be retransmitted and the PDCP Data PDUs to be retransmitted within a DSR. It is in particular FFS in which delay-reporting PDCP data volume PDCP control PDUs and retransmission PDCP SDUs/PDUs should be reported. In our view retransmission PDCP PDUs/SDUs should be treated in the same way as any other (initial) PDCP SDUs/PDUs. Since those retransmission PDCP PDUs/SDUs have a corresponding PDCP discard timer running, the retransmission PDUs/SDUs should be reported according to their remaining delay. Since PDCP retransmission are anyway happening only during specific events such as PDCP data recovery or PDCP re-establishment, we don’t think that any special handling is necessary. It would be better to just treat some w.r.t to DSR reporting as any other PDCP SDU/PDU.

Proposal 3: RAN2 to agree to report PDCP Data PDUs/SDUs to be retransmitted in the delay-reporting data volume according to their corresponding remaining delay value, e.g. treat them as any other PDCP SDU/ data PDU.

Since PDCP control PDUs don’t have an associated PDCP discard timer, we agree with the rapporteur’s suggestion to report them in the first PDCP delay-reporting data-volume.

Proposal 4: RAN2 to agree to report PDCP control PDUs in the first PDCO delay-reporting data volume, e.g. i=1.

- Conclusion

In this contribution, we discuss delay status reporting enhancements. We have the following proposals:

Proposal 1: If at least one LCG is configured with Rel-19 DSR reporting thresholds and there is data to be reported for an LCG in a triggered DSR, UE should use the new DSR format for all LCGs, wherein the EXT field is set to zero.

Proposal 2: UE places the delay status information in increasing order of the value of the remaining time filed.

Proposal 3: RAN2 to agree to report PDCP Data PDUs/SDUs to be retransmitted in the delay-reporting data volume according to their corresponding remaining delay value, e.g. treat them as any other PDCP SDU/ data PDU.

Proposal 4: RAN2 to agree to report PDCP control PDUs in the first PDCO delay-reporting data volume, e.g. i=1.

- References

### TS 38.321 v18.0.0