3GPP TSG-RAN WG2 #129bis	R2-2502904

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.1.3

Source:	Ericsson

Title:	NW-side data collection for beam management and positioning

Document for:	Discussion

# 1	Introduction

The discussion on the topic of NW-side data collection has progressed during the last RAN2 meetings. The discussion has focused on different technical aspects, such as when the UE should start/stop the NW-side data collection, how the UE should report the logged data, etc.

# 2	Discussion on NW-side data collection for beam management

The email discussion “[POST129][025][AI PHY] RRC running CR (Ericsson)” focused on the first draft of the RRC running CR for AI/ML for PHY. Based on the comments from multiple companies and the rapporteur’s view, there are the following main open issues:

- Where in the ASN.1 structure to introduce the logging configurations, e.g. directly under CSI-MeasConfig or in CSI-ReportConfig, and how the L3 event based configuration is provided;

- Where/how to capture the procedures for logging data in a variable at the UE;

- Triggers for logged data availability indication and dependencies on buffer full and/or low power indications;

- SRB on which the UE reports the logged data;

- (Need for) content in the UAI configuration in otherConfig;

- When to discard logged data at handover.

RAN2 to discuss the following open issues identified based on the running RRC CR:

- Where in the ASN.1 structure to introduce the logging configurations, e.g. directly under CSI-MeasConfig or in CSI-ReportConfig, and how the L3 event based configuration is provided;

- Where/how to capture the procedures for logging data in a variable at the UE;

- Triggers for logged data availability indication and dependencies on buffer full and/or low power indications;

- SRB on which the UE reports the logged data;

- (Need for) content in the UAI configuration in otherConfig;

- When to discard logged data at handover.

In the following, we further discuss these open issues and other FFSs from previous meetings.

## 2.1	Logging configuration aspects of the data collection report

In RAN2#127, it was agreed that when the network wants to perform NW-side data collection, it will transmit to the UE a measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. For the case of beam management, the NW may configure the UE with a specific set of CSI-RS/SSB resources specifically configured for training purposes. On such resources, the UE will then start performing layer-1 measurements and log the measurement results and any possible useful information associated to it.

From the configuration point of view, the radio resource configuration for the purpose of NW-side data collection consists of a list of logging configurations (e.g. each logging configuration can be associated to a periodic or event-based logging conditions), each associated to a CSI resource configuration (i.e. to one CSI-ResourceConfig IE) comprising one or more CSI-RS/SSB resource sets in which the UE performs the radio measurements.

In the running CR discussion, concerns have been raised by companies regarding the design choice of creating a new IE for the logging configuration, where the claim is that same effect can be achieved by enhancing the existing CSI-ReportConfig IE. We note that the legacy CSI-ReportConfig is used in TS 38.214 to report the L1 measurement results in the UCI, as per the description of the IE CSI-ReportConfig:

“ The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included.“

However, the above is not applicable for the NW-side data collection configuration, since it was agreed in RAN2 that the UE should not transmit the data using UCI, rather it needs to store the collected measurements and transmit them to network using RRC signalling. Furthermore, one major problem if using CSI-ReportConfig is that it contains a mandatory list of report configuration types, i.e., reportConfigType IE, which is not extendable. Thus, it is not straightforward to include conditions that enable the UE to log the collected measurement results.

Reusing CSI-ReportConfig to configure the UE with NW-side data collection configuration leads to impact on legacy beam management procedure and creates additional constraints on the data collection procedure.

Thus, in our view, it is more beneficial to separate the data collection configuration from the legacy CSI-ReportConfig IE. Such a design avoids dependency on the legacy procedures and enables easy integration of future releases. Specifically, we propose to create an extension of the L1 CSI-MeasConfig framework and decouple the additions for NW-side data collection configurations from the legacy dependencies. A visualization of the legacy and proposed extension is shown in Figure 1.

The radio resource configuration for the purpose of NW-side data collection consists of a list of logging configurations within the legacy CSI-MeasConfig and outside the legacy CSI-ReportConfig.

Each logging configuration contains a reference CSI resource configuration (i.e. a reference to a CSI-ResourceConfig IE) on which the UE shall perform radio measurements and the related logging.

Figure 1: Framework for NW side data collection

Related to the logging configuration, RAN2 agreed in RAN2#129 to support L3-based data logging.

From RAN2#129: Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log

How to refer to the L3 measurement configurations is an issue that RAN2 should discuss. In our view, two solutions can be envisaged:

- refer to a configured measurement identity (measId) which includes both the measurement object and the reporting configuration

- refer to a configured measurement object (measObjectId).

Both the configurations of the measurement identity and of the measurement object are in the realm of the gNB-CU, whereas the CSI logging configuration should be in the realm of the gNB-DU, since the data collection is for the inference at the gNB-DU. Hence, if option 1) above is selected, then it will not be possible for the gNB-DU to configure the L3 event as it wants, e.g. if the gNB-CU does not configure any L3 events for a certain frequency of interest, or it configures L3 events that are not of interest for the gNB-DU, then it will not be possible for the gNB-DU to perform the desired data collection. Given this consideration, we prefer the option 2 above, i.e. the gNB indicates in the CSI logging configuration the measurement object that the UE should monitor, and the related thresholds as well as other parameters (e.g. time-to-trigger).

For L3-event based data collection, the logging configuration contains a reference to a measurement object and the thresholds (and other possible parameters to be discussed) that the UE should monitor for the triggering of the data collection.

Furthermore, RAN2#128 agreed:

2. Measurements on aperiodic CSI resources are not reported for NW sided data collection.

Since this agreement explicitly excludes aperiodic CSI resources from the NW-side data collection, the question may arise of whether both periodic and semi-persistent CSI resources should be supported for NW-side data collection reports. From our perspective there is no reason to exclude semi-persistent CSI resources. Also, we think that the supported cases for NW-side data collection should be aligned with those agreed for UE-side data collection, since they both have the purpose of model training. Specifically, if semi-persistent resources are supported for UE-side data collection, there is no reason why they should not be supported for NW-side. From a technical point of view, this allows the gNB to activate the NW-side data collection based on L1 measurements received over the UCI. Especially since the NW-side data collection based on L1 events is not supported, then there is the need to give the possibility to the NW to dynamically activate the data collection based on real-time L1 measurements received from the UE. Otherwise, there is the risk with periodic resources that the gNB collects a lot of redundant data, that were already previously collected.

Measurements on periodic and semi-persistent CSI resources are collected by the UE for the NW-side data collection (as for the UE-side data collection).

Related to the logging periodicity, we believe that it should be controllable by the network, in order to reduce the amount of data to be logged and the impact on the UE. For example, the CSI-RS/SSB periodicity could be 80 ms as per the CSI resource configuration, but for periodic logging for NW-side data collection, it might be enough to just perform measurements (and hence logging) every 640 ms. And for example, if a certain event is triggered, the periodicity of logging can be increased to e.g. 160 ms, in order to provide more granular results to the network.

The logging periodicity/interval for both periodic and event-triggered data collection is configured in the L3 logging configuration.

## 2.2	Content of the data collection report

Related to the content of the data collection report, in RAN2#127-bis, it was agreed that “at least L1-RSRPs and/or beam-IDs needs to be collected by UE”. In our view, along with such information, also the timestamp of the measurement might be beneficial, especially for the case of event-based measurements. The timestamp of a logged measurement is already included in the logged MDT report, as a tool to assist the network in retrieving NW operating conditions at the moment in which the measurement was taken by the UE. In the same way, in the context of AIML, a timestamp would allow the network to correlate a given UE radio measurement with the NW conditions/configurations experienced at the network, which is critical to ensure a proper AIML model setup.

This is especially useful now that RAN2 has agreed that the UE can retain the logged data during HO. It may happen that the data collected according to a data collection configuration provided by a gNB-x are fetched by a gNB-y (and then passed back to the originating node gNB-x) after multiple handovers. Since some time may have elapsed between configuration and reception of the report, if source node gNB-x has already changed its beam configuration, then the gNB-x will not be able to correlate the received report with the proper NW-side conditions at the time of data collection. If the UE includes timestamp information in the report, then it is possible to have a NW-based solution in which the source node gNB-x can remember the UE configuration/NW-side conditions and utilize the report.

Along with the L1-RSRP and beams IDs, the UE includes the timestamp related to the point in time in which a reported measurement was taken.

## 2.3	Inputs to RAN1

The necessary signalling to configure the UE to perform the NW-side data collection is captured and under discussion in the ASN.1 part of the RRC running CR. In this context, the issue of where/how to capture the procedures for logging data in a variable at the UE in procedural text was raised. In our view, the procedures related to how the UE should perform the L1 measurements and the storing/logging of such measurements (i.e. UE behaviour upon reception of CSI-MeasConfig containing CSI-LoggedMeasurementConfig, collection of measurement results based on the periodic/event triggered conditions, and storage of the measurement results in a variable in the UE memory) should be captured in RAN1 specifications. This is because, in legacy, all the procedures on how the UE performs the channel measurements on configured CSI-RS/SSB resources and transmits the related reports are captured in TS 38.214. Hence, it should be up to RAN1 to evaluate if there is any impact on that specification.

L1 procedures to perform radio measurements and logging for the NW-side data collection purpose are not captured in RAN2 specification. RAN1 to evaluate impact in TS 38.214.

Since all the above assumptions captured in previous proposals are of RAN1 interest, we suggest RAN2 informing RAN1 on that.

RAN2 to inform RAN1 about any agreements related to UE configuration for NW-side data collection, contents of the report with logged data and needed procedural text for NW-side data collection.

## 2.4	When the UE should perform/stop the radio measurements for NW-side data collection

For the stopping of data collection, it was agreed in RAN2#127 that the data collection can be stopped once the UE faces issues such as reaching the buffer limitation. However, in general, the data collection, once started, can continue for a certain configurable time (as in logged MDT), or it can continue until certain events are fulfilled, for the case of event-triggered data collection. For periodic data collection, network can de-configure the UE to stop collecting measurement results.

Besides the case of memory limitation issues (already agreed in RAN2#127), the UE may stop the event triggered NW-side data collection procedure when:

  - data collection has been done for a certain configurable logging duration

  - the event that triggered the data collection is no longer fulfilled.

## 2.5	Design of the UEInformationResponse/Request framework for AIML

One of the remining issues to discuss is which SRB to use for the transmission of the UEInformationRequest/UEInformationResponse to retrieve the logged data for NW-side data collection. The UEInformationRequest is already mapped to SRB1, so in our view SRB1 can be reused. The UEInformationResponse is instead mapped to SRB1 or SRB2 (when logged measurement information is included).

In legacy, the UEInformationRequest is mapped to SRB1, and the UEInformationResponse is mapped to SRB1 or SRB2 (when logged measurement information is included).

In meeting RAN2#127-bis it was proposed that for the UEInformationResponse the SRB4 could be used. However, in our view this proposal is not very well motivated given that SRB4 is today used for QoE, which however uses a completely different data transmission mechanisms based on RRC segmentation, and not based on the “on-demand” framework, that instead RAN2 has decided to adopt for the reporting of AIML collected data. On the other hand, SRB2 is used for the logged MDT that, as said, resembles very much the AIML use cases, both on the signalling mechanism (both based on “on-demand”) and on the prioritization (both logged MDT and AIML-related collected data can be assumed to have the same priority). Hence, our preference is to reuse the SRB2, or a new SRB that can be assumed to be used for all the AIML possible use cases from now on.

The UEInformationRequest requesting AIML data is transmitted via SRB1 (as any other data request contained in such message), and the UEInformationResponse is transmitted via SRB2, or via a new defined SRB (to be used for all AIML use cases).

## 2.6	Signalling of logged data availability indication

RAN2#128 agreed that:

- 5. The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - 6. The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

Further, RAN2#129 agreed that:

- 2. Low power bit indication is supported

- 3. Data availability indication is supported.  FFS when this would be triggered

Thus, it still needs to be clarified when the data availability indication is triggered and whether it can be combined with the indications for low power or full buffer. In our view, the data availability indication should be sent in the UAI as soon as logged data becomes available for transmission upon starting the data collection, e.g. when a logging event is fulfilled, so that the NW is informed and has enough time to start retrieving the data and start emptying the UE buffer, before other conditions (e.g. buffer limitations) result in stopping the logging.

Furthermore, in our view it is better to keep the indication for low power independent of the data availability indication, for more flexibility. For instance, the UE may want to report a low power state immediately upon receiving a data collection configuration, even before it starts logging corresponding data. Otherwise, if the UE is forced to wait until it logs some data, it will anyway log very little data and then expect the NW to de-configure the logging configuration due to the low power state. This data may thus be too small to be useful to the NW. Regarding the buffer full indication, this should also be kept separate from the data availability indication, since it has a different purpose. This is especially if the buffer is common to different NW-side data collection use cases, or it is even shared with UE-side data collection, so the UE can report overall that the buffer is full, whereas the data availability indication should differentiate the use case/configuration for which logged data for NW-side data collection is ready for transmission.

The UE sends data availability indication in the UEAssistanceInformation message as soon as there are logged data available for transmission.

The indications for data availability, low power, and buffer full are reported independently of each other.

Another issue raised during the email discussion for the RRC running CR is whether the UE needs at all a UAI configuration in otherConfig, to report the indications above. An argument for not sending any UAI configuration from the NW was that, when the NW-side data collection is configured, the UE should report the indications above (which so far are not agreed to be optional), so the UAI can implicitly be used/configured. In our view, even if all the indications above need to be supported by the UE, it is still meaningful to configure the UAI via otherConfig. This is to align the procedures with legacy cases and avoid forward compatibility issues, if a future feature needs a UAI configuration. Furthermore, it is unclear what the benefit of omitting the UAI configuration in otherConfig. It seems at the moment this would save 1 bit, which is negligible.

The UE sends NW-side data collection indications in the UEAssistanceInformation upon receiving a configuration from the NW in otherConfig.

## 2.7 Handling of collected data during mobility

In RAN2#129 meeting, the following was agreed:

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

In our view, network control is required to determine whether the UE can transmit the collected measurement reports after HO. A simple solution is to configure the UE with a flag to indicate whether it is allowed to send the measurement availability indication upon handover. This flag can be sent to the UE during configuration of the NW-side data collection procedure or in the RRCReconfiguration with sync message.

The NW-side data collection configuration includes a flag to indicate to the UE whether it shall release the logged data upon handover.

## 2.8 Handling of collected data upon RRC change

Another important FFS is if the UE should continue to store the collected measurements upon changing to RRC IDLE state or upon declaring Radio Link Failure (RLF). Some companies proposed that it should continue to store the data and indicate this in the RRCSetupComplete message.

Needless to say that storing the measurement report is certainly preferrable from a technical point of view, because it allows the original gNB to retrieve all the collected data. However, a major problem with this is that network needs to control to which cells the UE is allowed to transmit the collected data. Since the network is unable to estimate where the UE would perform cell establishment (for RRC IDLE UE) or RRCReestablishment message (for UEs declaring RLF), the NW would need to configure the UEs with a potentially long list of allowed CGIs.

If the UE stores the logged data upon entering RRC IDLE or declaring RLF, the NW-side data collection configuration needs to contain a potentially long list of CGIs in which the UE is allowed to transmit the logged data.

Furthermore, since the node retrieving the data should transmit the collected data back to the original gNB, additional information related to the original gNB might be needed, and as such RAN3 involvement would be needed. Furthermore, RAN3 /SA5 involvement would be needed to consider OAM centric data collection, i.e., type of OAM identifier (similar to TCE id in logged MDT) may need to be sent to the UE.

Given the above considerations, for simplicity we propose to limit the scope of this discussion in Rel-19.

The UE discards/clears the logged data for NW-side data collection upon RRCRelease or declaring RLF.

# 3	Discussion on NW-side data collection for positioning

Network-side data collection for positioning occurs in the following network-based positioning methods:

- Case 3a: NG-RAN node assisted/LMF-based positioning with gNB side model (direct positioning)

- Case 3b: NG-RAN node assisted/LMF-based positioning with LMF-side model (assisted positioning)

Case 3a and Case 3b both are based on UL measurements performed at the gNB and share similar flows as the legacy UL-TDOA positioning method. As such the impact is primarily in RAN3 domain NRPPa.

Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact.

There would potentially be one issue that should be discussed; gNB needs to collect the measurements as well as

RAN2 to wait for RAN3 before checking/working on if there is any impact for RAN2.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Reusing CSI-ReportConfig to configure the UE with NW-side data collection configuration leads to impact on legacy beam management procedure and creates additional constraints on the data collection procedure.

Observation 2	In legacy, the UEInformationRequest is mapped to SRB1, and the UEInformationResponse is mapped to SRB1 or SRB2 (when logged measurement information is included).

Observation 3	If the UE stores the logged data upon entering RRC IDLE or declaring RLF, the NW-side data collection configuration needs to contain a potentially long list of CGIs in which the UE is allowed to transmit the logged data.

Observation 4	Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss the following open issues identified based on the running RRC CR:

	Where in the ASN.1 structure to introduce the logging configurations, e.g. directly under CSI-MeasConfig or in CSI-ReportConfig, and how the L3 event based configuration is provided;

	Where/how to capture the procedures for logging data in a variable at the UE;

	Triggers for logged data availability indication and dependencies on buffer full and/or low power indications;

	SRB on which the UE reports the logged data;

	(Need for) content in the UAI configuration in otherConfig;

	When to discard logged data at handover.

Proposal 2	The radio resource configuration for the purpose of NW-side data collection consists of a list of logging configurations within the legacy CSI-MeasConfig and outside the legacy CSI-ReportConfig.

Proposal 3	Each logging configuration contains a reference CSI resource configuration (i.e. a reference to a CSI-ResourceConfig IE) on which the UE shall perform radio measurements and the related logging.

Proposal 4	For L3-event based data collection, the logging configuration contains a reference to a measurement object and the thresholds (and other possible parameters to be discussed) that the UE should monitor for the triggering of the data collection.

Proposal 5	Measurements on periodic and semi-persistent CSI resources are collected by the UE for the NW-side data collection (as for the UE-side data collection).

Proposal 6	The logging periodicity/interval for both periodic and event-triggered data collection is configured in the L3 logging configuration.

Proposal 7	Along with the L1-RSRP and beams IDs, the UE includes the timestamp related to the point in time in which a reported measurement was taken.

Proposal 8	L1 procedures to perform radio measurements and logging for the NW-side data collection purpose are not captured in RAN2 specification. RAN1 to evaluate impact in TS 38.214.

Proposal 9	RAN2 to inform RAN1 about any agreements related to UE configuration for NW-side data collection, contents of the report with logged data and needed procedural text for NW-side data collection.

Proposal 10	Besides the case of memory limitation issues (already agreed in RAN2#127), the UE may stop the event triggered NW-side data collection procedure when:

a.	data collection has been done for a certain configurable logging duration

b.	the event that triggered the data collection is no longer fulfilled.

Proposal 11	The UEInformationRequest requesting AIML data is transmitted via SRB1 (as any other data request contained in such message), and the UEInformationResponse is transmitted via SRB2, or via a new defined SRB (to be used for all AIML use cases).

Proposal 12	The UE sends data availability indication in the UEAssistanceInformation message as soon as there are logged data available for transmission.

Proposal 13	The indications for data availability, low power, and buffer full are reported independently of each other.

Proposal 14	The UE sends NW-side data collection indications in the UEAssistanceInformation upon receiving a configuration from the NW in otherConfig.

Proposal 15	The NW-side data collection configuration includes a flag to indicate to the UE whether it shall release the logged data upon handover.

Proposal 16	The UE discards/clears the logged data for NW-side data collection upon RRCRelease or declaring RLF.

Proposal 17	RAN2 to wait for RAN3 before checking/working on if there is any impact for RAN2.