3GPP TSG-RAN WG2 Meeting #130	R2-2503388

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.12.2

Source:	NEC

Title:	RACH Procedure for Asymmetric DL sTRP/UL mTRP

Document for:	Discussion, Decision

# Introduction

In last RAN2 meeting (RAN2#129bis), the following agreement was reached for PRACH transmission for Asymmetric DL sTRP and UL mTRP.

Agreement

For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

In the last RAN1 meeting, there was a discussion on how to determine the PL RS for the PDCCH-order PRACH transmission but there was no consensus on the following proposal identified:

If the PL offset associated with the indicated joint/UL TCI state is included in the PRACH transmission power for PDCCH-order PRACH transmission, the pathlossReferenceRS-Id in the same indicated joint/UL TCI state is used to determine PL RS for the PDCCH-order PRACH transmission.

There were also some relevant agreements reached at RAN1 for PRACH transmission in its recent meetings (see Annex).

This paper intends to discuss the RACH Procedure for Asymmetric DL sTRP and UL mTRP from RAN2 perspective.

# Discussion

In the previous meetings, the discussion on RACH procedure was focused on the PDCCH-ordered RACH procedure. However, in our understanding, in order to fully develop the advantage of Asymmetric DL sTRP and UL mTRP for the system, the UE initiated RACH procedure to UL-only TRP should be also allowed.

Figure-1: An example of Asymmetric DL sTRP/UL mTRP scenario

Proposal: 1: UE initiated RACH procedure to UL-only TRP is supported.

According to the RACH procedure specified in TS32.321, the UE measures the RSRP of downlink pathloss reference (DL PS) and compares it with a sort of RSRP thresholds, for the selection of RS type, NUL and SUL, different RACH procedures, repetition, and different PRACH resources. As shown in Figure-1, in case of  asymmetric downlink/uplink scenario, the UE may measure the RSRP of DL PS transmitted from the anchored downlink TRP and then generate PRACH transmission to an uplink only TRP. The pathloss between anchored downlink TRP and UE may be different from that between UE and UL only TRP in different TRP deployment scenarios. In practice, the pathloss difference between anchored downlink TRP and UL only TRP may also depend on the exact UE location. However, without any compensation consideration on such difference, simply applying the RSRP comparison between the measurement RSRP of DL PS and the configured thresholds for the selection of RACH procedures/ PRACH resources will be inaccurate and may cause undesirable consequence. It is proposed that the pathloss offset between anchored downlink TRP and UL only TRP be applied to the final RSRP used for the comparison with the configured RSRP thresholds on top of RSRP measurement of the DL RS sent from anchored downlink TRP, for RA procedure/RA resource selection (e.g. 4-step/2-step RACH procedure, message repetition, preamble selection, etc.,).

Proposal 2: the pathloss offset between anchored downlink TRP and UL only TRP be applied to the final RSRP used for the comparison with the configured RSRP thresholds on top of RSRP measurement of the DL RS.

Then the next question is how the UE acquires the pathloss offset between anchored downlink TRP and UL only TRP before UE initiated RACH procedure. It should be noted that the PL offsets for different UEs may be different. The network may notify some parameters for the UE to calculate the PL offset for UL only TRP.

Proposal 3: To further study how the UE acquires the pathloss offset between anchored downlink TRP and UL only TRP.

# Conclusion and Proposal

We have the following proposals:

Proposal: 1: UE initiated RACH procedure to UL-only TRP is supported.

Proposal 2: the pathloss offset between anchored downlink TRP and UL only TRP be applied to the final RSRP used for the comparison with the configured RSRP thresholds on top of RSRP measurement of the DL RS.

Proposal 3: To further study how the UE acquires the pathloss offset between anchored downlink TRP and UL only TRP.

# Annex: Relevant RAN1 agreements on PRACH

Agreement

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification

- Above is subject to a separate UE capability signaling

Agreement

For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0:

- This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset.

- When one joint/UL TCI state is indicated in Rel-17 unified TCI,

  - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power.

- FFS: Whether the bit field can be used to indicate other information

- FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI

Agreement

For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission:

- Alt2:

  - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation.

  - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed.

FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission