3GPP TSG-RAN WG2 Meeting #130	                                                                                   R2-2503411

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on Conditional Intra-CU LTM

Agenda Item:    8.6.4

Document for:  Discussion and Decision

Introduction

This paper further discusses the remaining issues on the conditional intra-CU LTM [1]. In detail, the following issues are covered by our discussion,

- On CLTM TAT handling

- NW triggered LTM to the CLTM candidate with early TA

- On L1 based CLTM

- On target beam/cell selection of L3 based RACH-less CLTM

- CG resource selection

- TCI state determination upon RACH-less CLTM execution

- On the co-existence of CLTM and other features

The detailed discussion is provided in Section 2, and followed by Section 3 that summarizes the contribution of this paper.

Discussion

On CLTM TAT handling

As agreed by RAN2, the early TA is maintained by a new TAT associated with the corresponding candidate cell. Then the current RRC running CR provides the TAT configuration for early TA within the IE EarlyUL-SyncConfig-r18

EarlyUL-SyncConfig-r18 ::= SEQUENCE { … [[ ltm-tdd-UL-DL-ConfigurationCommon-r18 TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD ltm-restrictedSetConfig-r18 ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB} OPTIONAL -- Need R ]], [[ ltm-TimeAlignmentTimer-r19 TimeAlignmentTimer OPTIONAL, -- Need R ltm-TimeAlignmentTimerTag2-r19 TimeAlignmentTimer OPTIONAL -- Cond 2TA ]] }

That means the TAT value for monitoring the valid of early TA is generated by the target cell.

However, as mentioned above, the TAT is used to monitor the valid of the value for early TA associated with a candidate cell. Thus the determination of the TAT value may consider the UE mobility status such as the UE speed. Note that this information is well known by the source cell but not the candidate cell. So we propose that the TAT value used for monitoring the early TA is determined by the source cell. And which node generates the LTM TAT value will have impact on the inter-node interaction, hence the agreement should send to RAN3.

Proposal 1: The value of ltm-TimeAlignmentTimer-r19 is determined by the source cell. Send LS about the agreement to RAN3.

NW triggered LTM to the CLTM candidate with early TA

In the current MAC open issue list, it captures the following issue,

| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

The intention is to further clarify whether UE performs RACH-based LTM or RACH-less LTM to the target cell if the target cell is a CLTM candidate associated with running TAT.

As captured in the current MAC running CR, the enhanced LTM cell switch command MAC CE follows the similar principle on the TA field, i.e., mandatory present with a valid TA value or FFF to indicate not valid. From this perspective, it can be the NW implementation to include the TA value in the LTM command MAC CE to trigger the RACH-less access to the target cell.

If the UE is allowed to further check the running of the target cell TAT to determine the RACH-less access to the target cell even if the LTM command includes the FFF value, it will cause many other potential clarifications by this enhancement. For example, how to achieve the NW intention to trigger the RACH-based LTM to the target cell by not including the TA in the LTM command (optional including the RACH resource) but the UE indeed has a running TAT associated with that target cell. In this situation, the UE behaviour is not aligned with the NW intention.

In summary, we suggest not making enhancement on this issue.

Proposal 2: (MAC-14) Upon receiving (Enhanced) LTM Cell switch Command MAC CE, UE performs RACH-less LTM cell switch only if the MAC CE includes a valid TA value or UE has performed UE based TA measurement to the target cell, regardless of whether CLTM candidate TAT timer is running for the target cell.

On L1 based CLTM evaluation

It was agreed to reuse the existing L1 events for conditional LTM.

Re-use existing L1 events for CLTM.

As per the current signaling framework for event triggered L1 measurement reporting, the execution conditions are configured by LTM-CSI-ReportConfig-r18, which is associated with the corresponding L1 measurement resource by ltm-ResourcesForChannelMeasurement-r18 defined by the IE LTM-CSI-ResourceConfigId-r18. The current signalling structure allows the NW to include the RS(s) from one or multiple candidate cells within an LTM-CSI-ResourceConfig-r18.

However, in the current RRC running CR for inter-CU LTM and conditional LTM, the L1 execution condition of one candidate cell is defined as LTM-CSI-ReportConfigId. That means the L1 execution condition of a candidate cell may be associated to the measurement RSs of other candidate cells.

LTM-ExecutionCondition-r19 ::= SEQUENCE { ltm-CandidateId-r19 LTM-CandidateId-r18, executionCondition-r19 CHOICE { l1-Conditions-r19 LTM-CSI-ReportConfigId-r18, l3-Conditions-r19 SEQUENCE (SIZE (1..2)) OF MeasId } OPTIONAL, -- Need R ... }

For example, for candidate cell 1, the configured l1-Conditions is associated with LTM-CSI-ResourceConfig-r18 which is configured with the RS from candidate cell 1, candidate cell 2, candidate cell 3. According to the LTM-CSI-ReportConfig, UE will perform event evaluation to all the RSs from candidate cell 1, 2 and 3, however, it is not reasonable to perform the event evaluation to the RS of the other candidate cells (cell 2, cell 3) when the L1 execution condition is configured for candidate cell 1.

Observation 1: In the CLTM evaluation, if the L1 execution condition of a candidate cell associated with an LTM-CSI-ResourceConfig-r18 which includes RSs from other candidate cells, then the RSs of those other candidate cells are evaluated. This seems unreasonable.

Note that in the current MAC running CR for conditional LTM,

1> for each entry within the CLTM-ExecutionConditions: 2> if the LTM3 or LTM5 is configured in the corresponding ltm-CSI-ReportConfigId for l1-Conditions: 3> consider all beams of LTM candidate cell indicated by the ltm-CandidateId within this CLTM-ExecutionConditions to be applicable; 3> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for TTT for one or more applicable beams, i.e. reference signalling associated with LTM-CSI-SSB-ResourceConfigId or LTM-CSI-RS-ResourceConfigId in the LTM-CSI-ReportConfig, for the measurement from lower layer during timeToTrigger defined for this event; 4> consider the event associated with LTM-CSI-ReportConfigId to be fulfilled for the ltm-CandidateId associated with LTM-CSI-ReportConfigId; 4> perform the CLTM execution procedure for the LTM candidate configuration associated with ltm-CandidateId according to the procedure specified in 5.y.3;

It says all beams of LTM candidate cell, but it doesn’t mention the relationship of the evaluated RS and the RSs configured within the execution condition.

To solve above ambiguity, we have the following candidate options for clarification.

- Option 1: Specify that, for the LTM-CSI-ReportConfigId configured as execution condition, the ltm-ResourcesForChannelMeasurement-r18 could only include the RS from the associated candidate cell.

- Option 2: Specify that, when performing conditional LTM evaluation, the UE only evaluates the RS within the ltm-ResourcesForChannelMeasurement-r18 associated with the l1 execution condition which is from the corresponding evaluated candidate cell.

- Option 3: UE evaluate all the RSs from the evaluated candidate cell by default.

Option3-1: LTM-CSI-ReportConfigId-r18 is configured as the execution condition, UE ignore the ltm-ResourcesForChannelMeasurement-r18 configured in the associated LTM-CSI-ReportConfig.

Option3-2: redesign the event configuration signalling structure to avoid the association between L1 execution condition and measurement resource configuration. For example, a common event list is defined for both event triggered L1 measurement reporting and the conditional LTM. Then only the event index is configured as the L1 execution condition for each candidate configuration. By this way, the event configuration is decoupled from the L1 measurement resource configuration. UE measures all L1 RSs associated with a candidate cell for evaluation by default.

We are open to above options, so we suggested that,

Proposal 3a: In the CLTM evaluation, only the RSs of the same candidate cell can be evaluated if L1 execution condition is configured for the candidate cell.

Proposal 3b: If P3a is agreed, RAN2 down-selects between the following options,

Option 1: For the LTM-CSI-ReportConfigId configured as L1 execution condition of a candidate cell, NW ensures that the associated measurement resource config could only include the RSs of the corresponding candidate cell;

Option 2: If the L1 execution condition of a candidate cell associated with an LTM-CSI-ResourceConfig-r18 which includes RSs from other candidate cells, the UE only evaluates the RSs of the same candidate cell;

Option 3: UE evaluate all the RSs of candidate cell, not the RSs associated with the L1 execution condition:

Option 3-1: LTM-CSI-ReportConfigId-r18 is configured as the execution condition, UE ignore the ltm-ResourcesForChannelMeasurement-r18 configured in the associated LTM-CSI-ReportConfig

Option 3-2: redesign the event configuration signaling structure to avoid the association between L1 execution condition and measurement resource configuration.

On target beam/cell selection of L3-based CLTM

For L1-based RACH-less conditional LTM, the target beam is the beam that satisfies the execution condition. If multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

For L3 based CLTM, no agreement was made and thus with the following open issue included in the RRC open issue list,

## 2.3 RRC-3 Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution. Rapporteur comment: Current procedural text leaves this case to the UE implementation (similar to CHO), so the proposal is to keep current text as it is and delete the FFS.

From our perspective, it is straightforward to follow the same principle of CHO, that is to say, if more than one candidate cells satisfy the L3 execution condition, it’s up to UE implementation to select one of the LTM candidate configurations as the target cell.

Proposal 4: (RRC-3) For L3 based CLTM, it’s up to UE implementation to select one of the LTM candidate configurations as the target cell if more than one candidate cells satisfy the L3 execution condition.

Once the UE determines the target cell for L3-based CLTM, another issue is how to select the target beam for the selected cell,

| MAC-10 | Beam selection for L3 based RACH-less CLTM determination Editor’s NOTE: FFS how to determine the selected beam in case the CLTM is triggered by L3 event. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

If the UE has a valid TA associated with the target beam, UE will perform RACH-less cell switch to the target cell. From our view, the simplest way is just up to UE implementation to select a suitable beam to access to the target cell.

Proposal 5a: (MAC-10) For L3-based CLTM, it’s up to UE implementation to select the target beam.

Proposal 5b: (MAC-10) For L3-based CLTM, UE performs RACH-less C-LTM if UE has a valid TA for the selected beam. Otherwise, UE performs RACH-based C-LTM.

CG resource selection

CLTM supports CG based RACH-less, and CLTM supports CSI-RS based measurement. If UE determines the target beam with CSI-RS for RACH-less CLTM, then another issue is whether the CSI-RS can be associated with the CG resource,

| MAC-12 | How UE select the CG resource if the selected beam is CSI-RS? Editor’s NOTE: This part will be further updated based on RAN1 progress on CSI-RS. E.g. there is no agreeent on CSI-RS associated with CG. | Issue Type: Not essential but important How to address it: based on RAN1 inputs |
| --- | --- | --- |

If the target beam by CSI-RS is not associated with the CG resource, UE needs to find the QCLed SSB with the target CSI-RS so that it can determine the associated CG resource. But this will cause mis-alignment between the UE and target cell, as the UE uses the beam associated with CSI-RS (e.g. narrow beam) for first PUSCH transmission while the target cell uses the SSB (e.g. broad beam) to receive, i.e., the target cell is not aware of the real beam selected by UE. So we suggest supporting the association of CSI-RS with the CG resource for CLTM. And, we can LS to RAN1 for further confirmation.

A draft LS is provided in Annex A.

Proposal 6: (MAC-12) RAN2 assumes the CG resource for RACH-less CLTM can be associated with CSI-RS directly. Send LS to RAN1 for further confirmation.

On how to determine the CG resource for L3 based RACH-less CLTM,

| MAC-9 | In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for CLTM target cell, how UE selectes the valid CG resource for L3 based CLTM? Editor’s NOTE: FFS in case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for CLTM target cell, how UE selectes the valid CG resource for L3 based CLTM. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

Once the UE determines the target beam by UE implementation as suggested in Proposal 5a, it is straightforward to select the CG resource associated with the target beam for the first PUSCH transmission, regardless of SSB or CSI-RS for the target beam.

Proposal 7: (MAC-9) For L3-based RACH-less CLTM, UE selects the CG resource associated with the selected target beam.

The final open issue related to the CG resource selection is how to specify the UE behavior if the UE cannot determine a valid CG resource associated with the target beam,

| MAC-11 | In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for LTM target cell, while UE couldn’t obtain valid CG resource, whether UE will fall back to RACH-based LTM? | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

From our perspective, since only CG is supported for RACH-less CLTM, UE should perform RACH-based CLTM if UE cannot determine a valid CG resource for the first PUSCH transmission.

Proposal 8: (MAC-11) If the UE has a valid TA for a target beam but cannot determine a valid CG resource, it shall perform RACH-based CLTM to the target cell.

TCI state determination upon RACH-less CLTM execution

In R18 RACH-less LTM, the TCI state used in the target cell is indicated in the LTM cell switch command MAC CE. Upon receiving the MAC CE, the UE MAC layer will indicate the TCI state information to the physical layer so that the physical layer can apply the TCI state for transmitting/receiving in the target cell.

5.18.35 LTM Cell Switch Command The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75. The MAC entity shall: ….. 3> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

For RACH-less conditional LTM, only CG is supported for the first PUSCH transmission in the target cell. When UE determines the target beam no matter based on L1 execution condition or L3 execution condition, UE will use the CG resource associated with the target beam (target RS) to transmit the first PUSCH in the target cell. NW could determine the beam based on the selected CG resource, it couldn’t be aware the detailed TCI state if more than one TCI state has the QCL with the target beam. It seems UE does not need to determine a TCI state upon RACH-less conditional LTM execution. However, whether the physical layer needs the TCI state information is under RAN1 scope. So we suggest LS to RAN1 for further clarification.

A draft LS is provided in Annex A.

Proposal 9: Send LS to RAN1 to confirm whether UE needs to determine the target TCI state upon the RACH-less conditional LTM execution and indicate it to lower layer.

On the co-existence of CLTM and other features

The following editor notes still remain to address the co-existence of CLTM with other feature,

| MAC-13 | FFS the coexistence between CLTM and (e)RedCap, CovEnh? Editor’s NOTE: Whether/How CLTM could co-exist with (e)RedCap is FFS, i.e. whether follow Rel-18 intra-CU LTM as below. Editor’s NOTE: Whether/How CLTM could co-exist with CovEnh is FFS, i.e. whether follow Rel-18 intra-CU LTM as below. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

In R18, RAN2 had agreed to support the co-existence of NW triggered intra-CU LTM and (e)RedCap/CovEnh. The only difference compared to NW triggered intra-CU LTM is the UE evaluation of the execution condition by CLTM, but the main impact on LTM is on the RACH-based access. For CLTM, if CFRA is configured via RRC message, it could depend on NW to provide the suitable resource if co-exist with the (e)RedCap/CovEnh. For CBRA, the UE could select the suitable resource too based on the current spec. so the CLTM can co-exist with the (e)RedCap/CovEnh, similar as R18 intra-CU LTM.

Proposal 10a: (MAC-13) Support the co-existence of CLTM and (e)RedCap.

Proposal 10b: (MAC-13) Support the co-existence of CLTM and CovEnh.

Conclusion

Based on the analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: In the CLTM evaluation, if the L1 execution condition of a candidate cell associated with an LTM-CSI-ResourceConfig-r18 which includes RSs from other candidate cells, then the RSs of those other candidate cells are evaluated. This seems unreasonable.

On CLTM TAT handling

Proposal 1: The value of ltm-TimeAlignmentTimer-r19 is determined by the source cell. Send LS about the agreement to RAN3.

NW triggered LTM to the CLTM candidate with early TA

Proposal 2: (MAC-14) Upon receiving (Enhanced) LTM Cell switch Command MAC CE, UE performs RACH-less LTM cell switch only if the MAC CE includes a valid TA value or UE has performed UE based TA measurement to the target cell, regardless of whether CLTM candidate TAT timer is running for the target cell.

On L1 based CLTM

Proposal 3a: In the CLTM evaluation, only the RSs of the same candidate cell can be evaluated if L1 execution condition is configured for the candidate cell.

Proposal 3b: If P3a is agreed, RAN2 down-selects between the following options,

Option 1: For the LTM-CSI-ReportConfigId configured as L1 execution condition of a candidate cell, NW ensures that the associated measurement resource config could only include the RSs of the corresponding candidate cell;

Option 2: If the L1 execution condition of a candidate cell associated with an LTM-CSI-ResourceConfig-r18 which includes RSs from other candidate cells, the UE only evaluates the RSs of the same candidate cell;

Option 3: UE evaluate all the RSs of candidate cell, not the RSs associated with the L1 execution condition:

Option 3-1: LTM-CSI-ReportConfigId-r18 is configured as the execution condition, UE ignore the ltm-ResourcesForChannelMeasurement-r18 configured in the associated LTM-CSI-ReportConfig

Option 3-2: redesign the event configuration signaling structure to avoid the association between L1 execution condition and measurement resource configuration.

On target beam/cell selection of L3 based RACH-less CLTM

Proposal 4: (RRC-3) For L3 based CLTM, it’s up to UE implementation to select one of the LTM candidate configurations as the target cell if more than one candidate cells satisfy the L3 execution condition.

Proposal 5a: (MAC-10) For L3-based CLTM, it’s up to UE implementation to select the target beam.

Proposal 5b: (MAC-10) For L3-based CLTM, UE performs RACH-less C-LTM if UE has a valid TA for the selected beam. Otherwise, UE performs RACH-based C-LTM.

CG resource selection

Proposal 6: (MAC-12) RAN2 assumes the CG resource for RACH-less CLTM can be associated with CSI-RS directly. Send LS to RAN1 for further confirmation.

Proposal 7: (MAC-9) For L3-based RACH-less CLTM, UE selects the CG resource associated with the selected target beam.

Proposal 8: (MAC-11) If the UE has a valid TA for a target beam but cannot determine a valid CG resource, it shall perform RACH-based CLTM to the target cell.

TCI state determination upon RACH-less CLTM execution

Proposal 9: Send LS to RAN1 to confirm whether UE needs to determine the target TCI state upon the RACH-less conditional LTM execution and indicate it to lower layer.

On the co-existence of CLTM and other features

Proposal 10a: (MAC-13) Support the co-existence of CLTM and (e)RedCap.

Proposal 10b: (MAC-13) Support the co-existence of CLTM and CovEnh.

Reference

[1] Draft_RAN2_129bis_Meeting_Report_v2

Annex A

3GPP TSG-RAN WG2 Meeting #130                                                                R2-250xxxx

St.Julians, Malta,  May 19th – 23rd , 2025

Title:	LS on Conditional Intra-CU LTM

Response to:

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	RAN2

To:	RAN1

Cc:

Contact person:	Rui Zhou

zhourui@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

During the discussion in RAN2 130 meeting, RAN2 agreed the following assumption for RACH-less CLTM,

RAN2 assumes the CG resource for RACH-less CLTM can be associated with CSI-RS directly. LS to RAN1 for further confirmation.

RAN2 would like to seek inputs from RAN1 by asking the following question:

Question 1: For RACH-less CLTM, it is UE to determine the cell switch execution using the CG resource without pre-indication to the NW, could RAN1 confirm whether the RAN2 assumption above is feasible?

In R18 RACH-less LTM, the TCI state used in the target cell is indicated in the LTM cell switch command MAC CE. Upon receiving the MAC CE, the UE MAC layer will indicate the TCI state information to the physical layer so that the physical layer can apply the TCI state for transmitting/receiving in the target cell.

For RACH-less CLTM, only CG is supported for the first PUSCH transmission in the target cell. When UE determines the target beam, UE will use the CG resource associated with the target beam to transmit the first PUSCH in the target cell without LTM CSC command. Then RAN2 is discussing whether UE needs to determine a TCI state upon RACH-less CLTM execution and indicates it to the physical layer, which is similar as R18 RACH-less LTM. RAN2 would like to seek inputs from RAN1 by asking the following question:

Question 2: Upon RACH-less CLTM execution, whether UE needs to determine a TCI state upon RACH-less CLTM execution and indicates it to the physical layer?

- 2	Actions

To RAN1 group

ACTION: 	RAN2 respectfully asks RAN1 to provide feedback on above questions.

- 3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #131	25th – 29th Aug 2025	India, IN

TSG-RAN2 Meeting #131bis	13th – 17rd Oct 2025	Prague , CZ