3GPP TSG-RAN WG2 Meeting #130                 	                                                                                 R2-2503415

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Consideration on on-demand SIB1 issues

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the details on remaining issues on OD-SIB1 case 2. The contribution is organized as follows.

In section 2, we discuss the following aspects,

- [38.304-01] whether to ignore the legacy excluded cell lists

- Cell barring for NES OD-SIB1 Cell

- Cell access information in UL-WUS configuration

- SIB1 transmission for emergency case

- Clarification on how to get/check OD-SIB1

- Unnecessary Paging for legacy UE

Our proposals are summarized in Section 3.

Discussion

In this section, regarding the achieved agreements in [1], the further issues associated to RAN2 are listed and discussed for OD-SIB1.

  - Cases to ignore the legacy excluded cell lists

RAN2#127bis NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities). RAN2#128 Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

The agreements on excluded cell list are shown as above. In our view to the agreements and RAN2 discussion in RAN2#127bis meeting, the legacy UE was agreed to be prevented from accessing the OD-SIB1 enabled Cell based on the legacy excluded list intraFreqExcludedCellList/interFreqExcludedCellList, in which the OD-SIB1 enabled Cell will be listed, it is power saving compared with the way of UE additionally reading the MIB. Based on this agreement, the following issue is how to let the UE supporting OD-SIB1 ignore the OD-SIB1 enabled Cell in legacy intraFreqExcludedCellList/interFreqExcludedCellList. So, in RAN2#128 meeting, for the UE supporting OD-SIB1, two options were proposed and discussed, one option is let the UE supporting OD-SIB1 ignore the NES Cell in legacy excluded cell based on the OD-SIB1 enabled Cell information in SIBxx, another option is to introduce new R19 excluded cell list for the UE supporting OD-SIB1, the legacy UE or non-OD SIB1 supporting UE will follow the legacy intraFreqExcludedCellList/ interFreqExcludedCellList including possible OD-SIB1 enabled Cell. The second option was finally agreed in RAN2#128 meeting.

As mentioned in the open issue list in RAN2#129bis post email, one case is proposed, the network may not configure Rel-19 excluded cell list if there is no excluded cell for the UE supporting OD-SIB1 in real deployment. Some company proposed that legacy excluded cell list may include both NES cells and/or some normal cells (e.g. blacklist cells for all UE access), in this case, the network will also configure the normal cells in the R19 excluded list IEs for the UE supporting OD-SIB1, this is another case and different from the proposed case. Considering the solution needs to cover all cases, it is proposed that the UE supporting OD-SIB1 always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated R19 excluded cell lists being provided, which are also aligned to the history agreements. Besides, in order to make the history agreement clear, it is proposed to clarify that the OD-SIB1 supporting Cell will be included in the legacy excluded list intraFreqExcludedCellList/interFreqExcludedCellList if available, which could avoid the unnecessary accessing to OD-SIB1 enabled Cell for legacy UE although it has agreed that legacy UE could access to OD-SIB1 enabled Cell if it could acquire the broadcasted SIB1.

Proposal 1: (38.304-01) The UE supporting OD-SIB1 always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided.

Proposal 2: (38.304-01) OD-SIB1 enabled Cell is included in the legacy excluded list of a cell in which SIBxx is provided if available.

  - Cell barring for NES OD-SIB1 Cell

In legacy R18 NES as follows, network introduced a specific barring indication cellBarred in SIB1 for NES UEs, the NES UE will read the cellBarredNES to see whether the UE (with NES DTX/DRX feature) will be barred or not if cellBarred in the acquired MIB is set to barred. Based on this indication, the NES Cell working on CellDTX-DRX feature could allow only NES UEs supporting cell DTX/DRX capability to camp on the NES Cell by broadcasting cellBarredNES.

TS 38.331 5.2.2.4 Actions upon receipt of System Information <skip> 1> if the UE supports nes-CellDTX-DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting nes-CellDTX-DRX is in RRC_CONNECTED while T311 is running: 2> if cellBarred in the acquired MIB is set to barred: 3> if cellBarredNES is absent in the acquired SIB1: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends; <skip> – SIB1 SIB1-v1800-IEs ::= SEQUENCE { … cellBarredNES-r18 ENUMERATED {notBarred} OPTIONAL, -- Need R … } … cellBarredNES This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX as described in 5.2.2.4.2..

Observation 1: A barring indication in SIB1 is used for R18 NES UE with Cell DTX/DRX feature.

Agreements on OD-SIB1 There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

For R19 OD-SIB1 enabled Cell, it has been agreed that the legacy UE and R19 UE not supporting OD-SIB capability will be barred by the OD-SIB1 enabled Cell, and k_ssb signaling “no SIB1” indication in MIB will be used. It seems that all the UE not supporting OD-SIB1 capability will be barred by the k_ssb at NES Cell. There is also no good reason for OD-SIB1 enabled Cell to bar the UE supporting OD-SIB1, otherwise, no UE could camp on OD-SIB1 Cell. So it is suggested that RAN2 to clarify the barring mechanism is used or not at OD-SIB1 enabled Cell for UE supporting OD-SIB1. For non-OD-SIB1 enabled Cell, the legacy mechanism could be used to bar all UEs including the UE supporting OD-SIB1, the UE supporting OD-SIB1 could follow the legacy procedure to determine whether the non-OD-SIB1 enabled Cell is barred or not.

Proposal 3: Discuss whether there is a scenario where the UE supporting OD-SIB1 is not allowed to access OD-SIB1 enabled Cell.

  - Cell access information in UL-WUS configuration

In last meeting, RAN2 have already discussed whether the UL-WUS configuration will include the cell accessing information. As the access information including the PLMN ID, TAC, cellReservedForOtherUse, cellReservedForFutureUse and the specific barring bits for each feature (such as for NTN/RedCap/R18 NES feature and so on), the number of bits for access is considerably not small, which will also increase the complexity for normal UE to decode the SIBxx, so it is suggested not to include the cell access information in UL-WUS configuration.

Proposal 4: Cell access information is not included in UL-WUS configuration.

  - SIB1 transmission for emergency case

According to legacy procedure, if UE is ETWS capable or CMAS capable and receives the etwsAndCmasIndication bit in the Short Message, it will re-acquire the SIB1 immediately.

Observation 2: In legacy, UE needs to acquire the SIB1 immediately when receiving the etwsAndCmasIndication in Short Message, which is beneficial for latency.

In this case, the network will change to normal mode other than NES mode, or the network will broadcast the SIB1 periodically as agreed in last meeting, the NES UE could receive the SIB1 immediately without any delay. Then, there is no issue is explored, it could be considered as network implementation.

Proposal 5a: In emergency cases, it is up to network implementation to switch to normal mode or broadcast the SIB1 periodically.

Proposal 5b: UE acquires the SIB1 immediately on OD-SIB1 enabled cell without sending WUS request when receiving the etwsAndCmasIndication in Short Message.

  - Clarification on how to get/check OD-SIB1

RAN2#127bis agreements 7. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell. RAN2#129bis agreements 2.When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

However, it seems that there are ambiguity on how the K_SSB is set When the cell supporting on demand SIB1 is broadcasting SIB1 periodically. In our understanding, it could be up to NW implementation to set the K_SSB as in legacy or as in a OD-SIB1 NES cell. As in the RAN2 agreement, legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1(i.e., in case NW sets the the K_SSB as in legacy).

Proposal 6：When the cell supporting on demand SIB1 is broadcasting SIB1 periodically, it is up to NW implementation to set the K_SSB as in legacy or as in an OD-SIB1 enabled cell.

RAN1#120bis agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

Besides, RAN2 agreed that UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell, but RAN1 does not achieved an indication to let UE check, and considered it is UE implementation to whether to monitor Type 0 PDCCH for SIB1 for NES Cell with where K_SSB>=24 for FR1 or K_SSB>=12 for FR2. So it suggested RAN2 to clarify how UE checks if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Proposal 7：Discuss how UE supporting OD-SIB1can determine if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

  - Unnecessary Paging for legacy UE

RAN2#127bis agreement: 9. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. RAN2#128 agreement: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs. RAN2#129bis agreement: When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

Although RAN2#129bis agreed that legacy UE could camp on NES UE if it could read the SIB1 message, however, it can be seen that the legacy UE will be barred by ssb-SubcarrierOffset or the new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs, which could make legacy UE not camp on OD-SIB1 Cell, then the paging for legacy UE seems not necessary at NES Cell. It is also network power saving if the paging for legacy UE is not transmitted at NES Cell. Hence, considering the legacy UE is not allowed to camp on NES Cell, also from the view of network power saving, it can be concluded that the NES Cell will not transmit the legacy UE paging message.

Observation 3: The paging message for legacy UE is not necessary to be transmitted at OD-SIB1 enabled Cell.

In RAN2#129 meeting, the following was agreed for R19 paging adaptation:

RAN2#129 agreement: 5.A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

Similarly, in order to avoid OD-SIB1 Cell paging legacy UEs, a new UE capability for support of OD-SIB1 can be introduced in UE-RadioPagingInfo. As UE-RadioPagingInfo is included in the CN paging and RAN paging NW nodes, OD-SIB1 Cell can decide whether to initiate paging procedure for a UE based on  the new UE capability when CN paging or RAN paging is triggered. For example: When OD-SIB1 Cell receives a PAGING message from AMF and the new UE capability for support of OD-SIB1 is also included, OD-SIB1 Cell initiates paging procedure for the UE via Uu interface. Otherwise, OD-SIB1 Cell doesn’t initiate paging procedure via Uu interface as it identifies the PAGING message is for a UE that doesn’t support OD-SIB1 feature.

Proposal 8: A new UE capability for support of OD-SIB1 is introduced in UE-RadioPagingInfo.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: A barring indication in SIB1 is used for R18 NES UE with Cell DTX/DRX feature.

Observation 2: In legacy, UE needs to acquire the SIB1 immediately when receiving the etwsAndCmasIndication in Short Message, which is beneficial for latency.

Observation 3: The paging message for legacy UE is not necessary to be transmitted at OD-SIB1 enabled Cell.

Proposal 1: (38.304-01) The UE supporting OD-SIB1 always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided.

Proposal 2: (38.304-01) OD-SIB1 enabled Cell is included in the legacy excluded list of a cell in which SIBxx is provided if available.

Proposal 3: Discuss whether there is a scenario where the UE supporting OD-SIB1 is not allowed to access OD-SIB1 enabled Cell.

Proposal 4: Cell access information is not included in UL-WUS configuration.

Proposal 5a: In emergency cases, it is up to network implementation to switch to normal mode or broadcast the SIB1 periodically.

Proposal 5b: UE acquires the SIB1 immediately on OD-SIB1 enabled cell without sending WUS request when receiving the etwsAndCmasIndication in Short Message.

Proposal 6：When the cell supporting on demand SIB1 is broadcasting SIB1 periodically, it is up to NW implementation to set the K_SSB as in legacy or as in an OD-SIB1 enabled cell.

Proposal 7：Discuss how UE supporting OD-SIB1can determine if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Proposal 8: A new UE capability for support of OD-SIB1 is introduced in UE-RadioPagingInfo.

Reference

[1] Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES, Samsung, RAN2#129bis meeting.