3GPP TSG-RAN WG2 Meeting #130                                                                                                  R2-2503416

St Julian, Malta, 19th – 23rd May, 2025

Source:	CATT

Title:	Adaptation of common signal channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

In RAN2#129bis meeting, some agreements on paging adaptation and SSB adaptation were achieved.

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

In this contribution, remaining issues on paging adaptation and RACH adaptation are discussed.

Discussion

Paging adaptation

- FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

The following is the legacy PMO (i.e. firstPDCCH-MonitoringOccasionOfPO) configuration [1].

[[ firstPDCCH-MonitoringOccasionOfPO CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239), sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479), sCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959), sCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919) } OPTIONAL -- Cond OtherBWP ]], //skip firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE { sCS480KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839), sCS480KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679) } OPTIONAL, -- Need R

In legacy paging configurations, the maximum value of Ns is 4 and the maximum number of bits for each PMO is 17, where the maximum offset of the first PDCCH monitoring occasion for paging of each PO of the PF is 16 radio frames. The maximum signaling overhead for PMO configuration is maximum value of Ns*bits for each PMO (4*17=68 bits).

In RAN2#129 meeting, the following agreements were achieved on N and Ns.

1. For N, values smaller than T/32 are not supported. 2. The maximum possible value for Ns is 8.

firstPDCCH-MonitoringOccasionOfPO is configured to point out the first PDCCH monitoring occasion for paging of each PO of the PF. When the maximum number of Ns is extended to 8, the maximum PO per PF (i.e. maxPO-perPF) needs to be extended to 8 correspondingly. Although POs are expected to be clustering together, the possible values of the first PDCCH monitoring occasion for R19 paging of each PO of PF need to be extended correspondingly as well as they may be clustered in any time domain within 32 radio frames. The maximum number of bits for each PMO needs to be extended to 18. With these extensions, the maximum signaling overhead for firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is 144 bits (8*18), which is acceptable to the limited massage size of SIB1. Hence, we propose:

Proposal 1: A separate firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is introduced:

- The maximum PO per PF is extended to 8;

- The maximum offset value is extended to 32 radio frames at symbol level.

If paging adaptation is enabled, it is possible that the separated firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is absent. I.e., pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset are signaled in system information, but the separated firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is absent. In this case, there are two possible understandings for R19 UE supporting paging adaption:

- Alt1: Legacy firstPDCCH-MonitoringOccasionOfPO if configured is used.

- Alt2: No firstPDCCH-MonitoringOccasionOfPO is used.

With alt1, signaling overhead can be reduced with smart NW implementation. Hence, we slightly prefer alt1.

Proposal 2: If paging adaptation is enabled (i.e., pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset are signaled in system information) in the cell, but the firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is absent, UE supporting paging adaption uses legacy firstPDCCH-MonitoringOccasionOfPO if configured.

- FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

In RAN2#129 meeting, it was agreed to introduce a separate PEI configuration. And it is FFS whether to introduce a separate pei-ConfigBWP for paging adaptation.

As shown below, within pei-ConfigBWP, there are two fields, including pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O.

//skip pei-ConfigBWP-r17 SEQUENCE { pei-SearchSpace-r17 SearchSpaceId, firstPDCCH-MonitoringOccasionOfPEI-O-r17 CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239), sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479), sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959), sCS480KHZquarterT-SCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919), sCS480KHZoneEighthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839), sCS480KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679) } } //skip maxPEI-perPF-r17 INTEGER ::= 4 -- Maximum number of PEI occasion per paging frame

The same search space for paging is applied to both legacy paging monitoring and R19 paging monitoring. It can allow legacy and R19 UEs to co-exist in the same PF/PO. Similarly, the same search space can be applied to both legacy PEI and R19 PEI with paging adaptation. Then legacy and R19 UEs can co-exist in the same PEI. firstPDCCH-MonitoringOccasionOfPEI-O is used to indicate the offset from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O with symbol level. Similar as the extension of firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation, a new parameter to extend firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation needs to be introduced. Therefore, a separate pei-ConfigBWP for paging adaptation is not needed, but firstPDCCH-MonitoringOccasionOfPEI-O in the legacy pei-ConfigBWP-r17 is needed to be introduced separately for paging adapation.

The maximum number of PEI occasion per paging frame is extended to 8, while the maximum value of the offset, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O, is extended to 32 radio frames at symbol level.

Proposal 3a: A separate pei-ConfigBWP for paging adaptation is not needed, but firstPDCCH-MonitoringOccasionOfPEI-O in the legacy pei-ConfigBWP-r17 is needed to be introduced separately for paging adaptation, i.e., a separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation is introduced.

Proposal 3b:  For the separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation:

- The maximum number of PEI occasion per paging frame is extended to 8;

- The maximum offset value is extended to 32 radio frames at symbol level.

Similar as firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation, when pagingAdaptationPEI-Config is signaled in system information but the separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation is absent, there are two possible understandings for R19 UE supporting paging adaption:

- Alt1: Legacy firstPDCCH-MonitoringOccasionOfPEI-O if configured is used.

- Alt2: No firstPDCCH-MonitoringOccasionOfPEI-O is used.

Similar as firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation, we slightly prefer alt1 with reduced signalling overhead via smart NW implementation.

Proposal 4: If pagingAdaptationPEI-Config is signaled in system information but the separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation is absent, R19 UE supporting paging adaption uses legacy firstPDCCH-MonitoringOccasionOfPEI-O if configured.

- FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions

The following is the ASN.1 for pagingAdaptationNAndPagingFrameOffset. And it is FFS the field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions.

pagingAdaptationNAndPagingFrameOffset CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15), oneThirtySecondT INTEGER (0..31) }

Compared with legacy paging, R19 paging adaptation must increase interval between PFs. Therefore, we prefer to add the restriction in the field description that the number of total paging frames in T for paging adaptation should be less than that for legacy paging (i.e. the configuration within nAndPagingFrameOffset). Furthermore, with the restriction, “oneT” is not possible to be configured for pagingAdaptationNAndPagingFrameOffset. It can be removed from pagingAdaptationNAndPagingFrameOffset.

Proposal 5: Add a restriction in the field description of pagingAdaptationNAndPagingFrameOffset, i.e., the number of total paging frames in T with should be less than that with nAndPagingFrameOffset.

Proposal 6: Remove “oneT” from pagingAdaptationNAndPagingFrameOffset.

RACH adaptation

RAN1 sent an LS [2] to ask RAN2 to confirm the feasibility with the following RAN1 agreement:

Agreement For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

It is straightforward for RAN2 to confirm to use the first reserved bit (i.e.,5th bit) within the Short Message field for RACH adaptation indication in DCI format 1_0 with P-RNTI.

Proposal 7: Use the 5th bit within the Short Message field for RACH adaptation indication in DCI format 1_0 with P-RNTI.

According to RAN1 agreement, the RACH adaptation indication in DCI format 1_0 with P-RNTI is applied for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs. For connected UEs, paging search space may not be configured in active DL BWP. In this case, connected UEs cannot receive the RACH adaptation indication in DCI format 1_0 with P-RNTI. Then RACH adaptation cannot be applied to these connected UEs. The possible options to handle the case can be:

- Option 1: Up to NW implementation. If RACH adaptation via DCI format 1_0 with P-RNTI is applied to a connected UE, NW needs to ensure that paging search space is configured in active DL BWP.

- Option 2: Another RACH adaptation indication is introduced if paging search space is not configured in active DL BWP for connected UEs.

Option 1 is enough without specification impact. Hence, we propose:

Proposal 8: NW ensures that paging search space is configured in active DL BWP if RACH adaptation indication via DCI format 1_0 with P-RNTI is applied to a connected UE.

The following was agreed for RACH adaptation via DCI format 1_0 with P-RNTI:

Agreement - For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI - The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})

Hence, the granularity for RACH adaptation via DCI format 1_0 with P-RNTI is SI modification period. Short message includes SI change indication. IDLE/INACTIVE mode UEs while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s). Connected UEs shall monitor for SI change indication in any paging occasion at least once per SI modification period if paging search space is configured in active DL BWP. Therefore, for IDLE/INACTIVE mode UEs and CONNECTED UEs to monitor RACH adaptation indication via DCI format 1_0 with P-RNTI, it is enough to reply on legacy procedure of monitoring SI change indication. Specification impact is unseen.

Proposal 9: For IDLE/INACTIVE mode UEs and CONNECTED UEs to monitor RACH adaptation indication via DCI format 1_0 with P-RNTI, UE follows legacy procedure of monitoring SI change indication.

Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Paging Adaptation

Proposal 1: A separate firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is introduced:

- The maximum PO per PF is extended to 8;

- The maximum offset value is extended to 32 radio frames at symbol level.

Proposal 2: If paging adaptation is enabled (i.e., pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset are signaled in system information) in the cell, but the firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is absent, UE supporting paging adaption uses legacy firstPDCCH-MonitoringOccasionOfPO if configured.

Proposal 3a: A separate pei-ConfigBWP for paging adaptation is not needed, but firstPDCCH-MonitoringOccasionOfPEI-O in the legacy pei-ConfigBWP-r17 is needed to be introduced separately for paging adaptation, i.e., a separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation is introduced.

Proposal 3b:  For the separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation:

- The maximum number of PEI occasion per paging frame is extended to 8;

- The maximum offset value is extended to 32 radio frames at symbol level.

Proposal 4: If pagingAdaptationPEI-Config is signaled in system information but the separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation is absent, R19 UE supporting paging adaption uses legacy firstPDCCH-MonitoringOccasionOfPEI-O if configured.

Proposal 5: Add a restriction in the field description of pagingAdaptationNAndPagingFrameOffset, i.e., the number of total paging frames in T with should be less than that with nAndPagingFrameOffset.

Proposal 6: Remove “oneT” from pagingAdaptationNAndPagingFrameOffset.

RACH Adaptation

Proposal 7: Use the 5th bit within the Short Message field for RACH adaptation indication in DCI format 1_0 with P-RNTI.

Proposal 8: NW ensures that paging search space is configured in active DL BWP if RACH adaptation indication via DCI format 1_0 with P-RNTI is applied to a connected UE.

Proposal 9: For IDLE/INACTIVE mode UEs and CONNECTED UEs to monitor RACH adaptation indication via DCI format 1_0 with P-RNTI, UE follows legacy procedure of monitoring SI change indication.

Reference

- TS 38.331 v18.5.1

- R2-2503312, LS on DCI-based PRACH adaptation (R1-2503086; contact: Ericsson)