3GPP TSG-RAN WG2 Meeting #130	R2-2503419

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.2

Source: 	CATT

Title: 	Discussion on Paging for Ambient IoT

Document for: 	Discussion and Decision

Introduction

Based on the progress of RAN2#129bis [1] and the guideline of open issue list [2], this contribution further discusses the AIoT paging mechanisms, including paging procedure and paging message contents.

Discussion

2.1	Paging procedure

Based on the progress of past meetings, Figure 1 is given as a reference procedure for the paging mechanism. Further, some leftover issues for the paging procedure are discussed thereinafter.

Figure 1. AIoT paging procedure

- Open issue 1-2: Transaction ID aspects

On transaction ID, there is an open issue in the open issue list for further discussion.

| Issue 1-2: transaction ID | Whether/how to specify how the reader generate Transaction ID, and the size - Relevant agreements: - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size - Status in running CR: captured as Editor’s Note in 6.2.1.1. | To be discussed by company contributions |
| --- | --- | --- |

How does the reader generate transaction ID based on correlation ID can be left to reader implementation. Specified solution is not needed. Considering the size of transaction ID, we think 2 bits is enough to reduce the error case caused by mis-detections of the paging message without introducing excessive signalling overhead.

Proposal 1: (1-2) How to generate the transaction ID based on correlation ID is up to reader implementation. The transaction ID has a size of 2 bits.

- Storage of necessary status information

During the paging procedure, the device needs to store some status information for the ongoing service request, such as the transaction ID, which can be used by the device to determine whether to respond the subsequent paging messages. A key issue is to determine under which condition the device should store the transaction ID. There are several candidate options:

- Option 1: Upon receiving a new transaction ID (the current implementation in running CR [3])

When the device receives an A-IoT paging message containing a transaction ID, even if the device is not addressed by the paging message, it stores the received transaction ID unless it is same as the one stored by the device. After successfully responding to the paging message, the device updates its response status and it shall not respond to the subsequent paging message with the same transaction ID.

- Option 2: Upon receiving a new transaction ID by the paging message addressed to itself

When the device receives an A-IoT paging message containing a transaction ID, it first determines whether it is addressed by the paging message. If so, the device stores the received transaction ID, unless the received transaction ID is same as the one already stored by the device. After successfully responding to the paging, the device shall not respond to the subsequent paging messages with the same transaction ID. Compared to option 1, the device only needs to store the transaction ID when it is addressed by the AIoT paging.

- Option 3: Upon receiving a new transaction ID by the paging message and making successful respond

When the device receives an AIoT message containing a transaction ID, it first determines whether it is addressed by the paging message. If so, the device responds to the paging message. If the response is successful, the device stores the transaction ID. Subsequently, if the device receives another paging message with the same transaction ID as the stored one, it shall not respond to that paging message. In this approach, the device stores the transaction ID when it successfully responds and the device does not need to maintain a response status in its memory.

All three options are workable. However, option 1 requires the non-targeted devices to store the transaction ID, which introduce unnecessary consumption for the devices. So we prefer option 2 and option 3.

Proposal 2: RAN2 to discuss how to store the transaction ID upon receiving the paging message by the device:

- WF1: Upon receiving a new transaction ID by the paging message addressed to itself.

- WF2: Upon receiving a new transaction ID by the paging message and making successful respond.

Proposal 2a: As a consequence of Proposal 2, “upon successful response, the device shall not respond to the subsequent paging messages” can be specified by:

- WF1: The device stores the response status associated with the transaction ID. When a successful response status for a transaction ID is registered, it shall not respond to the subsequent paging messages containing the same transaction ID.

- WF2: If the device receives paging message with the same transaction ID as the stored one, it shall not respond to that paging message.

Another issue is where to store the transaction ID and response state (if needed). RAN1 has agreed to support the following two types of memory [4].

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

Considering the status information is used in short-term and changes frequently, it is more suitable to store the information in the temporary memory.

Proposal 3: The device maintains the status information, including transaction ID and other possible information, in the volatile memory.

- Indicate the end of the paging procedure

Another issue is whether the reader should explicitly indicate the end of the paging procedure to the devices, as mentioned in the Issue 1-1 of the open issue list. Some companies may think that a previous paging procedure can be implicitly ended by a new paging procedure, i.e., a paging message containing a different transaction ID. However, this mechanism is not efficient, as the next service request from CN may occur a long time later. In such cases, the device would need to maintain its status information until it receives the paging message for the next service request or out of power. It is not beneficial for device power consumption and may cause the device to miss possible paging messages when it is out of power.

| Issue 1-1: multi-reader paging | If a device gets a new service request while one procedure is still ongoing, whether/how to specify device behaviour or leave it to implementation, and the end of procedure if needed. - Relevant agreements: - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). - FFS on end of procedure - Status in running CR: captured as Editor’s Note in 5.2. | To be discussed by company contributions |
| --- | --- | --- |

One way to address the aforementioned issue is to explicitly indicate the end of the paging procedure to the devices. Regarding how to indicate the end of the paging procedure, two solutions are currently on the table: using a timer or introducing an end indication in the last paging message. However, some companies are concerned that using a timer would introduce complexity to the device. For this reason, it is proposed to adopt the 1-bit end indication method. Upon receiving this indication, the device determines that this is the last paging message and can then releases all stored status information related to this paging procedure from its temporary memory.

Proposal 4: (1-1) Introduce a 1-bit end indication in the paging message. The presence of this end indication means the current message is the last paging message in the ongoing paging procedure.

Proposal 4a: (1-1) Upon receiving the end indication, the device could determine when to release all stored status information related to this paging procedure, including the transaction ID and other possible information.

2.2	Paging message content and format

2.2.1		Paging identifier

- Issue 1-3: Paging ID length field

On Paging ID length field, there is an open issue in the open issue list for further discussion.

| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g. value range, how many bits, format design - Relevant agreements: - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - RAN2 sent LS to CT4 and SA2 in R2-2503197 asking for their feedback on the above agreement, for RAN2 to determine the field for paging ID length. - Note: SA2 already agreed the filtering information and captured it in clause 5.8 in 23.369, this may enable some extent of RAN2 discussion before their feedback. - Status in running CR: the field name is captured in 6.2.1.1 without the detailed format. | To be discussed by company contributions |
| --- | --- | --- |

After checking SA2’s progress on the definitions of Device Permanent Identifier and filter [5], it is observed that the Device Permanent Identifier has a limited length options, while the filter seems can be of any length.

The filtering information includes a list of filtering element information. The filtering element information includes: - Information indicating a which component of the AIoT Device Permanent Identifier that is used to match the bitstring. - A bitstring which is used to compare with the component. Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier. NOTE: The bitstring match can start at any position in the corresponding component.

Therefore, it seems impractical to restrict the length of the paging ID to only a limited number of options. Instead, it would be more effective to directly indicate the length of the paging ID within the paging ID field. Since we have already sent LS to SA2 and CT4, we can revisit this approach once we receive the reply LSs.

Additionally, it is worth noting that SA3 will introduce a temporary ID for security purposes [6] [7]. We understand the temporary ID is primarily used to page single UE over the AIoT interface. In other words, the paging ID in the paging message may ultimately be in format of the temporary ID rather than the Device Permanent Identifier. The design of paging ID field will also involve SA3 progress.

The following aspects and principles are agreed for the conclusion on KI#3 - A mechanism to protect AIoT device ID based on the use of temporary ID shall be supported. - Mechanism shall allow unambiguous identification of the AIoT device - A mechanism to re-synchronize de-synchronized temporary IDs shall be supported. Editor’s Note: Additional conclusions on solution are FFS

Proposal 5: (1-3) The length of the paging ID is directly indicated in the paging ID length field. This approach can be revisited once more information is inputted from SA2/SA3/CT4.

- Issue 1-6: Whether the paging identifier is visible to AIoT MAC

In the open issue list, there is an open issue on visibility of the Paging ID, which is marked as not critical.

In past meetings, discussion on the visibility of the paging ID in RAN2 have focused on whether there is a need for visibility at the MAC layer. However, the visibility of the paging ID to the MAC layer actually depends on how the AIOTF sends the paging ID to reader, i.e., design of SA2 signalling, rather than on whether it is useful at the AIoT interface. After checking the specification of SA2 [5], we think there is a need to revisit the assumption achieved in RAN#129.

| Issue 1-6: Paging ID visibility | Whether Paging ID is invisible or visible to MAC. - Relevant agreements: - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer. - From the previous discussion, there are some motivations to make paging ID visible to MAC: - 1. Reader can operate on the paging ID for further sub-grouping. The Rapp understands this can be considered as an enhancement from reader side for better system efficiency. From device side, since there is an explicit indication for CBRA and CFRA, the device (even in multi-device CFRA) can determine how to perform random access instead of paging ID/group ID. In this case, such visibility is not an essential function. And according to guidance from chair lady, such enhancement can be considered with lower priority. - 2. Reader can associate the paging ID/device ID and AS ID for a given device within a service request. The Rapp understands according to RAN3 LS R3-252481, reader will allocate NGAP device ID for each device and maintain the per-session per-device context, via which the reader can associate the command receiving from the NG interface with the AS ID assigned for a device, i.e. such device management/association does not rely on the paging ID/device ID. - 3. The Temp ID may have impact on this visibility discussion. The Rapp understands SA3 has not concluded on the solution of Temp ID. But majority seems think this Temp ID is maintained/managed between CN and device, since they already concluded there is no AS security in A-IoT. Therefore, no RAN2 discussion is needed before SA3 further inputs. | Not critical |
| --- | --- | --- |

Reference to 23.369, we observe that the assumption achieved by RAN2 is not aligned with the understanding of SA2. Take the step 7 in “Figure 6.2.2-1: Inventory Procedure” as an example, it can be seen that the inventory request carries no AIoT NAS data. That is, the AIOTF sends the inventory request with AIoT identification information as cleartext IE. As a consequence, the reader only can contain the paging ID as cleartext field in the paging message.

7. The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN. Editor's note: Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined. […] 10. NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s). The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s). NOTE: When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

Observation 1: (1-6) In TS 23.369, the AIOTF send the inventory request message with AIoT Identification Information as cleartext IE.

Therefore, it is proposed that the paging ID is visible to AIoT MAC layer. We could also send an LS to SA2 and RAN3 for confirmation on that.

Proposal 6: (1-6) The paging ID is visible to AIoT MAC layer. Send an LS to SA2 and RAN3 for confirmation.

2.2.2		Configuration of access occasion

In the open issue list, there is an open issue on the configuration of access occasion.

| Issue 1-4: AO number field | How to indicate the number of access occasions, e.g. the maximum number, the length of field, format design. - Relevant agreements: - the A-IoT paging message can include a number of msg1 resources - Status in running CR: the field name is captured in 6.2.1.1 without the detailed format. | To be discussed by company contributions |
| --- | --- | --- |

On aspect of access occasion, RAN1 and RAN2 achieved the following agreements.

RAN2#129 - Agreements - the A-IoT paging message can include a number of msg1 resources. RAN2#129bis - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. RAN1#120bis Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1

Based on these agreements, the access occasion modelling is quiet clear. An example is provided in Figure 2. Considering the configuration for the access occasion, the reader needs to indicate the number of R2D messages following the paging message within the paging message. Additionally, the X value of each R2D message also needs to be indicated to the device. For simplicity, the X value of all the R2D messages can be configured to be the same and indicated only in the paging message.

Figure 2. Modelling of the access occasion

In summary, the paging message contains the number of R2D messages “N” following the (subsequent) paging message and the number of access occasions “X” following the R2D messages and (subsequent) paging message.

Proposal 7: (1-4) The (subsequent) paging message contains the number of R2D trigger messages “N” following the (subsequent) paging message and the number of access occasions “X” following the R2D trigger messages or (subsequent) paging message.

2.2.3		Whether some fields can be omitted in CFRA

An open issue in the open issue list is:

| Issue 1-5: Paging content for CFRA | Whether paging in CFRA can omit the CBRA related fields, such as transaction ID, Indication of Paging ID present/absence, Number of access occasions in Paging message. - In last meeting, there was a discussion whether transaction ID is needed for CFRA in Paging message. Then during the CR drafting, the Rapp identifies other fields like no paging ID indication, number of access occasions are not useful for CFRA. Since RAN2 also agreed to introduce an explicit indication to indicate CFRA and CBRA, it should be feasible to have different fields in the paging message for CFRA and CBRA. Thus, this can be discussed further. - Status in running CR: captured as Editor’s Note in 6.2.1.1. | To be discussed by company contributions |
| --- | --- | --- |

We agree with the rapporteur that the fields of transaction ID, indication of paging ID present/absent and number of access occasion are not quiet useful for CFRA. For simplicity, we could specify these fields as absent for CFRA.

Proposal 8: (1-5) For CFRA, the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent.

Conclusion

In this contribution, paging functionality for A-IoT is discussed with giving corresponding observations and proposals:

Paging procedure

Proposal 1: (1-2) How to generate the transaction ID based on correlation ID is up to reader implementation. The transaction ID has a size of 2 bits.

Proposal 2: RAN2 to discuss how to store the transaction ID upon receiving the paging message by the device:

- WF1: Upon receiving a new transaction ID by the paging message addressed to itself.

- WF2: Upon receiving a new transaction ID by the paging message and making successful respond.

Proposal 2a: As a consequence of Proposal 2, “upon successful response, the device shall not respond to the subsequent paging messages” can be specified by:

- WF1: The device stores the response status associated with the transaction ID. When a successful response status for a transaction ID is registered, it shall not respond to the subsequent paging messages containing the same transaction ID.

- WF2: If the device receives paging message with the same transaction ID as the stored one, it shall not respond to that paging message.

Proposal 3: The device maintains the status information, including transaction ID and other possible information, in the volatile memory.

Proposal 4: (1-1) Introduce a 1-bit end indication in the paging message. The presence of this end indication means the current message is the last paging message in the ongoing paging procedure.

Proposal 4a: (1-1) Upon receiving the end indication, the device could determine when to release all stored status information related to this paging procedure, including the transaction ID and other possible information.

Paging message content and format

Proposal 5: (1-3) The length of the paging ID is directly indicated in the paging ID length field. This approach can be revisited once more information is inputted from SA2/SA3/CT4.

Observation 1: (1-6) In TS 23.369, the AIOTF send the inventory request message with AIoT Identification Information as cleartext IE.

Proposal 6: (1-6) The paging ID is visible to AIoT MAC layer. Send an LS to SA2 and RAN3 for confirmation.

Proposal 7: (1-4) The (subsequent) paging message contains the number of R2D trigger messages “N” following the (subsequent) paging message and the number of access occasions “X” following each R2D trigger message or (subsequent) paging message.

Proposal 8: (1-5) For CFRA, the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent.

Reference

- RAN2#129 Chair’s notes.

- R2-2503998	Summary of A-IoT MAC open issues

- Initial Text Proposal for A-IoT MAC specification.

- RAN1#116 Chair’s notes.

- 3GPP TR 23.369	Architecture support for Ambient power-enabled Internet of Things.

- R2-2501502	Reply LS on security aspects of Ambient IoT

- S3-251048	Conclusion on KI#3